# — 00_INDEX.md —

# GRIFO M-PBIT Test Environment - Documentazione Tecnica Completa

Versione: 1.0
Data: Febbraio 2026
Autore: Test Automation Team
Progetto: GrifoAutomaticTestEnv - Sistema di Test Automatico per Radar GRIFO

## 📚 Indice Documentazione

### 01 - Introduzione e Overview del Sistema

Scopo del progetto

Architettura generale

Componenti principali

Requisiti di sistema

Flusso di utilizzo tipico

Vantaggi e limitazioni

### 02 - Architettura Software e Moduli

Struttura delle directory

Moduli core (leo_grifo_*)

Dipendenze e interfacce

Diagramma delle relazioni

API comuni

### 03 - Script Principale GRIFO_M_PBIT.py

Struttura del codice

Flusso di esecuzione

Funzioni principali

Gestione degli stati

Ciclo di vita del test

### 04 - Sistema di Test - Workflow e Logica

Ciclo di test completo

Scenari di test (Random, Recycle, TgtGen, ecc.)

Test B6 e B8 (messaggi 1553)

Test seriale e messaggi terminale

Test rilevamento target

### 05 - Modalità di Esecuzione

Modalità Hardware (–target)

Modalità Simulazione (–simulate)

Sistema Mock e Test Senza Hardware

Parametri di configurazione

Launcher scripts

### 06 - Interfaccia Grafica (GUI Real-Time)

Architettura GUI

Aggiornamenti thread-safe

Visualizzazione live dei risultati

Tabella run e statistiche

Tooltip e interazioni

### 07 - Output e Report

Report PDF (struttura e formato)

File CSV statistiche

File JSON dati completi

Naming convention e organizzazione

Esempi di output

### 08 - Configurazione e Personalizzazione

File di configurazione

Scenari personalizzati

Known failures

Parametri di test

Customizzazione scenari

### 09 - Integrazione Hardware

Interfaccia 1553 (SWIG bindings)

Controllo seriale

Power control (BrainBox)

Gestione errori hardware

Protocolli di comunicazione

### 10 - Best Practices e Troubleshooting

Guida all’uso

Problemi comuni e soluzioni

Log e debugging

Manutenzione

FAQ

## 📖 Come Usare Questa Documentazione

### Per Nuovi Utenti

Inizia da Capitolo 01 per comprendere l’architettura generaleX

Prosegui con Capitolo 05 per eseguire il primo testX

Consulta Capitolo 10 per problemi comuniX

### Per Sviluppatori

Leggi Capitolo 02 per capire la struttura del codiceX

Approfondisci Capitolo 03 per il workflow internoX

Consulta Capitolo 09 per interfacce hardwareX

### Per Test Engineers

Inizia da Capitolo 04 per capire i testX

Leggi Capitolo 08 per personalizzare scenariX

Consulta Capitolo 07 per interpretare i risultatiX

|  |  | ## 🔄 Stato Documentazione |
| --- | --- | --- |

Capitolo

Stato

Data Completamento

01 - Introduzione

✅ Completato

04/02/2026

02 - Architettura

✅ Completato

05/02/2026

03 - Script Principale

✅ Completato

05/02/2026

04 - Workflow Test

✅ Completato

05/02/2026

05 - Modalità Esecuzione

✅ Completato

05/02/2026

06 - GUI

✅ Completato

05/02/2026

07 - Output

✅ Completato

05/02/2026

08 - Configurazione

✅ Completato

05/02/2026

09 - Hardware

✅ Completato

05/02/2026

10 - Best Practices

✅ Completato

05/02/2026

## 📝 Convenzioni del Documento

- Codice inline: variabile o funzione()

- Blocchi codice:

- # Esempio Python

- File paths: TestEnvironment/scripts/GRIFO_M_PBIT.py

- Comandi: .\run_simulate.bat

- Note importanti: ⚠️ Prefissati con icona

- Best practice: ✅ Prefissati con check

- Problemi noti: ❌ Prefissati con X

## 📧 Contatti e Supporto

Per domande o suggerimenti su questa documentazione:
- Team: Test Automation Team
- Repository: GrifoAutomaticTestEnv
- Documentazione aggiuntiva: _doc/ directory

Ultima revisione: 05 Febbraio 2026

# — 01_Introduzione_e_Overview.md —

# Capitolo 1: Introduzione e Overview del Sistema

← Torna all’Indice | Prossimo: Architettura Software →X

## 1.1 Scopo del Progetto

Il GRIFO M-PBIT Test Environment è un sistema di test automatizzato progettato per validare il comportamento del radar GRIFO durante l’esecuzione del PBIT (Power-On Built-In Test) - un test diagnostico critico che verifica l’integrità funzionale del sistema radar all’accensione.

### Obiettivi Principali

Automazione Completa: Eliminare l’intervento manuale nei test ripetitivi

Validazione Estensiva: Eseguire centinaia di test in sequenza per identificare failure intermittenti

Tracciabilità: Generare report dettagliati (PDF, CSV, JSON) per ogni esecuzione

Flessibilità: Supportare sia test su hardware reale che simulazioni senza hardware

Monitoraggio Real-Time: Interfaccia grafica per visualizzare lo stato dei test durante l’esecuzione

### Contesto Operativo

Il radar GRIFO è un sistema complesso che comunica tramite:
- Bus 1553: Messaggi di comando/controllo e telemetria
- Seriale RS-232: Output diagnostici, log eventi, messaggi di errore
- Power Control: Gestione accensione/spegnimento tramite BrainBox

Il PBIT è un test critico che:
- Verifica l’integrità dei sottosistemi interni
- Rileva failure hardware (processori, memorie, sensori)
- Produce messaggi diagnostici sul bus 1553 e seriale
- Deve completarsi entro ~20 secondi dall’accensione

## 1.2 Architettura Generale

Il sistema è composto da tre layer principali:

┌─────────────────────────────────────────────────────────────┐
│                    Test Script Layer                         │
│  ┌───────────────────────────────────────────────────────┐  │
│  │       GRIFO_M_PBIT.py (Main Test Script)             │  │
│  │  - Workflow orchestration                             │  │
│  │  - Scenario management                                │  │
│  │  - Statistics aggregation                             │  │
│  │  - Report generation                                  │  │
│  └───────────────────────────────────────────────────────┘  │
└─────────────────────────────────────────────────────────────┘
                              ↓
┌─────────────────────────────────────────────────────────────┐
│                  Framework Layer (env/)                      │
│  ┌──────────────┐  ┌──────────────┐  ┌──────────────┐      │
│  │ leo_grifo_   │  │ leo_grifo_   │  │ leo_grifo_   │      │
│  │ 1553.py      │  │ terminal.py  │  │ io_box.py    │      │
│  │ (1553 bus)   │  │ (Serial)     │  │ (Power)      │      │
│  └──────────────┘  └──────────────┘  └──────────────┘      │
│  ┌──────────────┐  ┌──────────────┐  ┌──────────────┐      │
│  │ leo_grifo_   │  │ test_common_ │  │ leo_grifo_   │      │
│  │ core.py      │  │ function.py  │  │ test_report  │      │
│  │ (Helpers)    │  │ (API)        │  │ .py (PDF)    │      │
│  └──────────────┘  └──────────────┘  └──────────────┘      │
└─────────────────────────────────────────────────────────────┘
                              ↓
┌─────────────────────────────────────────────────────────────┐
│              Hardware Abstraction Layer                      │
│  ┌──────────────┐  ┌──────────────┐  ┌──────────────┐      │
│  │ interpreter  │  │ pySerial     │  │ BrainBox     │      │
│  │ (SWIG 1553)  │  │ (Serial)     │  │ (Power Ctrl) │      │
│  └──────────────┘  └──────────────┘  └──────────────┘      │
└─────────────────────────────────────────────────────────────┘
                              ↓
┌─────────────────────────────────────────────────────────────┐
│                   Physical Layer                             │
│         [GRIFO Radar Hardware] ←→ [1553 / Serial]           │
└─────────────────────────────────────────────────────────────┘

### Separazione Hardware/Simulazione

Un aspetto chiave dell’architettura è la possibilità di eseguire test con o senza hardware:

Modalità Hardware (--target): Comunica con radar reale via 1553/seriale

Modalità Simulazione (--simulate): Usa mock objects che emulano risposte hardware

Questo è possibile grazie al monkey patching dei singleton globali:

# In modalità simulate, i singleton reali vengono sostituiti
theGrifo1553 = MockGrifo1553Interface()  # Invece del vero 1553
theBrainBox = MockBrainBox()             # Invece del vero power control

## 1.3 Componenti Principali

### 1.3.1 Script di Test (TestEnvironment/scripts/)

GRIFO_M_PBIT.py: Script principale di test

GRIFO_M_PBIT_mock.py: Implementazione mock per simulazione

GRIFO_M_PBIT_gui.py: Interfaccia grafica real-time

|  |  | ### 1.3.2 Framework di Test (TestEnvironment/env/) |
| --- | --- | --- |

Moduli che implementano l’interfaccia hardware:

Modulo

Responsabilità

Singleton

leo_grifo_1553.py

Comunicazione bus 1553

theGrifo1553

leo_grifo_terminal.py

Parsing messaggi seriali

N/A

leo_grifo_io_box.py

Controllo alimentazione

theBrainBox

leo_grifo_core.py

Utilities e recorder

theRecorder

test_common_function.py

API check/get/set

N/A

leo_grifo_test_report.py

Generazione PDF

testReport

### 1.3.3 Bindings Hardware (PlatformSimulator/bin/)

interpreter.py: Wrapper Python SWIG per libreria 1553 C++

**_interpreter.pyd**: Libreria nativa compilata (Windows DLL)

### 1.3.4 Launcher Scripts

run_target.bat: Esecuzione su hardware reale

run_simulate.bat: Esecuzione in simulazione (Python embedded)

run_simulate_simple.bat: Simulazione con Python di sistema

## 1.4 Requisiti di Sistema

### Hardware (per modalità target)

PC Windows con porte seriali (USB-to-Serial supportato)

Scheda 1553 compatibile con libreria interpreter

BrainBox (opzionale) per controllo automatico alimentazione

Radar GRIFO connesso

### Software

Python 3.7+ (3.8 raccomandato)

Librerie Python:

pyserial (comunicazione seriale)

reportlab (generazione PDF)

tkinter (GUI - incluso in Python standard)

Embedded Python: Incluso in PlatformSimulator/bin/ per esecuzioni standalone

### Filesystem

Spazio disco: ~50MB per installazione base + ~10MB per 100 run di test

Permessi: Lettura/scrittura in TestEnvironment/pdf_reports/, TestEnvironment/LOG/

## 1.5 Flusso di Utilizzo Tipico

### Scenario 1: Test Rapido in Simulazione

# Dalla root del repository
.\run_simulate_simple.bat

Risultato: Esegue 10 run di test simulati in ~2 minuti, genera PDF/CSV/JSON in TestEnvironment/pdf_reports/

### Scenario 2: Test su Hardware Reale

# Configurare COM port in test, poi:
.\run_target.bat

Risultato: Esegue test su radar connesso, controlla alimentazione, genera report

### Scenario 3: Test Custom con GUI

python TestEnvironment/scripts/GRIFO_M_PBIT.py --simulate --runs 50 --scenario random

Risultato: 50 run con scenario “Random Component Failures”, GUI real-time attiva

## 1.6 Vantaggi del Sistema

### Per Sviluppatori

Debug Rapido: Simulazione locale senza hardware

Regression Testing: Validazione automatica dopo modifiche codice

Coverage: Test estensivi su scenari rari (intermittent failures)

### Per Test Team

Automazione: Nessun intervento manuale durante esecuzione

Tracciabilità: Report dettagliati per ogni run

Visualizzazione: GUI mostra progresso in tempo reale

### Per Quality Assurance

Ripetibilità: Test identici eseguibili infinite volte

Scenari Configurabili: Simulazione di failure specifici

Statistiche Aggregate: CSV per analisi trend su centinaia di run

## 1.7 Limitazioni e Considerazioni

### Simulazione vs Hardware Reale

Simulazione: Veloce, deterministico, ma non sostituisce test su hardware

Hardware Reale: Cattura problemi timing-dependent, race conditions, failure intermittenti

### Copertura dei Test

Il sistema valida:
- ✅ Funzionalità PBIT (test diagnostici B6/B8)
- ✅ Messaggi seriali e log eventi
- ✅ Rilevamento target (scenario TgtGen)

Non valida:
- ❌ Funzionalità radar operative (tracking, modalità operative)
- ❌ Performance real-time in missione
- ❌ Integrazione con sistemi di piattaforma

### Manutenzione

Known Failures: Lista failure noti va mantenuta aggiornata

ICD Changes: Modifiche al protocollo 1553 richiedono aggiornamenti codice

Mock Updates: Mock objects devono riflettere comportamento hardware reale

← Torna all’Indice | Prossimo: Architettura Software →X

# — 02_Architettura_Software_e_Moduli.md —

# Capitolo 2: Architettura Software e Moduli

← Introduzione | Torna all’Indice | Prossimo: Script Principale →X

## 2.1 Struttura delle Directory

Il repository è organizzato con una chiara separazione tra codice di test, framework, simulatore e documentazione:

SXXXXXXX_PlatSim/
│
├── TestEnvironment/                    # ⭐ Core Test Environment
│   ├── env/                           # Framework modules
│   │   ├── leo_grifo_1553.py         # 1553 bus interface
│   │   ├── leo_grifo_terminal.py     # Serial parsing
│   │   ├── leo_grifo_io_box.py       # Power control
│   │   ├── leo_grifo_core.py         # Core utilities
│   │   ├── leo_grifo_test_report.py  # PDF generation
│   │   ├── test_common_function.py   # Common API
│   │   ├── brainbox_interface.py     # BrainBox abstraction
│   │   └── site-packages/            # Dependencies (reportlab, etc.)
│   │
│   ├── scripts/                       # Test scripts
│   │   ├── GRIFO_M_PBIT.py          # ⭐ Main test script
│   │   ├── GRIFO_M_PBIT_mock.py     # Mock implementation
│   │   ├── GRIFO_M_PBIT_gui.py      # GUI monitor
│   │   └── __init__.py               # Path bootstrap
│   │
│   ├── json/                          # Test scenarios & templates
│   │   ├── scenarios_template.json   # Scenario definitions
│   │   └── test_report_template.json # PDF report structure
│   │
│   ├── pdf_reports/                   # 📄 Generated outputs
│   └── LOG/                           # Test execution logs
│
├── PlatformSimulator/                 # Hardware simulator
│   ├── bin/                          # Embedded Python + binaries
│   │   ├── python.exe                # Python 3.8 embedded
│   │   ├── interpreter.py            # SWIG wrapper
│   │   ├── _interpreter.pyd          # Native 1553 library
│   │   └── DLLs/                     # Python dependencies
│   │
│   └── LeonardoCompany/              # Native libraries
│       └── x86_64-w64-mingw32/
│
├── _doc/                              # 📚 Documentation
│   ├── GRIFO_M_PBIT_Documentation/   # This documentation
│   ├── IMPLEMENTATION_SUMMARY.md
│   ├── LAUNCHER_GUIDE.md
│   └── ...
│
├── tests/                             # Unit tests & debug scripts
│   ├── test_tgt_gen_implementation.py
│   └── test_lazy_proxy.py
│
└── *.bat                              # 🚀 Launcher scripts
    ├── run_target.bat
    ├── run_simulate.bat
    └── run_simulate_simple.bat

### 2.1.1 Motivazioni Architetturali

Separazione env/ e scripts/:
- env/: Framework riutilizzabile per diversi test
- scripts/: Test specifici che usano il framework

Python Embedded in PlatformSimulator/:
- Permette esecuzione standalone senza installazione Python
- Include tutte le dipendenze (reportlab, pyserial)
- Garantisce versione Python consistente

Bootstrap in scripts/__init__.py:

# Aggiunge ../env a sys.path per import diretti
import sys
import os
sys.path.insert(0, os.path.join(os.path.dirname(__file__), '..', 'env'))
sys.path.insert(0, os.path.join(os.path.dirname(__file__), '..', 'env', 'site-packages'))

## 2.2 Moduli Core del Framework

### 2.2.1 leo_grifo_1553.py - Interfaccia Bus 1553

Responsabilità: Gestione comunicazione con radar via bus 1553 (MIL-STD-1553B)

Classi Principali:

class GrifoInstrumentInterface:
    """
    Interfaccia di alto livello per comunicazione 1553.
    Wrappa il modulo interpreter (SWIG) con API Python-friendly.
    """
    
    def __init__(self, configuration_file: str):
        """Carica configurazione messaggi da file XML/JSON"""
        pass
    
    def check(self, RT: int, expected_value: Any, 
              msg_name: str, field_name: str) -> Tuple[bool, str]:
        """Verifica valore campo su messaggio 1553"""
        pass
    
    def get(self, RT: int, msg_name: str, field_name: str) -> Any:
        """Legge valore campo da messaggio 1553"""
        pass
    
    def set(self, RT: int, value: Any, msg_name: str, 
            field_name: str, commitChanges: bool = False) -> Tuple[bool, str]:
        """Scrive valore campo su messaggio 1553"""
        pass
    
    def enable_notification(self, RT: int, msg_name: str):
        """Abilita notifiche per messaggio specifico"""
        pass

Singleton Globale:

|  |  |  | # In modalità normale (hardware) theGrifo1553 = GrifoInstrumentInterface("config_1553.xml") # In modalità simulate (mock) theGrifo1553 = MockGrifo1553Interface() # Stesso API, comportamento simulato |
| --- | --- | --- | --- |

Messaggi 1553 Principali:

Messaggio

RT

Direzione

Scopo

A1_MsgRdrSettingsAndParameters

1

BC→RT

Configurazione radar (intensità, gain, ecc.)

A2_MsgRdrOperationCommand

1

BC→RT

Comandi operativi (mode, PBIT, STBY)

A4_MsgNavDataAndCursor

1

BC→RT

Dati navigazione (heading, posizione)

A5_MsgInuHighSpeed

1

BC→RT

Dati INU ad alta frequenza

B6_MsgRdrSettingsAndParametersTellback

1

RT→BC

Tellback configurazione e BIT status

B8_MsgBitReport

1

RT→BC

Report dettagliato BIT (field per ogni test)

B9_MsgRdrTrackAndTargetsData

1

RT→BC

Dati target rilevati (range, azimuth)

Esempio Utilizzo:

from leo_grifo_1553 import theGrifo1553

# Invia comando PBIT
ret, err = setValue(theGrifo1553, 1, "A2_MsgRdrOperationCommand", 
                    "a2_radarMode", commitChanges=True)

# Verifica completamento PBIT
ret, err = check(theGrifo1553, "false", "B6_MsgRdrSettingsAndParametersTellback", 
                 "b6_t1_rdr_fail_status")

### 2.2.2 leo_grifo_terminal.py - Parsing Messaggi Seriali

Responsabilità: Acquisizione e parsing messaggi dalla porta seriale del radar

Classi Principali:

|  |  | class SerialMessageParser: """ Parser per messaggi seriali GRIFO. Riconosce pattern specifici (%%E, %%F, RECYCLE) e estrae dati. """ def __init__(self): self.statistics = { 'total_messages': 0, 'error_messages': 0, # %%E 'fatal_messages': 0, # %%F 'recycle_events': 0 # RECYCLE detected } def parse_message(self, line: str) -> Dict[str, Any]: """Analizza singola riga seriale e estrae informazioni""" pass def extract_statistics(self) -> Dict[str, int]: """Ritorna statistiche aggregate messaggi""" pass |
| --- | --- | --- |

Pattern Messaggi Riconosciuti:

Pattern

Significato

Azione

%%E

Error message

Incrementa error_messages, log con severità WARNING

%%F

Fatal error

Incrementa fatal_messages, log con severità ERROR

RECYCLE

Processor recycle event

Incrementa recycle_events, marca come evento critico

PBIT COMPLETED

PBIT concluso

Estrae tempo completamento

BIT AVAILABLE

BIT disponibile

Aggiorna stato disponibilità

Esempio Utilizzo:

from leo_grifo_terminal import SerialMessageParser

parser = SerialMessageParser()

# Durante test, per ogni riga seriale ricevuta
line = serial_port.readline().decode('utf-8')
msg_info = parser.parse_message(line)

# A fine test
stats = parser.extract_statistics()
print(f"Errors: {stats['error_messages']}, Fatal: {stats['fatal_messages']}")

### 2.2.3 leo_grifo_io_box.py - Controllo Alimentazione

Responsabilità: Gestione accensione/spegnimento radar tramite BrainBox

Classi Principali:

class BrainBoxInterface:
    """
    Interfaccia per controllo alimentazione radar via BrainBox.
    """
    
    def __init__(self, port: str = "COM3"):
        """Inizializza connessione seriale con BrainBox"""
        pass
    
    def power_on(self, channel: int = 1) -> bool:
        """Accende canale specificato"""
        pass
    
    def power_off(self, channel: int = 1) -> bool:
        """Spegne canale specificato"""
        pass
    
    def get_status(self, channel: int = 1) -> str:
        """Legge stato corrente canale (ON/OFF)"""
        pass

Singleton Globale:

theBrainBox = BrainBoxInterface(port="COM3")

Funzioni Helper:

def power_grifo_on(wait_after: float = 2.0):
    """
    Accende radar e attende stabilizzazione.
    
    Args:
        wait_after: Secondi di attesa dopo accensione (default 2.0)
    """
    theBrainBox.power_on(channel=1)
    time.sleep(wait_after)

def power_grifo_off(wait_before: float = 1.0, wait_after: float = 5.0):
    """
    Spegne radar con timing appropriato.
    
    Args:
        wait_before: Secondi di attesa prima dello spegnimento
        wait_after: Secondi di attesa dopo spegnimento (per discharge condensatori)
    """
    time.sleep(wait_before)
    theBrainBox.power_off(channel=1)
    time.sleep(wait_after)

### 2.2.4 leo_grifo_core.py - Utilities e Recorder

Responsabilità: Funzioni di supporto e registrazione step di test

Classi Principali:

class TestRecorder:
    """
    Registra step di test per generazione report finale.
    Pattern Singleton.
    """
    
    def __init__(self):
        self.steps = []
        self.session_data = {}
    
    def add_step(self, step_name: str, result: str, 
                 details: Dict[str, Any] = None):
        """Aggiunge step di test con risultato"""
        pass
    
    def close_session(self, overall_result: str):
        """Chiude sessione test e prepara dati per report"""
        pass
    
    def get_session_data(self) -> Dict[str, Any]:
        """Ritorna dati sessione per generazione report"""
        pass

Singleton Globale:

theRecorder = TestRecorder()

Funzioni Utility:

def timestamp_iso() -> str:
    """Ritorna timestamp ISO 8601 corrente"""
    return datetime.now().isoformat()

def format_duration(seconds: float) -> str:
    """Formatta durata in formato leggibile (es. "1h 23m 45s")"""
    pass

def safe_cast(value: Any, type_func: Callable) -> Optional[Any]:
    """Cast sicuro con gestione eccezioni"""
    try:
        return type_func(value)
    except (ValueError, TypeError):
        return None

### 2.2.5 test_common_function.py - API Comuni

Responsabilità: Fornisce API uniforme per interazione con tutti gli strumenti

Funzioni Principali:

def check(instrument, expected_value, msg_name: str, 
          field_name: str) -> Tuple[bool, str]:
    """
    Verifica valore campo su qualsiasi strumento.
    Wrapper generico che delega a instrument.check().
    
    Returns:
        (success: bool, error_message: str)
    """
    return instrument.check(1, expected_value, msg_name, field_name)

def getValue(instrument, msg_name: str, field_name: str) -> Any:
    """
    Legge valore campo da qualsiasi strumento.
    """
    return instrument.get(1, msg_name, field_name)

def setValue(instrument, value: Any, msg_name: str, field_name: str,
             commitChanges: bool = False) -> Tuple[bool, str]:
    """
    Scrive valore campo su qualsiasi strumento.
    """
    return instrument.set(1, value, msg_name, field_name, commitChanges)

Vantaggi dell’Astrazione:
- Test code non dipende da implementazione specifica strumento
- Facilita mocking (basta sostituire singleton con mock che rispetta API)
- Permette logging centralizzato di tutte le operazioni

### 2.2.6 leo_grifo_test_report.py - Generazione PDF

Responsabilità: Generazione report PDF strutturati con tabelle e grafici

Classi Principali:

class PDFClass:
    """
    Generatore report PDF usando ReportLab.
    """
    
    def __init__(self):
        self.pdf_buffer = None
        self.template = None
    
    def load_template(self, json_template_path: str):
        """Carica template JSON per struttura report"""
        pass
    
    def pdf_generate(self, json_test_data: Dict[str, Any]) -> str:
        """
        Genera PDF da dati test.
        
        Returns:
            path: Path del file PDF generato
        """
        pass
    
    def pdf_add_per_run_summary(self, runs_data: List[Dict]):
        """Aggiunge tabella riepilogo run"""
        pass
    
    def pdf_add_statistics_chart(self, stats: Dict):
        """Aggiunge grafico statistiche aggregate"""
        pass

Singleton Globale:

testReport = PDFClass()

Workflow Generazione:
1. Test script popola dati in formato JSON
2. testReport.load_template() carica struttura report
3. testReport.pdf_generate() crea PDF con tabelle/grafici
4. File salvato in TestEnvironment/pdf_reports/

## 2.3 Diagramma delle Dipendenze

┌─────────────────────────────────────────────────────────────┐
│                    GRIFO_M_PBIT.py                          │
│                   (Test Orchestrator)                        │
└────────┬─────────────────────────────────────┬──────────────┘
         │                                     │
         │ imports                             │ imports
         ↓                                     ↓
┌─────────────────────┐              ┌────────────────────────┐
│ test_common_        │              │ GRIFO_M_PBIT_mock.py   │
│ function.py         │              │ (Simulation Logic)     │
│ - check()           │              └────────────────────────┘
│ - getValue()        │                         │
│ - setValue()        │                         │ replaces singletons
└──────┬──────────────┘                         ↓
       │                              ┌────────────────────────┐
       │ delegates                    │ Mock Objects           │
       ↓                              │ - MockGrifo1553        │
┌─────────────────────┐              │ - MockBrainBox         │
│ Singleton Instances │              └────────────────────────┘
│ - theGrifo1553      │←──────────────────────┘
│ - theBrainBox       │
│ - theRecorder       │
└──────┬──────────────┘
       │ implemented by
       ↓
┌──────────────────────────────────────────────────────────┐
│           Framework Modules (env/)                        │
├──────────────────┬───────────────────┬───────────────────┤
│ leo_grifo_       │ leo_grifo_        │ leo_grifo_        │
│ 1553.py          │ terminal.py       │ io_box.py         │
└──────┬───────────┴──────┬────────────┴──────┬────────────┘
       │                  │                   │
       │ uses             │ uses              │ uses
       ↓                  ↓                   ↓
┌──────────────────────────────────────────────────────────┐
│         Hardware Abstraction (bin/)                       │
├──────────────────┬───────────────────┬───────────────────┤
│ interpreter.py   │ pySerial          │ BrainBox Protocol │
│ (SWIG wrapper)   │                   │                   │
└──────┬───────────┴───────────────────┴───────────────────┘
       │
       │ calls native code
       ↓
┌──────────────────────────────────────────────────────────┐
│              Native Libraries                             │
│  - _interpreter.pyd (1553 driver DLL)                    │
└──────────────────────────────────────────────────────────┘

## 2.4 Pattern Architetturali

### 2.4.1 Singleton Pattern

Motivazione: Garantire unica istanza di interfacce hardware condivisa tra funzioni

Implementazione:

# leo_grifo_1553.py
_instance = None

def get_grifo_1553_instance():
    global _instance
    if _instance is None:
        _instance = GrifoInstrumentInterface("config.xml")
    return _instance

theGrifo1553 = get_grifo_1553_instance()

Pro:
- Evita re-inizializzazioni costose (apertura porte, caricamento config)
- Stato condiviso (statistiche, buffer messaggi)

Contro:
- Testing complicato (stato globale)
- Risolto con monkey patching in modalità simulate

### 2.4.2 Facade Pattern

Motivazione: Semplificare interfaccia complessa 1553/seriale con API high-level

Implementazione: test_common_function.py fa da facade:

# Invece di:
instrument.get(1, "B6_MsgRdrSettingsAndParametersTellback", "b6_t1_rdr_fail_status")

# User code usa:
getValue(theGrifo1553, "B6_MsgRdrSettingsAndParametersTellback", "b6_t1_rdr_fail_status")

### 2.4.3 Strategy Pattern (Simulation)

Motivazione: Intercambiare implementazione hardware/mock senza cambiare test code

Implementazione:

# GRIFO_M_PBIT_mock.py definisce mock con stessa interfaccia
class MockGrifo1553Interface:
    def check(self, RT, expected, msg, field):
        # Logica simulata
        return (True, "")  # Simula successo
    
    def get(self, RT, msg, field):
        # Ritorna valore simulato basato su scenario
        return self._simulated_values.get(field, 0)

# In GRIFO_M_PBIT.py con --simulate
if args.simulate:
    from GRIFO_M_PBIT_mock import MockGrifo1553Interface, MockBrainBox
    theGrifo1553 = MockGrifo1553Interface()
    theBrainBox = MockBrainBox()

## 2.5 Gestione Dipendenze

### 2.5.1 Dipendenze Python

Librerie Richieste:
- reportlab (≥3.6): Generazione PDF
- pyserial (≥3.5): Comunicazione seriale
- tkinter (built-in): GUI

Installazione:

# Se usi Python di sistema
pip install reportlab pyserial

# In embedded Python (già incluse in site-packages/)
# Nessuna installazione necessaria

### 2.5.2 Dipendenze Native

Librerie C/C++:
- _interpreter.pyd: Driver 1553 compilato (DLL Windows)
- PlatformSimulator/LeonardoCompany/: Librerie native aggiuntive

Binding SWIG:

# interpreter.py (generato automaticamente da SWIG)
import _interpreter  # Importa DLL nativa

class Msg1553:
    """Wrapper Python per classe C++ Msg1553"""
    def __init__(self, *args):
        self.this = _interpreter.new_Msg1553(*args)

### 2.5.3 Path Management

Bootstrap in scripts/__init__.py:

# Eseguito automaticamente quando si importa da TestEnvironment/scripts/
import sys
import os

# Aggiunge ../env a sys.path
script_dir = os.path.dirname(__file__)
env_dir = os.path.abspath(os.path.join(script_dir, '..', 'env'))
site_packages = os.path.join(env_dir, 'site-packages')

sys.path.insert(0, env_dir)
sys.path.insert(0, site_packages)

Risultato: Import diretti funzionano ovunque:

from leo_grifo_1553 import theGrifo1553  # ✅ Funziona
from test_common_function import check   # ✅ Funziona

## 2.6 Convenzioni di Naming

### 2.6.1 Moduli

leo_grifo_*: Moduli framework specifici GRIFO

test_common_*: Utilities generiche riutilizzabili

### 2.6.2 Singleton

Pattern the<Nome>: theGrifo1553, theBrainBox, theRecorder

Indica istanza globale unica

### 2.6.3 Messaggi 1553

Prefisso A<n>: Messaggi BC→RT (comandi)

Prefisso B<n>: Messaggi RT→BC (telemetria)

Esempio: A2_MsgRdrOperationCommand, B6_MsgRdrSettingsAndParametersTellback

### 2.6.4 Campi Messaggi

Formato <msg_prefix>_<field_name>: b6_t1_rdr_fail_status

Underscore separatore

Snake_case per tutti i field

## 2.7 Thread Safety e Concurrency

### 2.7.1 GUI Threading

Problema: GUI Tkinter deve girare su main thread, test su worker thread

Soluzione: Queue-based communication

# GRIFO_M_PBIT_gui.py
class TestMonitorGUI:
    def __init__(self):
        self.update_queue = queue.Queue()  # Thread-safe
    
    def update_run(self, run, **kwargs):
        """Chiamato da test thread"""
        self.update_queue.put(('run_row', {'run': run, ...}))
    
    def _poll_updates(self):
        """Eseguito su main thread (Tkinter)"""
        while not self.update_queue.empty():
            update_type, data = self.update_queue.get_nowait()
            self._apply_update(update_type, data)
        
        self.root.after(100, self._poll_updates)  # Re-schedule

### 2.7.2 Hardware Access

Nota: Accesso hardware NON è thread-safe
- Tutti i comandi 1553/seriale da singolo thread (test thread)
- GUI legge solo dati processati (no accesso diretto hardware)

← Introduzione | Torna all’Indice | Prossimo: Script Principale →X

# — 03_Script_Principale_GRIFO_M_PBIT.md —

# Capitolo 3: Script Principale GRIFO_M_PBIT.py

← Architettura Software | Torna all’Indice | Prossimo: Workflow Test →X

## 3.1 Overview dello Script

GRIFO_M_PBIT.py è lo script principale che orchestra l’intero processo di test automatizzato. Con ~2500 righe di codice, è responsabile di:

Gestione ciclo di vita test: Power cycling, timing, ripetizioni

Orchestrazione scenari: Configurazione failure simulati

Monitoring multi-canale: 1553, seriale, power control

Aggregazione statistiche: B6/B8/Serial/Target metrics

Generazione report: PDF, CSV, JSON output

|  | ### 3.1.1 Caratteristiche Chiave |
| --- | --- |

Caratteristica

Descrizione

LOC

~2500 linee

Funzioni principali

15+ funzioni di test

Scenari supportati

6 (Random, Recycle, Perfect, TgtGen, ecc.)

Output formati

PDF, CSV, JSON

Modalità esecuzione

Hardware (--target) / Simulazione (--simulate)

GUI integration

Thread-safe real-time monitoring

## 3.2 Struttura del File

### 3.2.1 Organizzazione Sezioni

# ============================================================
# SEZIONE 1: HEADER E DOCUMENTAZIONE (righe 1-40)
# ============================================================
"""
Docstring completo con:
- Test flow description
- Configuration options
- Timing constants
- Author & version info
"""

# ============================================================
# SEZIONE 2: IMPORT E DIPENDENZE (righe 41-65)
# ============================================================
import __init__  # Path bootstrap
from leo_grifo_common import *
from test_common_function import *
from leo_grifo_1553 import theGrifo1553
from leo_grifo_test_report import testReport
# ... altri import

# ============================================================
# SEZIONE 3: CONFIGURAZIONE E COSTANTI (righe 66-150)
# ============================================================
NUMBER_OF_REPETITIONS = 10
PBIT_SEC_TIME = 182
COMM_LOSS_THRESHOLD = 20
KNOWN_FAILURES = [...]
TARGET_DETECTION_TIMEOUT_SEC = 20.0
# ... altre configurazioni

# ============================================================
# SEZIONE 4: SCENARI DI TEST (righe 151-300)
# ============================================================
SCENARIOS = {
    'random': {...},
    'recycle': {...},
    'perfect': {...},
    # ... altri scenari
}

# ============================================================
# SEZIONE 5: FUNZIONI HELPER (righe 301-700)
# ============================================================
def radar_setup():
    """Configura messaggi 1553 iniziali"""
    
def enter_pbit():
    """Invia comando PBIT al radar"""
    
def wait_pbit_completion():
    """Attende completamento PBIT con timeout"""
    
def check_comm_loss():
    """Verifica perdita comunicazione 1553"""

# ============================================================
# SEZIONE 6: TEST B6/B8 (righe 701-1500)
# ============================================================
def verify_b6_status():
    """Verifica 12 campi B6 LRU status"""
    
def drill_down_b8():
    """Drill-down 185 campi diagnostici B8"""

# ============================================================
# SEZIONE 7: TEST TARGET (righe 1501-1900)
# ============================================================
def tgt_gen(scenario_config):
    """Test rilevamento target (opzionale)"""

# ============================================================
# SEZIONE 8: MAIN TEST LOOP (righe 1901-2400)
# ============================================================
def main_test():
    """Loop principale test con power cycling"""
    for repetition in range(NUMBER_OF_REPETITIONS):
        # Power cycle
        # Execute PBIT
        # Verify B6/B8
        # Check serial
        # Target test (if enabled)
        # Update GUI
        # Aggregate statistics

# ============================================================
# SEZIONE 9: REPORT E STATISTICHE (righe 2401-2494)
# ============================================================
def generate_final_statistics_report():
    """Aggrega statistiche e genera report finale"""
    
def export_statistics_to_csv():
    """Esporta statistiche in CSV"""

if __name__ == '__main__':
    main_test()

## 3.3 Flusso di Esecuzione Principale

### 3.3.1 Diagramma di Flusso Completo

┌─────────────────────────────────────────────────────┐
│                  START PROGRAM                       │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  Parse Command Line Arguments                        │
│  - --simulate / --target                             │
│  - --runs N                                          │
│  - --scenario <name>                                 │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  Load Scenario Configuration                         │
│  - Select from SCENARIOS dict                        │
│  - Apply mock replacements if --simulate             │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  Initialize Components                               │
│  - testReport (PDF generator)                        │
│  - gui_monitor (if not disabled)                     │
│  - Serial terminal connection                        │
│  - Statistics collectors                             │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│         MAIN TEST LOOP                               │
│  for repetition in range(NUMBER_OF_REPETITIONS):    │
└────────────┬────────────────────────────────────────┘
             │
             ↓
     ┌───────────────┐
     │ Run N/TOTAL   │
     └───────┬───────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  PHASE 1: POWER CYCLING                              │
│  ├─ power_grifo_off(wait_before=1s, wait_after=4s)  │
│  ├─ power_grifo_on(wait_after=1.5s)                 │
│  └─ Wait radar initialization                        │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  PHASE 2: RADAR SETUP                                │
│  ├─ radar_setup()                                    │
│  │   ├─ Set A1 intensity = 127                      │
│  │   ├─ Set A2 mode = PBIT                          │
│  │   ├─ Set A4 nav data (heading, position)         │
│  │   └─ Set A5 INU mode word                        │
│  └─ Enable B6/B8/B9 notifications                    │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  PHASE 3: PBIT EXECUTION                             │
│  ├─ enter_pbit() → Send PBIT command                │
│  ├─ wait_pbit_completion(timeout=182s)              │
│  │   ├─ Poll B6 message                             │
│  │   ├─ Check bit_available flag                    │
│  │   └─ Monitor comm_loss                           │
│  └─ Record PBIT time                                 │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  PHASE 4: B6 VERIFICATION                            │
│  ├─ verify_b6_status()                               │
│  │   ├─ Check 12 LRU status fields                  │
│  │   ├─ Classify: PASS / FAIL / KNOWN               │
│  │   └─ Track: b6_pass, b6_fail, b6_known_fail      │
│  └─ Update statistics                                │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  PHASE 5: B8 DRILL-DOWN (conditional)                │
│  ├─ IF real_failures > 0:                            │
│  │   └─ drill_down_b8()                              │
│  │       ├─ Check 185 diagnostic fields              │
│  │       ├─ Track: b8_pass, b8_fail, b8_checked     │
│  │       └─ Record failure details                   │
│  └─ ELSE: Skip B8 (known failures only)             │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  PHASE 6: SERIAL MONITORING                          │
│  ├─ Parse serial buffer                              │
│  ├─ Count: errors (%%E), fatal (%%F), recycle       │
│  └─ Update: serial_errors, serial_fatal, recycles    │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  PHASE 7: TARGET TEST (optional)                     │
│  ├─ IF scenario has target_gen:                      │
│  │   └─ tgt_gen(scenario_config)                     │
│  │       ├─ Set A2 mode = OPERATE                    │
│  │       ├─ Inject A4/A5 target parameters           │
│  │       ├─ Monitor B9 for target detection          │
│  │       ├─ Verify range within tolerance            │
│  │       └─ Record: target_detected, range, cycle    │
│  └─ Update: target_simulated in run_stats           │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  PHASE 8: RUN FINALIZATION                           │
│  ├─ Aggregate run_stats:                             │
│  │   ├─ success (PASS/FAIL)                         │
│  │   ├─ pbit_time                                    │
│  │   ├─ b6_pass/fail/known                          │
│  │   ├─ b8_pass/fail/checked                        │
│  │   ├─ serial_errors/fatal/recycles                │
│  │   ├─ target_detected/range/iterations            │
│  │   └─ failures list                                │
│  ├─ Update GUI (gui_monitor.update_run())            │
│  └─ Append to test_statistics[]                      │
└────────────┬────────────────────────────────────────┘
             │
             ↓
     ┌───────────────┐
     │ More runs?    │
     └───────┬───────┘
             │ Yes
             ↓ (loop)
             │ No
             ↓
┌─────────────────────────────────────────────────────┐
│  FINAL REPORT GENERATION                             │
│  ├─ generate_final_statistics_report()               │
│  │   ├─ Aggregate all run_stats                     │
│  │   ├─ Calculate averages/percentages               │
│  │   └─ Format per-run summary table                │
│  ├─ testReport.pdf_generate()                        │
│  │   ├─ Create PDF with tables/charts               │
│  │   └─ Save to pdf_reports/                        │
│  ├─ export_statistics_to_csv()                       │
│  │   └─ Save CSV to pdf_reports/                    │
│  └─ Export JSON statistics                           │
└────────────┬────────────────────────────────────────┘
             │
             ↓
┌─────────────────────────────────────────────────────┐
│  CLEANUP                                             │
│  ├─ power_grifo_off()                                │
│  ├─ terminal.disconnect()                            │
│  ├─ gui_monitor.show_results()                       │
│  └─ EXIT                                             │
└─────────────────────────────────────────────────────┘

### 3.3.2 Stati del Test

Durante l’esecuzione, il test attraversa diversi stati:

class TestState(Enum):
    INIT = "Initialization"
    POWER_CYCLING = "Power Cycling"
    RADAR_SETUP = "Radar Setup"
    PBIT_EXECUTION = "PBIT Execution"
    B6_VERIFICATION = "B6 Verification"
    B8_DRILLDOWN = "B8 Drill-Down"
    SERIAL_CHECK = "Serial Check"
    TARGET_TEST = "Target Test"
    RUN_FINALIZATION = "Run Finalization"
    REPORT_GENERATION = "Report Generation"
    CLEANUP = "Cleanup"

## 3.4 Funzioni Principali

### 3.4.1 main_test() - Entry Point

def main_test():
    """
    Main test orchestration function.
    
    Workflow:
        1. Parse command line arguments
        2. Load scenario configuration
        3. Initialize components (report, GUI, serial)
        4. Execute test loop (N repetitions)
        5. Generate final report (PDF/CSV/JSON)
        6. Cleanup and exit
    
    Returns:
        bool: True if all runs passed, False otherwise
    """

Responsabilità:
- Parsing argomenti CLI (--simulate, --runs, --scenario)
- Inizializzazione singleton (testReport, gui_monitor)
- Gestione eccezioni globali e interrupt (Ctrl-C)
- Coordinamento fasi test

Gestione Interrupt:

def signal_handler(sig, frame):
    """Handle Ctrl-C gracefully"""
    global interruptRequest
    interruptRequest = True
    logging.warning("Interrupt signal received - will stop after current run")

signal.signal(signal.SIGINT, signal_handler)

### 3.4.2 radar_setup() - Configurazione 1553

def radar_setup():
    """
    Configure radar via 1553 messages before PBIT.
    
    Sets:
        - A1: Intensity = 127 (maximum)
        - A2: Mode = STANDBY initially
        - A4: Navigation data (heading, position)
        - A5: INU mode word
    
    Returns:
        bool: True if setup successful
    """

Messaggi Configurati:
| Messaggio | Campo | Valore | Scopo |
|———–|——-|——–|——-|
| A1 | a1_display_intensity | 127 | Intensità display massima |
| A2 | a2_radarMode | 1 (STBY) | Modalità iniziale STANDBY |
| A4 | a4_heading | 1060 | Heading corrente (ICD units) |
| A4 | a4_x_pos, a4_y_pos | 2500, 2500 | Posizione piattaforma |
| A5 | a5_inu_mode_word | 0xAA54 | INU mode word valido |

### 3.4.3 enter_pbit() - Comando PBIT

def enter_pbit():
    """
    Send PBIT command to radar via A2 message.
    
    Sequence:
        1. Set a2_radarMode = 2 (OPERATE)
        2. Set a2_radarModeInitiation = 1 (PBIT)
        3. Wait 100ms for radar to register command
    
    Returns:
        bool: True if command sent successfully
    """

Implementazione:

# Set OPERATE mode
setValue(theGrifo1553, 2, "A2_MsgRdrOperationCommand", 
         "a2_radarMode", commitChanges=True)

# Trigger PBIT
setValue(theGrifo1553, 1, "A2_MsgRdrOperationCommand",
         "a2_radarModeInitiation", commitChanges=True)

time.sleep(0.1)  # Allow radar to process command

### 3.4.4 wait_pbit_completion() - Attesa PBIT

def wait_pbit_completion(timeout_sec: float = PBIT_SEC_TIME):
    """
    Wait for PBIT to complete with timeout.
    
    Monitors:
        - B6 bit_available flag (True when PBIT done)
        - 1553 message counter (comm loss detection)
    
    Args:
        timeout_sec: Maximum wait time (default 182s)
    
    Returns:
        Tuple[bool, float, str]:
            - success: True if PBIT completed
            - elapsed_time: Time taken for PBIT
            - status: 'completed' / 'timeout' / 'comm_loss'
    """

Logica Polling:

start_time = time.time()
last_msg_count = 0
no_change_iterations = 0

while (time.time() - start_time) < timeout_sec:
    # Check bit_available flag
    bit_available = getValue(theGrifo1553, "B6_MsgRdrSettingsAndParametersTellback",
                             "b6_t1_bit_available")
    
    if bit_available == "true":
        elapsed = time.time() - start_time
        return (True, elapsed, 'completed')
    
    # Check comm loss (message counter not incrementing)
    current_msg_count = theGrifo1553.get_message_count("B6")
    if current_msg_count == last_msg_count:
        no_change_iterations += 1
        if no_change_iterations >= COMM_LOSS_THRESHOLD:
            return (False, time.time() - start_time, 'comm_loss')
    else:
        no_change_iterations = 0
        last_msg_count = current_msg_count
    
    time.sleep(0.1)  # Poll every 100ms

# Timeout
return (False, timeout_sec, 'timeout')

### 3.4.5 verify_b6_status() - Verifica B6

def verify_b6_status(known_failures: List[str]):
    """
    Verify all 12 B6 LRU status fields.
    
    Fields Checked:
        - b6_t1_rdr_fail_status
        - b6_t1_processor_1_status
        - b6_t1_processor_2_status
        - b6_t1_receiver_status
        - b6_t1_exciter_status
        - b6_t1_transmitter_status
        - b6_t1_antenna_drive_unit_status
        - b6_t1_power_supply_status
        - b6_t1_auxilliary_processor_status
        - b6_t1_waveguide_pressurization_status
        - b6_t1_auxilliary_siu_status
        - b6_t1_receiver_transmitter_status
    
    Args:
        known_failures: List of expected failures (HW setup limitations)
    
    Returns:
        Dict:
            {
                'b6_pass': int,
                'b6_fail': int,
                'b6_known_fail': int,
                'failures': List[Tuple[str, str]]  # (field, value)
            }
    """

Classificazione Failure:

for field in B6_FIELDS:
    ret, err = check(theGrifo1553, "false", "B6_MsgRdrSettingsAndParametersTellback", field)
    
    if ret:  # Field = "false" (no failure)
        b6_pass += 1
    else:    # Field = "true" (failure detected)
        if field in known_failures:
            b6_known_fail += 1  # Expected failure
        else:
            b6_fail += 1  # Unexpected failure
            failures.append((field, "true"))

### 3.4.6 drill_down_b8() - Drill-Down B8

def drill_down_b8():
    """
    Perform detailed B8 diagnostic drill-down.
    
    Checks 185 diagnostic fields for detailed failure analysis.
    Only executed when real (non-known) failures detected.
    
    Returns:
        Dict:
            {
                'b8_pass': int,
                'b8_fail': int,
                'b8_checked': int,
                'b8_failures': List[Tuple[str, str]]
            }
    """

Performance Note: B8 drill-down può richiedere 5-10 secondi per 185 field checks.

### 3.4.7 tgt_gen() - Test Target Generation

def tgt_gen(scenario_config: Dict) -> Dict:
    """
    Execute target detection test (optional).
    
    Workflow:
        1. Set A2 mode = OPERATE
        2. Configure A4/A5 for target scenario:
           - a4_heading: Target bearing
           - a4_x_pos, a4_y_pos: Platform position
           - a5_inu_mode_word: INU valid
        3. Monitor B9 message for target reports
        4. Verify range within tolerance
        5. Count successful detections
    
    Args:
        scenario_config: Scenario dict with target_gen configuration
    
    Returns:
        Dict:
            {
                'target_detected': bool,
                'target_range': int (meters),
                'target_iterations': int (cycle found),
                'target_test_time': float (seconds)
            }
    """

Esempio Configurazione Target:

scenario_config = {
    'target_gen': {
        'distance': 2000,  # meters
        'appeared_after_cycles': 3,  # Visible dopo 3 cicli B9
        'heading': 1060,
        'x_pos': 2500,
        'y_pos': 2500
    }
}

### 3.4.8 generate_final_statistics_report() - Report Finale

def generate_final_statistics_report(report, test_statistics: List[Dict]):
    """
    Aggregate statistics from all runs and format for report.
    
    Calculates:
        - Total runs: N
        - Pass rate: X% (runs with success=True)
        - Fail rate: Y%
        - Average PBIT time: Xs
        - B6/B8/Serial aggregates
        - Per-run summary table
    
    Args:
        report: TestReport instance
        test_statistics: List of run_stats dicts
    
    Returns:
        Dict: Formatted statistics for PDF/CSV/JSON export
    """

## 3.5 Gestione Scenari

### 3.5.1 Struttura Scenario

Ogni scenario definisce:
- Nome e descrizione
- Failure da iniettare (per simulazione)
- Configurazione target (opzionale)

SCENARIOS = {
    'random': {
        'name': 'Random Component Failures',
        'description': 'Random selection of component failures (30% proc, 20% rx, 15% rx)',
        'expected_failures': [
            'processor_status',
            'transmitter_status',
            'receiver_status'
        ],
        'failure_probability': {
            'processor_status': 0.30,
            'transmitter_status': 0.20,
            'receiver_status': 0.15
        },
        'target_gen': None  # No target test
    },
    
    'tgtgen': {
        'name': 'Target Generation Test',
        'description': 'PBIT + Target detection test at 2000m',
        'expected_failures': [],
        'target_gen': {
            'distance': 2000,
            'appeared_after_cycles': 3,
            'heading': 1060,
            'x_pos': 2500,
            'y_pos': 2500
        }
    },
    
    'perfect': {
        'name': 'Perfect Run (No Failures)',
        'description': 'All tests pass - ideal scenario',
        'expected_failures': [],
        'target_gen': None
    }
}

### 3.5.2 Selezione Scenario

# Command line
python GRIFO_M_PBIT.py --simulate --scenario random

# In code
selected_scenario = SCENARIOS.get(args.scenario, SCENARIOS['random'])

## 3.6 Integrazione GUI

### 3.6.1 Inizializzazione GUI

if not args.no_gui:
    from GRIFO_M_PBIT_gui import TestMonitorGUI
    gui_monitor = TestMonitorGUI()
    gui_monitor.start()  # Lancia GUI su thread separato
else:
    gui_monitor = None

### 3.6.2 Aggiornamenti GUI Thread-Safe

# Durante test (worker thread)
gui_monitor.update_status(
    run_current=repetition+1,
    run_total=NUMBER_OF_REPETITIONS,
    pbit_time=pbit_time,
    pbit_available=True,
    power_on=True
)

gui_monitor.update_run(
    run=repetition+1,
    result='PASS' if success else 'FAIL',
    pbit=pbit_time,
    b6_pass=b6_pass,
    b6_fail=b6_fail,
    # ... altri parametri
)

# GUI thread processa queue e aggiorna widgets

## 3.7 Output e Naming Convention

### 3.7.1 Formato Nome File

Tutti gli output seguono il pattern:

{TEST_ID}_{YYYYMMDD}_{HHMMSS}_{suffix}.{ext}

Esempio:

GRIFO_M_PBIT_20260204_143052_report.pdf
GRIFO_M_PBIT_20260204_143052_statistic.csv
GRIFO_M_PBIT_20260204_143052_statistic.json

Vantaggi:
- Ordinamento alfabetico = ordinamento cronologico
- ID univoco tramite timestamp con secondi
- Suffix identifica tipo file (report / statistic)

### 3.7.2 Contenuto Output

PDF Report:
- Tabella per-run summary (8 colonne: Run, Status, PBIT Time, B6, B8, Serial, Target, Target Test)
- Statistiche aggregate (totali, medie, percentuali)
- Grafici (opzionale)

CSV Statistic:
- Header con nomi colonne
- Riga per ogni run
- Campi separati da virgola

JSON Statistic:
- Struttura gerarchica completa
- Include metadata (scenario, timestamp, ecc.)
- Array runs[] con dati di ogni run

## 3.8 Error Handling e Robustezza

### 3.8.1 Gestione Eccezioni

try:
    # Test execution
    success = execute_run()
except KeyboardInterrupt:
    # Ctrl-C: graceful shutdown
    logging.warning("Test interrupted by user")
    break
except Exception as e:
    # Unexpected error: log and continue
    logging.error(f"Run {repetition+1} failed with exception: {e}")
    logging.error(traceback.format_exc())
    run_stats['success'] = False
    run_stats['exception'] = str(e)
finally:
    # Always cleanup (power off, disconnect serial)
    cleanup()

### 3.8.2 Timeout Protection

Tutti i wait hanno timeout:

# PBIT completion
success, time, status = wait_pbit_completion(timeout_sec=182)

# Target detection
target_found = wait_target_detection(timeout_sec=20)

# Serial read
line = serial_port.readline(timeout=1.0)

### 3.8.3 Comm Loss Detection

# Rileva perdita comunicazione 1553 (radar spento/crashed)
if no_new_messages_for(iterations=20):
    logging.error("1553 comm loss detected - aborting run")
    return False

← Architettura Software | Torna all’Indice | Prossimo: Workflow Test →X

# — 04_Sistema_Test_Workflow_Logica.md —

# Capitolo 4: Sistema di Test - Workflow e Logica

← Script Principale | Torna all’Indice | Prossimo: Modalità Esecuzione →X

## 4.1 Ciclo di Test Completo

Ogni run di test segue un workflow standardizzato con 7 fasi principali. Questo approccio garantisce ripetibilità e tracciabilità completa.

### 4.1.1 Timeline Tipica di una Run

Time    Phase                  Activity                          Duration
═════════════════════════════════════════════════════════════════════════
00:00   Power Cycling          Power OFF radar                   ~5s
        └─ wait_before=1s      Stabilizzazione pre-spegnimento
        └─ wait_after=4s       Scarica condensatori
        
00:05   Power Cycling          Power ON radar                    ~2s
        └─ wait_after=1.5s     Inizializzazione radar
        
00:07   Radar Setup            Configura messaggi 1553           ~1s
        └─ A1/A2/A4/A5         Intensità, mode, nav, INU
        
00:08   PBIT Execution         Invia comando PBIT                ~15-20s
        └─ enter_pbit()        Attiva Built-In Test
        └─ wait_completion()   Polling bit_available flag
        
00:28   B6 Verification        Verifica 12 campi LRU status      ~1s
        └─ verify_b6_status()  Classifica PASS/FAIL/KNOWN
        
00:29   B8 Drill-Down          Drill-down 185 campi diagnostici  ~5-10s
        └─ drill_down_b8()     (solo se real failures)
        
00:39   Serial Check           Parsing buffer seriale            ~1s
        └─ extract_stats()     Conta %%E, %%F, RECYCLE
        
00:40   Target Test            Rilevamento target (opzionale)    ~15-20s
        └─ tgt_gen()           Monitoring B9 messages
        
01:00   Finalization           Aggregazione statistiche          ~1s
        └─ update_gui()        Aggiorna interfaccia
═════════════════════════════════════════════════════════════════════════
TOTAL:  ~60 secondi per run (senza target) / ~80s (con target)

### 4.1.2 Diagramma Stati Run

┌──────────────┐
                    │  RUN START   │
                    └──────┬───────┘
                           │
                    ┌──────▼───────┐
                    │ POWER_OFF    │──┐ Failure: Abort Run
                    └──────┬───────┘  │
                           │          │
                    ┌──────▼───────┐  │
                    │ POWER_ON     │──┤
                    └──────┬───────┘  │
                           │          │
                    ┌──────▼───────┐  │
                    │ RADAR_SETUP  │──┤
                    └──────┬───────┘  │
                           │          │
                    ┌──────▼───────┐  │
                    │ PBIT_START   │──┤
                    └──────┬───────┘  │
                           │          │
              ┌────────────▼────────┐ │
              │ PBIT_WAIT_COMPLETE  │─┤
              └────────────┬────────┘ │
                           │          │
                    ┌──────▼───────┐  │
                    │ B6_CHECK     │  │
                    └──────┬───────┘  │
                           │          │
                  ┌────────▼────────┐ │
                  │ B8_DRILLDOWN?   │ │
                  │ (if failures)   │ │
                  └────────┬────────┘ │
                           │          │
                    ┌──────▼───────┐  │
                    │ SERIAL_CHECK │  │
                    └──────┬───────┘  │
                           │          │
                  ┌────────▼────────┐ │
                  │ TARGET_TEST?    │ │
                  │ (if enabled)    │ │
                  └────────┬────────┘ │
                           │          │
                    ┌──────▼───────┐  │
                    │  FINALIZE    │◄─┘
                    └──────┬───────┘
                           │
                    ┌──────▼───────┐
                    │   RUN END    │
                    │ (success or  │
                    │   failure)   │
                    └──────────────┘

## 4.2 Scenari di Test

Il sistema supporta 6 scenari pre-configurati, ciascuno progettato per testare aspetti specifici del radar.

### 4.2.1 Scenario: Random Component Failures

Scopo: Simulare failure casuali per testare robustezza sistema

Configurazione:

{
    'name': 'Random Component Failures',
    'description': 'Random selection of component failures (30% proc, 20% tx, 15% rx)',
    'expected_failures': [
        'processor_status',
        'transmitter_status', 
        'receiver_status'
    ],
    'failure_probability': {
        'processor_status': 0.30,    # 30% chance ogni run
        'transmitter_status': 0.20,  # 20% chance
        'receiver_status': 0.15      # 15% chance
    },
    'target_gen': None
}

Behavior Simulato:
- Ogni run, mock decide casualmente quali componenti falliscono
- Probabilità configurabili per failure realistici
- Usa per stress-testing handling failure multipli

Risultati Attesi: ~50-60% run FAIL (combinazione probabilità)

### 4.2.2 Scenario: Recycle Self-Healing

Scopo: Verifica capacità auto-recupero dopo processor recycle

Configurazione:

{
    'name': 'RECYCLE Self-Healing (1 cycle)',
    'description': 'Processor fails, then auto-recovers after 1 RECYCLE event',
    'expected_failures': [
        'processor_status'  # Initial failure
    ],
    'serial_events': [
        {'type': 'recycle', 'iteration': 1}  # Recycle dopo 1 iterazione
    ],
    'auto_recovery': True,  # Failure scompare dopo recycle
    'target_gen': None
}

Sequence:
1. Run 1: Processor fails → Serial mostra “RECYCLE” event
2. Run 2+: Processor OK (auto-recovery completato)

Risultati Attesi: Run 1 FAIL, Run 2+ PASS

### 4.2.3 Scenario: Perfect Run

Scopo: Baseline - tutti i test passano

Configurazione:

{
    'name': 'Perfect Run (No Failures)',
    'description': 'All tests pass - ideal scenario',
    'expected_failures': [],
    'failure_probability': {},
    'target_gen': None
}

Risultati Attesi: 100% run PASS

### 4.2.4 Scenario: Known Failures Only

Scopo: Simula limitazioni HW setup noto (es. antenna disconnessa)

Configurazione:

{
    'name': 'Known Failures Only',
    'description': 'Only expected failures due to HW setup limitations',
    'expected_failures': [
        'antenna_drive_unit_status',
        'waveguide_pressurization_status'
    ],
    'failure_probability': {
        'antenna_drive_unit_status': 1.0,      # Sempre fail
        'waveguide_pressurization_status': 1.0  # Sempre fail
    },
    'target_gen': None
}

Comportamento:
- Run risulta PASS (known failures ignorati)
- Non trigger B8 drill-down
- Utile per test su HW parzialmente configurato

### 4.2.5 Scenario: Target Generation (TgtGen)

Scopo: Verifica capacità rilevamento target + PBIT

Configurazione:

{
    'name': 'Target Generation Test',
    'description': 'PBIT + Target detection at 2000m @ cycle 3',
    'expected_failures': [],
    'target_gen': {
        'distance': 2000,              # metri
        'appeared_after_cycles': 3,    # Visibile dopo 3 cicli B9
        'heading': 1060,               # ICD units
        'x_pos': 2500,
        'y_pos': 2500,
        'range_tolerance_low': -1000,  # Tolerance ±1000m
        'range_tolerance_high': 200
    }
}

Sequence:
1. PBIT execution (normale)
2. B6/B8 verification
3. Target test attivato:
- Set A2 mode = OPERATE
- Configura A4/A5 per target scenario
- Monitor B9 per 20 secondi
- Verifica range detection

Risultati Attesi: PBIT PASS + Target DETECTED

### 4.2.6 Scenario: Multi-Failure Cascade

Scopo: Testa worst-case con failure multipli simultanei

Configurazione:

{
    'name': 'Multi-Failure Cascade',
    'description': 'Multiple failures across different subsystems',
    'expected_failures': [],  # Tutti unexpected
    'failure_probability': {
        'processor_1_status': 0.80,
        'processor_2_status': 0.80,
        'receiver_status': 0.70,
        'transmitter_status': 0.70,
        'exciter_status': 0.60
    },
    'target_gen': None
}

Risultati Attesi: ~95% run FAIL (stress test massimo)

## 4.3 Test B6 - Verifiche LRU Status

### 4.3.1 Messaggi B6 nel Protocollo 1553

Il messaggio B6_MsgRdrSettingsAndParametersTellback è il cuore della diagnostica BIT.

Struttura B6:

Message: B6_MsgRdrSettingsAndParametersTellback
RT: 1 (Radar)
Direction: RT → BC (Telemetry)
Frequency: ~10 Hz (100ms period)
Size: ~32 words (64 bytes)

Campi Rilevanti:
| Campo | Tipo | Descrizione |
|——-|——|————-|
| b6_t1_bit_available | bool | BIT completato (true=done) |
| b6_t1_rdr_fail_status | bool | Overall radar failure |
| b6_t1_processor_1_status | bool | Processor 1 LRU status |
| b6_t1_processor_2_status | bool | Processor 2 LRU status |
| b6_t1_receiver_status | bool | Receiver LRU status |
| b6_t1_exciter_status | bool | Exciter LRU status |
| b6_t1_transmitter_status | bool | Transmitter LRU status |
| b6_t1_antenna_drive_unit_status | bool | ADU status |
| b6_t1_power_supply_status | bool | Power supply status |
| b6_t1_auxilliary_processor_status | bool | Aux processor status |
| b6_t1_waveguide_pressurization_status | bool | Pressurization status |
| b6_t1_auxilliary_siu_status | bool | Aux SIU status |
| b6_t1_receiver_transmitter_status | bool | Rx/Tx combined status |

Semantica Valori:
- "false" = LRU OK (no failure)
- "true" = LRU FAIL (failure detected)

### 4.3.2 Logica Verifica B6

Implementazione:

def verify_b6_status(known_failures: List[str]) -> Dict:
    """
    Verifica tutti i 12 campi B6 LRU status.
    Classifica failure come PASS/FAIL/KNOWN.
    """
    B6_FIELDS = [
        "b6_t1_rdr_fail_status",
        "b6_t1_processor_1_status",
        "b6_t1_processor_2_status",
        "b6_t1_receiver_status",
        "b6_t1_exciter_status",
        "b6_t1_transmitter_status",
        "b6_t1_antenna_drive_unit_status",
        "b6_t1_power_supply_status",
        "b6_t1_auxilliary_processor_status",
        "b6_t1_waveguide_pressurization_status",
        "b6_t1_auxilliary_siu_status",
        "b6_t1_receiver_transmitter_status"
    ]
    
    b6_pass = 0
    b6_fail = 0
    b6_known_fail = 0
    failures = []
    
    for field in B6_FIELDS:
        # Check: expected value = "false" (no failure)
        ret, err = check(theGrifo1553, "false", 
                        "B6_MsgRdrSettingsAndParametersTellback", field)
        
        if ret:  # Field == "false" → OK
            b6_pass += 1
        else:    # Field == "true" → FAIL
            # Leggi valore attuale per logging
            actual_value = getValue(theGrifo1553,
                                   "B6_MsgRdrSettingsAndParametersTellback", field)
            
            # Classifica: Known o Real failure?
            if field in known_failures:
                b6_known_fail += 1
                logging.info(f"B6 Known failure: {field} = {actual_value}")
            else:
                b6_fail += 1
                failures.append((field, actual_value))
                logging.error(f"B6 REAL FAILURE: {field} = {actual_value}")
    
    return {
        'b6_pass': b6_pass,
        'b6_fail': b6_fail,
        'b6_known_fail': b6_known_fail,
        'failures': failures
    }

Decision Logic:

FOR each B6 field:
    IF field_value == "false":
        → PASS (incrementa b6_pass)
    ELSE IF field_value == "true":
        IF field in known_failures_list:
            → KNOWN (incrementa b6_known_fail)
        ELSE:
            → FAIL (incrementa b6_fail, append to failures)

Criterio Run Success:

run_success = (b6_fail == 0)  # PASS solo se NO real failures
# Known failures NON influenzano run_success

### 4.3.3 Esempio Output B6

Scenario: 2 real failures, 1 known

B6 Verification Results:
  Pass:         9 fields
  Fail:         2 fields (REAL)
    - b6_t1_processor_1_status = true
    - b6_t1_transmitter_status = true
  Known Fail:   1 field
    - b6_t1_antenna_drive_unit_status = true (EXPECTED)

Run Result: FAIL (due to 2 real failures)

## 4.4 Test B8 - Drill-Down Diagnostico

### 4.4.1 Quando Eseguire B8 Drill-Down

Trigger Condition:

if b6_fail > 0:  # Almeno un real failure in B6
    drill_down_b8()
else:
    # Skip B8 (solo known failures o tutto OK)
    logging.info("B8 drill-down skipped (no real failures)")

Rationale: B8 contiene 185 campi diagnostici dettagliati. Eseguire sempre sarebbe dispendioso (5-10s). Si esegue solo quando necessario per analisi failure.

### 4.4.2 Struttura Messaggio B8

B8_MsgBitReport:

Message: B8_MsgBitReport
RT: 1 (Radar)
Direction: RT → BC
Frequency: On-demand (solo se BIT eseguito)
Size: ~185 fields (diagnostic details)

Categorie Campi B8:
| Categoria | # Campi | Esempio |
|———–|———|———|
| Processor Tests | 45 | b8_processor_1_rom_test, b8_processor_2_ram_test |
| Receiver Tests | 35 | b8_receiver_sensitivity_test, b8_receiver_noise_figure |
| Transmitter Tests | 30 | b8_transmit_power_test, b8_modulator_test |
| Exciter Tests | 25 | b8_exciter_frequency_test, b8_exciter_stability |
| Antenna Tests | 20 | b8_antenna_rotation_test, b8_antenna_position |
| Power Supply Tests | 15 | b8_psu_voltage_28v, b8_psu_current_limit |
| Misc Tests | 15 | b8_cooling_fan_test, b8_temperature_sensor |

Semantica:
- "false" = Test PASS
- "true" = Test FAIL

### 4.4.3 Implementazione Drill-Down B8

def drill_down_b8() -> Dict:
    """
    Drill-down dettagliato B8: verifica 185 campi diagnostici.
    Eseguito solo quando B6 mostra real failures.
    """
    B8_FIELDS = [
        "b8_processor_1_rom_test",
        "b8_processor_1_ram_test",
        "b8_processor_2_rom_test",
        # ... (185 total fields)
    ]
    
    b8_pass = 0
    b8_fail = 0
    b8_checked = len(B8_FIELDS)
    b8_failures = []
    
    logging.info(f"Starting B8 drill-down: {b8_checked} fields to check")
    start_time = time.time()
    
    for field in B8_FIELDS:
        try:
            ret, err = check(theGrifo1553, "false",
                           "B8_MsgBitReport", field)
            
            if ret:
                b8_pass += 1
            else:
                b8_fail += 1
                value = getValue(theGrifo1553, "B8_MsgBitReport", field)
                b8_failures.append((field, value))
                logging.warning(f"B8 FAIL: {field} = {value}")
        
        except Exception as e:
            logging.error(f"B8 check failed for {field}: {e}")
            # Continua con campo successivo
    
    elapsed = time.time() - start_time
    logging.info(f"B8 drill-down completed in {elapsed:.2f}s")
    logging.info(f"B8 Results: {b8_pass} PASS, {b8_fail} FAIL")
    
    return {
        'b8_pass': b8_pass,
        'b8_fail': b8_fail,
        'b8_checked': b8_checked,
        'b8_failures': b8_failures,
        'b8_time': elapsed
    }

Performance: ~5-10 secondi per 185 field checks (depends on 1553 bus latency)

### 4.4.4 Correlazione B6 → B8

Quando B6 mostra failure LRU, B8 drill-down identifica il test specifico fallito:

Esempio Correlazione:

B6: b6_t1_processor_1_status = true (FAIL)
  ↓ B8 Drill-Down
B8: b8_processor_1_ram_test = true (FAIL)
    b8_processor_1_rom_test = false (PASS)
    b8_processor_1_watchdog = false (PASS)
    ...
  → Root Cause: Processor 1 RAM test failed

## 4.5 Test Seriale - Monitoring Messaggi

### 4.5.1 Porta Seriale e Protocollo

Configurazione:

serial_config = {
    'port': 'COM4',         # Windows
    'baudrate': 115200,
    'bytesize': 8,
    'parity': 'N',
    'stopbits': 1,
    'timeout': 1.0          # Read timeout
}

Formato Messaggi:

[HH:MM:SS.mmm] <message_text>

|  |  |  | ### 4.5.2 Pattern Messaggi Riconosciuti |
| --- | --- | --- | --- |

Pattern

Significato

Contatore

Severità

%%E

Error message

serial_errors

WARNING

%%F

Fatal error

serial_fatal

ERROR

RECYCLE

Processor recycle event

serial_recycles

CRITICAL

PBIT COMPLETED

PBIT done

N/A

INFO

BIT AVAILABLE

BIT results ready

N/A

INFO

Esempi Messaggi Reali:

[15:24:35.123] %%E Memory allocation failed in module RX_PROC
[15:24:35.456] %%F Critical fault detected - initiating RECYCLE
[15:24:36.789] RECYCLE event - processor restarting
[15:24:50.012] PBIT COMPLETED in 15.2 seconds
[15:24:50.123] BIT AVAILABLE - results on B6/B8

### 4.5.3 Parser Implementazione

class SerialMessageParser:
    def __init__(self):
        self.statistics = {
            'total_messages': 0,
            'error_messages': 0,
            'fatal_messages': 0,
            'recycle_events': 0
        }
        self.message_buffer = []
    
    def parse_message(self, line: str) -> Dict[str, Any]:
        """Analizza singola riga seriale"""
        self.statistics['total_messages'] += 1
        
        # Extract timestamp
        timestamp_match = re.match(r'\[(\d{2}:\d{2}:\d{2}\.\d{3})\]', line)
        timestamp = timestamp_match.group(1) if timestamp_match else None
        
        # Remove timestamp from message
        message_text = re.sub(r'^\[\d{2}:\d{2}:\d{2}\.\d{3}\]\s*', '', line)
        
        # Detect pattern
        severity = 'INFO'
        if '%%E' in message_text:
            self.statistics['error_messages'] += 1
            severity = 'WARNING'
        elif '%%F' in message_text:
            self.statistics['fatal_messages'] += 1
            severity = 'ERROR'
        elif 'RECYCLE' in message_text:
            self.statistics['recycle_events'] += 1
            severity = 'CRITICAL'
        
        msg_info = {
            'timestamp': timestamp,
            'text': message_text,
            'severity': severity
        }
        
        self.message_buffer.append(msg_info)
        return msg_info
    
    def extract_statistics(self) -> Dict[str, int]:
        """Ritorna statistiche aggregate"""
        return self.statistics.copy()

### 4.5.4 Integrazione nel Test Flow

# Durante test run
terminal = SerialTerminal(port='COM4', baudrate=115200)
terminal.connect()
parser = SerialMessageParser()

# In background, leggi messaggi seriali
while test_running:
    try:
        line = terminal.readline(timeout=1.0)
        if line:
            msg_info = parser.parse_message(line)
            logging.info(f"Serial [{msg_info['severity']}]: {msg_info['text']}")
    except Exception as e:
        logging.error(f"Serial read error: {e}")

# A fine run
serial_stats = parser.extract_statistics()
run_stats.update({
    'serial_errors': serial_stats['error_messages'],
    'serial_fatal': serial_stats['fatal_messages'],
    'serial_recycles': serial_stats['recycle_events']
})

## 4.6 Test Target Generation (TgtGen)

### 4.6.1 Workflow Target Detection

Precondizioni:
- PBIT completato con successo
- B6/B8 verifiche OK
- Scenario config contiene target_gen section

Sequence:

1. Configure A2: Set mode = OPERATE (exit BIT, enter operational mode)
2. Configure A4: Inject target parameters
   - a4_heading: Target bearing (ICD units)
   - a4_x_pos, a4_y_pos: Platform position
3. Configure A5: INU mode word (valid data)
4. Monitor B9: Poll B9_MsgRdrTrackAndTargetsData for target reports
5. Verify Range: Check if detected range within tolerance
6. Count Hits: Require at least N successful detections
7. Timeout: Max 20 seconds for target detection

### 4.6.2 Messaggi 1553 per Target

A4_MsgNavDataAndCursor (BC → RT):

# Platform navigation data
setValue(theGrifo1553, 1060, "A4_MsgNavDataAndCursor", "a4_heading")  # degrees * 10
setValue(theGrifo1553, 2500, "A4_MsgNavDataAndCursor", "a4_x_pos")    # meters
setValue(theGrifo1553, 2500, "A4_MsgNavDataAndCursor", "a4_y_pos")    # meters

A5_MsgInuHighSpeed (BC → RT):

# INU (Inertial Navigation Unit) mode word
setValue(theGrifo1553, 0xAA54, "A5_MsgInuHighSpeed", "a5_inu_mode_word")

B9_MsgRdrTrackAndTargetsData (RT → BC):

# Target report from radar
target_range = getValue(theGrifo1553, "B9_MsgRdrTrackAndTargetsData", "b9_t1_rng")
target_azimuth = getValue(theGrifo1553, "B9_MsgRdrTrackAndTargetsData", "b9_t1_az")

### 4.6.3 Implementazione Target Test

def tgt_gen(scenario_config: Dict) -> Dict:
    """
    Execute target detection test.
    """
    tgt_cfg = scenario_config.get('target_gen', {})
    if not tgt_cfg:
        return {'target_detected': None}
    
    # Extract configuration
    expected_distance = tgt_cfg['distance']  # meters
    appeared_after_cycles = tgt_cfg['appeared_after_cycles']
    heading = tgt_cfg['heading']
    x_pos = tgt_cfg['x_pos']
    y_pos = tgt_cfg['y_pos']
    
    logging.info(f"Starting target detection test: {expected_distance}m @ cycle {appeared_after_cycles}")
    
    # Set OPERATE mode
    setValue(theGrifo1553, 2, "A2_MsgRdrOperationCommand", "a2_radarMode", commitChanges=True)
    time.sleep(0.5)
    
    # Configure navigation
    setValue(theGrifo1553, heading, "A4_MsgNavDataAndCursor", "a4_heading", commitChanges=False)
    setValue(theGrifo1553, x_pos, "A4_MsgNavDataAndCursor", "a4_x_pos", commitChanges=False)
    setValue(theGrifo1553, y_pos, "A4_MsgNavDataAndCursor", "a4_y_pos", commitChanges=True)
    
    # Configure INU
    setValue(theGrifo1553, 0xAA54, "A5_MsgInuHighSpeed", "a5_inu_mode_word", commitChanges=True)
    time.sleep(1.0)
    
    # Monitor B9 for target
    target_found = False
    found_iteration = None
    found_range = None
    
    timeout = TARGET_DETECTION_TIMEOUT_SEC
    start_time = time.time()
    iteration = 0
    hit_count = 0
    
    while (time.time() - start_time) < timeout:
        iteration += 1
        
        try:
            # Read target range from B9
            range_value = getValue(theGrifo1553, "B9_MsgRdrTrackAndTargetsData", "b9_t1_rng")
            
            # Verify range within tolerance
            range_min = expected_distance + TARGET_RANGE_TOLERANCE_LOW
            range_max = expected_distance + TARGET_RANGE_TOLERANCE_HIGH
            
            if range_min <= range_value <= range_max:
                hit_count += 1
                logging.info(f"Target HIT #{hit_count}: range={range_value}m @ iteration {iteration}")
                
                if hit_count >= TARGET_HIT_THRESHOLD:
                    target_found = True
                    found_iteration = iteration
                    found_range = range_value
                    logging.info(f"Target DETECTED: {found_range}m @ cycle {found_iteration}")
                    break
        
        except Exception as e:
            logging.warning(f"B9 read error iteration {iteration}: {e}")
        
        time.sleep(0.1)  # Poll every 100ms
    
    elapsed = time.time() - start_time
    
    if not target_found:
        logging.warning(f"Target NOT detected after {elapsed:.1f}s ({iteration} cycles)")
    
    return {
        'target_detected': target_found,
        'target_range': found_range if target_found else None,
        'target_iterations': found_iteration if target_found else iteration,
        'target_test_time': elapsed
    }

### 4.6.4 Criteri Successo Target

# Target considerato DETECTED se:
SUCCESS = (hit_count >= TARGET_HIT_THRESHOLD) AND \
          (range_min <= detected_range <= range_max) AND \
          (elapsed_time < TARGET_DETECTION_TIMEOUT_SEC)

# Parametri tipici:
TARGET_HIT_THRESHOLD = 2          # Almeno 2 detection consecutive
TARGET_RANGE_TOLERANCE_LOW = -1000  # ±1000m tolerance
TARGET_RANGE_TOLERANCE_HIGH = 200
TARGET_DETECTION_TIMEOUT_SEC = 20.0

← Script Principale | Torna all’Indice | Prossimo: Modalità Esecuzione →X

# — 05_Modalita_Esecuzione.md —

# Capitolo 5: Modalità di Esecuzione

← Workflow Test | Torna all’Indice | Prossimo: GUI →X

|  |  |  |  | ## 5.1 Panoramica Modalità |
| --- | --- | --- | --- | --- |

Il sistema supporta due modalità di esecuzione distinte, progettate per coprire scenari diversi:

Modalità

Uso Principale

Hardware Richiesto

Velocità

Determinismo

Hardware (--target)

Validation su radar reale, test integrazione

Radar + 1553 + Seriale + Power

~60-80s/run

Basso (timing reali)

Simulazione (--simulate)

Development, debugging, regression testing

Solo PC

~10-15s/run

Alto (mock deterministici)

### 5.1.1 Quando Usare Quale Modalità

Usa Hardware Mode quando:
- ✅ Validazione finale prima del rilascio
- ✅ Test di integrazione HW/SW
- ✅ Identificazione timing issues o race conditions
- ✅ Verifica comportamento sotto condizioni reali (temperatura, EMI, ecc.)
- ✅ Cattura failure intermittenti

Usa Simulation Mode quando:
- ✅ Sviluppo locale senza accesso hardware
- ✅ Debugging rapido di logica test
- ✅ Regression testing in CI/CD pipeline
- ✅ Test scenari rari (failure multipli, edge cases)
- ✅ Validazione modifiche codice senza rischio hardware

## 5.2 Modalità Hardware (--target)

### 5.2.1 Configurazione Hardware

Setup Fisico Richiesto:

┌─────────────────────────────────────────────────────────┐
│                      Test PC                             │
│  ┌──────────────────────────────────────────────────┐  │
│  │  GRIFO_M_PBIT.py                                 │  │
│  │  - Python 3.8                                    │  │
│  │  - interpreter.pyd (1553 driver)                 │  │
│  │  - pyserial (serial driver)                      │  │
│  └───┬──────────────────────────────────────┬───────┘  │
└──────┼──────────────────────────────────────┼──────────┘
       │                                      │
       │ 1553 Bus                             │ Serial RS-232
       ↓                                      ↓
┌──────────────────────────────────────────────────────────┐
│                    GRIFO Radar                            │
│  - RT Address: 1                                          │
│  - 1553 Messages: A1-A5 (BC→RT), B6-B9 (RT→BC)          │
│  - Serial Port: 115200 baud, 8N1                         │
└───────────────────────────┬──────────────────────────────┘
                            │
                            │ Power Control
                            ↓
                    ┌────────────────┐
                    │   BrainBox     │
                    │  (Power Relay) │
                    └────────────────┘

Connessioni Fisiche:
1. 1553 Bus: Scheda 1553 PC ↔ Radar (BC-RT topology)
2. Seriale: COM port PC ↔ Radar RS-232
3. Power Control (opzionale): BrainBox COM port ↔ PC

### 5.2.2 Esecuzione Hardware

Launcher Script: run_target.bat

@echo off
REM ============================================================
REM run_target.bat - Execute test on real GRIFO hardware
REM ============================================================

echo ╔════════════════════════════════════════════════════════╗
echo ║   GRIFO M-PBIT Test - HARDWARE MODE (--target)         ║
echo ╚════════════════════════════════════════════════════════╝
echo.

REM Check hardware connections
echo [INFO] Checking hardware connections...
echo   - 1553 Bus: Check BC-RT connection
echo   - Serial Port: COM4 (verify in Device Manager)
echo   - Power Control: BrainBox on COM3
echo.

pause

REM Set Python path (use embedded Python from PlatformSimulator)
set PYTHON_HOME=%~dp0PlatformSimulator\bin
set PATH=%PYTHON_HOME%;%PATH%

REM Navigate to scripts directory
cd TestEnvironment\scripts

REM Execute test with target mode
echo [INFO] Starting test execution...
python GRIFO_M_PBIT.py --target --runs 10

REM Check exit code
if %ERRORLEVEL% NEQ 0 (
    echo [ERROR] Test execution failed with error code %ERRORLEVEL%
    pause
    exit /b %ERRORLEVEL%
)

echo.
echo [SUCCESS] Test completed successfully
echo [INFO] Reports saved to: TestEnvironment\pdf_reports\
echo.

pause

Esecuzione Diretta:

# Dalla root del repository
cd TestEnvironment/scripts
python GRIFO_M_PBIT.py --target --runs 10 --scenario random

Parametri CLI Disponibili:

python GRIFO_M_PBIT.py --target \
    --runs 10 \                    # Numero ripetizioni (default: 10)
    --scenario random \            # Scenario test (default: random)
    --no-gui \                     # Disabilita GUI (solo log)
    --com-port COM4 \              # Override COM port seriale
    --brainbox-port COM3           # Override BrainBox port

### 5.2.3 Workflow Hardware Mode

1. INITIALIZATION
   ├─ Load 1553 driver (_interpreter.pyd)
   ├─ Connect serial port (COM4, 115200 baud)
   ├─ Initialize BrainBox (COM3)
   └─ Start GUI monitor (if enabled)

2. PRE-TEST CHECKS
   ├─ Verify 1553 communication (ping radar)
   ├─ Check serial port responsive
   └─ Test BrainBox power control

3. TEST EXECUTION
   FOR each run:
       ├─ Power OFF radar (via BrainBox)
       ├─ Wait discharge (4s)
       ├─ Power ON radar (via BrainBox)
       ├─ Wait initialization (1.5s)
       ├─ Configure 1553 messages (A1/A2/A4/A5)
       ├─ Execute PBIT (send command via A2)
       ├─ Monitor completion (poll B6 bit_available)
       ├─ Verify B6 status (12 fields)
       ├─ Drill-down B8 (if failures)
       ├─ Parse serial buffer (%%E, %%F, RECYCLE)
       ├─ Target test (if scenario enabled)
       ├─ Aggregate statistics
       └─ Update GUI

4. POST-TEST
   ├─ Generate PDF report
   ├─ Export CSV statistics
   ├─ Export JSON data
   ├─ Power OFF radar (safe shutdown)
   └─ Disconnect serial/1553

### 5.2.4 Troubleshooting Hardware Mode

Problema: “1553 Communication Lost”

Cause: Radar non risponde su bus 1553
Solutions:
  1. Verificare cablaggio 1553 (terminazioni corrette)
  2. Controllare alimentazione radar
  3. Verificare RT address configurato (deve essere 1)
  4. Controllare driver 1553 installato correttamente

Problema: “Serial Port Open Failed”

Cause: Porta seriale non accessibile
Solutions:
  1. Verificare COM port in Device Manager
  2. Chiudere altri programmi che usano la porta (PuTTY, HyperTerminal)
  3. Verificare permessi utente (potrebbe richiedere admin)
  4. Controllare cavo USB-to-Serial funzionante

Problema: “BrainBox Not Responding”

Cause: Power control non funzionante
Solutions:
  1. Verificare BrainBox connesso e acceso
  2. Controllare COM port BrainBox (default: COM3)
  3. Test manuale: python -c "import brainbox_interface; bb = brainbox_interface.BrainBoxInterface('COM3'); print(bb.power_on(1))"
  4. Workaround: Disabilitare power cycling automatico, controllare manualmente

## 5.3 Modalità Simulazione (--simulate)

### 5.3.1 Principi Simulazione

La modalità simulazione non richiede hardware e usa mock objects per emulare risposte radar.

Vantaggi:
- ⚡ Velocità: 6-8x più veloce del hardware (no power cycling, no wait reali)
- 🎯 Determinismo: Risultati ripetibili e prevedibili
- 🔧 Debug Friendly: Logging dettagliato, breakpoint funzionanti
- 🚀 CI/CD Ready: Eseguibile in pipeline automatiche

Limitazioni:
- ❌ Non cattura timing issues reali
- ❌ Non testa integrazione HW/SW
- ❌ Mock behavior potrebbe divergere da hardware reale

### 5.3.2 Esecuzione Simulazione

Launcher Script: run_simulate.bat

@echo off
REM ============================================================
REM run_simulate.bat - Execute test in SIMULATION mode
REM Uses embedded Python from PlatformSimulator/bin
REM ============================================================

echo ╔════════════════════════════════════════════════════════╗
echo ║   GRIFO M-PBIT Test - SIMULATION MODE                 ║
echo ╚════════════════════════════════════════════════════════╝
echo.

REM Use embedded Python with all dependencies
set PYTHON_HOME=%~dp0PlatformSimulator\bin
set PATH=%PYTHON_HOME%;%PYTHON_HOME%\DLLs;%PATH%

echo [INFO] Using embedded Python: %PYTHON_HOME%\python.exe
echo [INFO] Starting simulation...
echo.

REM Navigate to scripts and execute
cd TestEnvironment\scripts
python GRIFO_M_PBIT.py --simulate --runs 10 --scenario random

echo.
echo [INFO] Simulation completed
echo [INFO] Reports: TestEnvironment\pdf_reports\
echo.

pause

Launcher Script Semplificato: run_simulate_simple.bat

@echo off
REM Uses system Python (must have dependencies installed)

echo [INFO] Using system Python
cd TestEnvironment\scripts
python GRIFO_M_PBIT.py --simulate --runs 10
pause

Esecuzione Diretta:

# Dalla root del repository
python TestEnvironment/scripts/GRIFO_M_PBIT.py --simulate --runs 10 --scenario tgtgen

### 5.3.3 Sistema Mock Objects

Architettura Mock:

# File: GRIFO_M_PBIT_mock.py

class MockGrifo1553Interface:
    """
    Mock per theGrifo1553 singleton.
    Emula risposte 1553 basate su scenario configurato.
    """
    
    def __init__(self, scenario_config: Dict):
        self.scenario = scenario_config
        self.message_data = {}  # Simulated message values
        self._init_message_defaults()
        self._apply_scenario_failures()
    
    def check(self, RT: int, expected: Any, msg: str, field: str) -> Tuple[bool, str]:
        """Simula check 1553 field"""
        actual = self.get(RT, msg, field)
        return (actual == expected, "")
    
    def get(self, RT: int, msg: str, field: str) -> Any:
        """Ritorna valore simulato per campo"""
        key = f"{msg}.{field}"
        return self.message_data.get(key, "false")  # Default: no failure
    
    def set(self, RT: int, value: Any, msg: str, field: str, commitChanges: bool = False) -> Tuple[bool, str]:
        """Simula set 1553 field (memorizza valore)"""
        key = f"{msg}.{field}"
        self.message_data[key] = value
        return (True, "")
    
    def _init_message_defaults(self):
        """Inizializza valori default messaggi"""
        # B6 defaults (all LRU OK)
        for field in B6_FIELDS:
            self.message_data[f"B6_MsgRdrSettingsAndParametersTellback.{field}"] = "false"
        
        # B6 bit_available = false initially (PBIT not done)
        self.message_data["B6_MsgRdrSettingsAndParametersTellback.b6_t1_bit_available"] = "false"
        
        # B8 defaults (all tests PASS)
        for field in B8_FIELDS:
            self.message_data[f"B8_MsgBitReport.{field}"] = "false"
        
        # B9 defaults (no target)
        self.message_data["B9_MsgRdrTrackAndTargetsData.b9_t1_rng"] = 0
    
    def _apply_scenario_failures(self):
        """Applica failure configurati in scenario"""
        failures = self.scenario.get('expected_failures', [])
        failure_probs = self.scenario.get('failure_probability', {})
        
        for field_short_name, probability in failure_probs.items():
            # Decide casualmente se applicare failure
            if random.random() < probability:
                # Map short name → full field name
                full_field = f"b6_t1_{field_short_name}"
                self.message_data[f"B6_MsgRdrSettingsAndParametersTellback.{full_field}"] = "true"

Mock BrainBox:

class MockBrainBox:
    """Mock per theBrainBox singleton"""
    
    def __init__(self):
        self.power_state = False
    
    def power_on(self, channel: int = 1) -> bool:
        logging.info(f"[MOCK] BrainBox: Power ON channel {channel}")
        self.power_state = True
        return True
    
    def power_off(self, channel: int = 1) -> bool:
        logging.info(f"[MOCK] BrainBox: Power OFF channel {channel}")
        self.power_state = False
        return True
    
    def get_status(self, channel: int = 1) -> str:
        return "ON" if self.power_state else "OFF"

Mock Serial Terminal:

class MockSerialTerminal:
    """Mock per serial terminal"""
    
    def __init__(self, scenario_config: Dict):
        self.scenario = scenario_config
        self.buffer = []
        self._generate_mock_messages()
    
    def _generate_mock_messages(self):
        """Genera messaggi seriali simulati basati su scenario"""
        # Messaggi base
        self.buffer.append("[15:24:30.000] GRIFO Radar v3.2 - Startup")
        self.buffer.append("[15:24:31.000] Initialization complete")
        
        # Aggiungi %%E se scenario prevede errori
        if self.scenario.get('serial_error_rate', 0) > 0:
            self.buffer.append("[15:24:35.123] %%E Memory allocation warning")
        
        # Aggiungi RECYCLE se previsto
        if 'recycle' in self.scenario.get('serial_events', []):
            self.buffer.append("[15:24:36.456] %%F Critical fault detected")
            self.buffer.append("[15:24:36.789] RECYCLE event - processor restarting")
        
        # Messaggi PBIT
        self.buffer.append("[15:24:50.000] PBIT COMPLETED in 15.2 seconds")
        self.buffer.append("[15:24:50.100] BIT AVAILABLE")
    
    def readline(self, timeout: float = 1.0) -> str:
        """Ritorna prossimo messaggio dal buffer"""
        if self.buffer:
            return self.buffer.pop(0)
        return ""
    
    def connect(self):
        logging.info("[MOCK] Serial: Connected (simulated)")
    
    def disconnect(self):
        logging.info("[MOCK] Serial: Disconnected (simulated)")

### 5.3.4 Monkey Patching

Come Mock Viene Iniettato:

# In GRIFO_M_PBIT.py

if args.simulate:
    # Import mock implementations
    from GRIFO_M_PBIT_mock import (
        MockGrifo1553Interface,
        MockBrainBox,
        MockSerialTerminal
    )
    
    # Load scenario configuration
    scenario_config = SCENARIOS[args.scenario]
    
    # Replace global singletons with mock objects
    import leo_grifo_1553
    import leo_grifo_io_box
    
    leo_grifo_1553.theGrifo1553 = MockGrifo1553Interface(scenario_config)
    leo_grifo_io_box.theBrainBox = MockBrainBox()
    
    # Use mock terminal instead of real serial
    terminal = MockSerialTerminal(scenario_config)
    
    logging.info("SIMULATION MODE: Mock objects injected")
else:
    # Use real hardware interfaces
    from leo_grifo_1553 import theGrifo1553
    from leo_grifo_io_box import theBrainBox
    terminal = SerialTerminal(port='COM4', baudrate=115200)
    
    logging.info("HARDWARE MODE: Real interfaces loaded")

Vantaggi Monkey Patching:
- ✅ Test code identico per hardware e simulazione
- ✅ Nessun if simulate: scattered nel codice
- ✅ Mock trasparente (stessa API)

### 5.3.5 Timing Simulation

Mock objects simulano anche timing realistici:

class MockGrifo1553Interface:
    def wait_pbit_completion(self, timeout: float) -> Tuple[bool, float, str]:
        """Simula PBIT completion con timing realistico"""
        # Simula PBIT duration (15-20 secondi)
        pbit_duration = random.uniform(15.0, 20.0)
        
        # Simula attesa (velocità 10x per simulazione)
        time.sleep(pbit_duration / 10.0)  # 1.5-2 secondi reali
        
        # Imposta bit_available = true
        self.message_data["B6_MsgRdrSettingsAndParametersTellback.b6_t1_bit_available"] = "true"
        
        return (True, pbit_duration, 'completed')

Speedup Factor: Simulazione è ~6-8x più veloce grazie a:
- ⏩ No power cycling fisico (5s → 0.5s)
- ⏩ PBIT simulato ridotto (18s → 2s)
- ⏩ No wait hardware stabilization

## 5.4 Parametri di Configurazione

### 5.4.1 Parametri Command Line

Sintassi Completa:

python GRIFO_M_PBIT.py \
    [--target | --simulate] \              # OBBLIGATORIO: modalità
    [--runs N] \                           # Numero ripetizioni (default: 10)
    [--scenario NAME] \                    # Scenario test (default: random)
    [--no-gui] \                           # Disabilita GUI
    [--com-port PORT] \                    # Serial port (default: COM4)
    [--brainbox-port PORT] \               # BrainBox port (default: COM3)
    [--export-csv] \                       # Forza export CSV (default: auto)
    [--no-export-csv]                      # Disabilita export CSV

Esempi:

# Simulazione rapida 5 run scenario perfect
python GRIFO_M_PBIT.py --simulate --runs 5 --scenario perfect

# Hardware 20 run scenario random senza GUI
python GRIFO_M_PBIT.py --target --runs 20 --scenario random --no-gui

# Simulazione target detection
python GRIFO_M_PBIT.py --simulate --runs 10 --scenario tgtgen

# Hardware con COM port custom
python GRIFO_M_PBIT.py --target --com-port COM5 --brainbox-port COM6

### 5.4.2 Costanti Configurabili

File: GRIFO_M_PBIT.py (sezione CONFIGURATION)

# ===== TEST EXECUTION =====
NUMBER_OF_REPETITIONS = 10           # Default runs
PBIT_SEC_TIME = 182                  # PBIT timeout (secondi)
COMM_LOSS_THRESHOLD = 20             # Iterations senza messaggi = comm loss

# ===== TARGET DETECTION =====
TARGET_DETECTION_TIMEOUT_SEC = 20.0  # Timeout target test
TARGET_EXPECTED_RANGE = 2536         # Range atteso (ICD units)
TARGET_RANGE_TOLERANCE_LOW = -1000   # Tolerance bassa
TARGET_RANGE_TOLERANCE_HIGH = 200    # Tolerance alta
TARGET_HIT_THRESHOLD = 2             # Numero detection richieste

# ===== SERIAL CONFIGURATION =====
SERIAL_PORT = 'COM4'                 # Default serial port
SERIAL_BAUDRATE = 115200             # Baud rate
SERIAL_TIMEOUT = 1.0                 # Read timeout

# ===== POWER CONTROL =====
BRAINBOX_PORT = 'COM3'               # BrainBox COM port
POWER_OFF_WAIT_BEFORE = 1.0          # Attesa pre-spegnimento
POWER_OFF_WAIT_AFTER = 4.0           # Attesa post-spegnimento (discharge)
POWER_ON_WAIT_AFTER = 1.5            # Attesa post-accensione (init)

# ===== KNOWN FAILURES =====
KNOWN_FAILURES = [
    "b6_t1_antenna_drive_unit_status",
    "b6_t1_waveguide_pressurization_status"
]

# ===== OUTPUT =====
EXPORT_STATISTICS_CSV = True         # Export CSV automatico

### 5.4.3 Scenari Custom

Definire Scenario Personalizzato:

# In GRIFO_M_PBIT.py, aggiungi a SCENARIOS dict

SCENARIOS['my_custom_scenario'] = {
    'name': 'My Custom Test',
    'description': 'Custom failure pattern for specific test case',
    'expected_failures': [
        'processor_1_status'
    ],
    'failure_probability': {
        'processor_1_status': 0.50,  # 50% chance
        'receiver_status': 0.30       # 30% chance
    },
    'serial_events': [
        {'type': 'error', 'count': 3},    # 3 %%E messages
        {'type': 'recycle', 'iteration': 2}  # RECYCLE at run 2
    ],
    'target_gen': {
        'distance': 3000,                 # 3km target
        'appeared_after_cycles': 5,
        'heading': 900,
        'x_pos': 1000,
        'y_pos': 1000
    }
}

Esecuzione:

python GRIFO_M_PBIT.py --simulate --scenario my_custom_scenario

## 5.5 Comparazione Modalità

|  |  | ### 5.5.1 Tabella Comparativa |
| --- | --- | --- |

Caratteristica

Hardware Mode

Simulation Mode

Hardware richiesto

Radar + PC + 1553 + Serial

Solo PC

Tempo/run

60-80 secondi

10-15 secondi

Setup time

~5 minuti

~30 secondi

Determinismo

Basso (timing variabili)

Alto (ripetibile)

Debug

Difficile (timing critici)

Facile (breakpoint OK)

Failure detection

Reali + intermittenti

Solo configurati

CI/CD

No (hardware-dependent)

Sì (fully automated)

Cost

Alto (tempo + hardware)

Basso (solo CPU)

Use case

Final validation

Development, regression

### 5.5.2 Workflow Raccomandato

Development Cycle:
1. [SIMULATE] Develop new test logic locally
2. [SIMULATE] Debug and iterate quickly
3. [SIMULATE] Run regression suite (all scenarios)
4. [HARDWARE] Execute validation run (1-2 scenarios)
5. [HARDWARE] Long-duration stress test (100+ runs)
6. [SIMULATE] Re-run regression after fixes

CI/CD Pipeline:
1. Commit → [SIMULATE] Quick smoke test (5 runs)
2. PR → [SIMULATE] Full regression (all scenarios, 10 runs each)
3. Merge → [HARDWARE] Nightly validation (selected scenarios)
4. Release → [HARDWARE] Extensive validation (all scenarios, 50+ runs)

← Workflow Test | Torna all’Indice | Prossimo: GUI →X

# — 06_Interfaccia_Grafica_GUI.md —

# Capitolo 6: Interfaccia Grafica (GUI Real-Time)

← Modalità Esecuzione | Torna all’Indice | Prossimo: Output e Report →X

## 6.1 Overview GUI

L’interfaccia grafica fornisce monitoraggio real-time dell’esecuzione test, mostrando progresso, statistiche e risultati mentre il test è in corso.

|  |  | ### 6.1.1 Caratteristiche Principali |
| --- | --- | --- |

Feature

Descrizione

Beneficio

Real-Time Updates

Aggiornamenti live durante esecuzione

Visibilità immediata stato test

Thread-Safe

Queue-based communication

Nessun deadlock o race condition

Per-Run Table

Tabella dettagliata ogni run

Tracking granulare risultati

Color Coding

Verde=PASS, Rosso=FAIL

Identificazione rapida problemi

Tooltip Informativi

Hover su intestazioni per spiegazioni

UX migliorata per nuovi utenti

Event Log

Log scrollabile eventi test

Debug e troubleshooting

Non-Blocking

GUI su thread separato

Test continua anche se GUI chiusa

### 6.1.2 Screenshot Concettuale Layout

┌─────────────────────────────────────────────────────────────────┐
│  GRIFO M-PBIT Test Monitor                       ● RUNNING      │
├─────────────────────────────────────────────────────────────────┤
│                                                                  │
│  Test Progress                                                   │
│  ┌──────────────────────────────────────────────────────────┐  │
│  │  Current Run:  8 / 10                                    │  │
│  │  ▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓░░░░░░░░░░░░░  80%               │  │
│  │  PBIT Time:    17.3s                                     │  │
│  │  Estimated Remaining: 2m 15s (20%)                       │  │
│  └──────────────────────────────────────────────────────────┘  │
│                                                                  │
│  Current Scenario                                                │
│  ┌──────────────────────────────────────────────────────────┐  │
│  │  Scenario: Random Component Failures                     │  │
│  │  Description: Random selection of failures (30% proc...) │  │
│  │  Expected Failures:                                      │  │
│  │    ✗ processor_status                                    │  │
│  │    ✗ transmitter_status                                  │  │
│  └──────────────────────────────────────────────────────────┘  │
│                                                                  │
│  Runs Overview                                                   │
│  ┌──────────────────────────────────────────────────────────┐  │
│  │ Run  Status  PBIT   B6 Status  B8 Checks  Serial   Targ..│  │
│  │            Time    (P/F/K)    (Pass/Fail) Msgs     et    │  │
│  │                                           (E/F/R)         │  │
│  ├──────────────────────────────────────────────────────────┤  │
│  │ 1/10 PASS   2.2s   11/0/1     173/0      0/0/0    2000m  │  │
│  │ 2/10 PASS   0.7s   11/0/1     173/0      0/0/0    -      │  │
│  │ 3/10 FAIL   0.1s   9/2/1      -          0/3/3    -      │  │
│  │ 4/10 FAIL   0.1s   9/2/1      173/0      0/3/0    -      │  │
│  │ 5/10 PASS   2.0s   9/2/1      173/0      0/0/0    -      │  │
│  │ 6/10 PASS   0.3s   11/0/1     173/0      0/2/2    -      │  │
│  │ 7/10 PASS   0.0s   11/0/1     173/0      0/2/2    2000m  │  │
│  │ 8/10 PASS   ....   ......     ......     ......   ....   │  │◄ Running
│  │ 9/10                                                      │  │
│  │10/10                                                      │  │
│  └──────────────────────────────────────────────────────────┘  │
│    ▲ Verde = PASS  │  Rosso = FAIL  │  Double-click = Details │
│                                                                  │
│  Event Log                                                       │
│  ┌──────────────────────────────────────────────────────────┐  │
│  │ [15:24:52] Starting run 8                                │  │
│  │ [15:24:54] Power ON - setup starting immediately        │  │
│  │ [15:24:54] SILENCE, STANDBAY, and scan params set       │  │
│  │ [15:24:54] Tellback verification skipped (fast mode)    │  │
│  │ [15:24:54] Starting PBIT test...                         │  │
│  │ [15:24:54] BIT completed in 2.9s                         │  │
│  │ [15:24:54] BIT available after 0.0s                      │  │
│  │ [15:24:54] B6: ALL checks passed                         │  │
│  │ [15:24:54] STANDBY removed                               │  │
│  │ [15:24:54] Run 8: Target detection test starting...     │  │
│  │ [15:31:25] Target HIT #1: range=2000m @ iteration 3     │  │
│  └──────────────────────────────────────────────────────────┘  │
│                                                                  │
└─────────────────────────────────────────────────────────────────┘

## 6.2 Architettura GUI

### 6.2.1 Pattern Threading

Problema: Tkinter non è thread-safe. Widgets devono essere aggiornati dal main thread (GUI thread).

Soluzione: Queue-based Producer-Consumer Pattern

┌──────────────────────────────────────────────────────────┐
│                   Main Thread                             │
│                  (GUI Thread)                             │
│  ┌────────────────────────────────────────────────────┐  │
│  │  Tkinter Event Loop                                │  │
│  │  - root.mainloop()                                 │  │
│  │  - Handles UI events (clicks, resize, etc.)       │  │
│  │  - Calls _poll_updates() every 100ms               │  │
│  └──────────────┬─────────────────────────────────────┘  │
│                 │                                         │
│                 │ Reads from                              │
│                 ↓                                         │
│  ┌────────────────────────────────────────────────────┐  │
│  │  update_queue (queue.Queue)                        │  │
│  │  Thread-safe FIFO queue                            │  │
│  └──────────────▲─────────────────────────────────────┘  │
│                 │                                         │
└─────────────────┼─────────────────────────────────────────┘
                  │
                  │ Writes to
                  │
┌─────────────────┼─────────────────────────────────────────┐
│                 │          Worker Thread                   │
│                 │         (Test Thread)                    │
│  ┌──────────────┴─────────────────────────────────────┐  │
│  │  Test Execution                                    │  │
│  │  - main_test()                                     │  │
│  │  - Power cycling                                   │  │
│  │  - PBIT execution                                  │  │
│  │  - B6/B8 checks                                    │  │
│  │  - Calls gui_monitor.update_*() methods           │  │
│  └────────────────────────────────────────────────────┘  │
│                                                            │
└────────────────────────────────────────────────────────────┘

### 6.2.2 Classe TestMonitorGUI

File: GRIFO_M_PBIT_gui.py (~1100 righe)

Struttura Classe:

class TestMonitorGUI:
    """
    Real-time GUI monitor for GRIFO PBIT test execution.
    Thread-safe via queue-based updates.
    """
    
    def __init__(self):
        """Initialize (does not show window yet)"""
        self.root = None              # Tkinter root window
        self.thread = None            # GUI thread
        self.running = False          # Control flag
        self.update_queue = queue.Queue()  # Thread-safe queue
        
        # Data storage (read by GUI thread)
        self.data = {
            'run_current': 0,
            'run_total': 0,
            'scenario_name': 'Initializing...',
            'pbit_time': 0.0,
            'b6_pass': 0,
            'b6_fail': 0,
            # ... altri campi
        }
        
        # Widget references
        self.widgets = {}
        
        # Per-run data for detail dialog
        self.runs_data = {}  # {run_id: {metadata}}
    
    # ===== PUBLIC API (chiamate da test thread) =====
    
    def start(self):
        """Start GUI on separate thread"""
        self.thread = threading.Thread(target=self._run_gui, daemon=True)
        self.thread.start()
        time.sleep(0.5)  # Wait for window creation
    
    def stop(self):
        """Stop GUI gracefully"""
        self.running = False
        if self.thread:
            self.thread.join(timeout=2.0)
    
    def update_status(self, run_current=None, run_total=None, ...):
        """Update test progress (thread-safe)"""
        data = {}
        if run_current is not None:
            data['run_current'] = run_current
        # ... altri campi
        if data:
            self.update_queue.put(('status', data))
    
    def update_run(self, run: int, **kwargs):
        """Update single run row (thread-safe)"""
        data = {'run': run}
        data.update(kwargs)
        self.update_queue.put(('run_row', data))
    
    def log_event(self, severity: str, message: str):
        """Add log entry (thread-safe)"""
        self.update_queue.put(('log', {
            'severity': severity,
            'message': message
        }))
    
    # ===== PRIVATE GUI THREAD METHODS =====
    
    def _run_gui(self):
        """GUI thread main function"""
        try:
            self.root = tk.Tk()
            self.root.title("GRIFO M-PBIT Test Monitor")
            self.running = True
            
            self._build_ui()       # Create all widgets
            self._poll_updates()   # Start queue polling
            
            self.root.protocol("WM_DELETE_WINDOW", self._on_closing)
            self.root.mainloop()   # Blocking call
        
        except Exception as e:
            logging.error(f"GUI thread error: {e}")
        finally:
            self.running = False
    
    def _poll_updates(self):
        """Poll update queue and apply changes (called every 100ms)"""
        if not self.running:
            return
        
        try:
            # Process all queued updates
            while not self.update_queue.empty():
                update_type, data = self.update_queue.get_nowait()
                
                if update_type == 'status':
                    self._update_status(data)
                elif update_type == 'run_row':
                    self._update_run_row(data)
                elif update_type == 'log':
                    self._add_log_entry(data)
                # ... altri tipi
        
        except queue.Empty:
            pass
        except Exception as e:
            logging.error(f"GUI update error: {e}")
        
        # Re-schedule next poll
        self.root.after(100, self._poll_updates)  # 100ms = 10 Hz
    
    def _build_ui(self):
        """Create all widgets"""
        # ... (vedi sezione 6.3)
    
    def _update_status(self, data: Dict):
        """Apply status update to widgets"""
        # ... (aggiorna progress bar, label, ecc.)
    
    def _update_run_row(self, data: Dict):
        """Update table row for specific run"""
        # ... (vedi sezione 6.4)

### 6.2.3 Lifecycle GUI

1. INITIALIZATION (main thread)
   ├─ gui_monitor = TestMonitorGUI()
   ├─ gui_monitor.start()  → Spawns GUI thread
   └─ Wait 0.5s for window creation

2. GUI THREAD START
   ├─ Create Tkinter root window
   ├─ Build all widgets (_build_ui)
   ├─ Start queue polling (_poll_updates every 100ms)
   └─ Enter mainloop() [BLOCKING]

3. TEST EXECUTION (worker thread)
   ├─ Run test loop
   ├─ Call gui_monitor.update_status(...)
   ├─ Call gui_monitor.update_run(...)
   └─ Call gui_monitor.log_event(...)
        ↓
   Updates enqueued to update_queue

4. GUI POLLING (GUI thread, every 100ms)
   ├─ Dequeue all pending updates
   ├─ Apply updates to widgets
   └─ Schedule next poll (root.after(100, ...))

5. COMPLETION (worker thread)
   ├─ Test finishes
   ├─ Call gui_monitor.show_results(output_folder)
   └─ Call gui_monitor.stop() [optional]

6. CLEANUP
   ├─ User closes window OR stop() called
   ├─ Exit mainloop()
   ├─ running = False
   └─ Thread terminates

## 6.3 Costruzione UI (_build_ui)

### 6.3.1 Layout Grid

La GUI usa Tkinter Grid Layout con 5 righe principali:

def _build_ui(self):
    """Create all UI widgets"""
    main_frame = ttk.Frame(self.root, padding="10")
    main_frame.grid(row=0, column=0, sticky=(tk.W, tk.E, tk.N, tk.S))
    
    # Configure grid weights for resizing
    self.root.columnconfigure(0, weight=1)
    self.root.rowconfigure(0, weight=1)
    main_frame.columnconfigure(0, weight=1)
    main_frame.columnconfigure(1, weight=1)
    
    # ROW 0: Test Progress
    self._create_progress_section(main_frame, row=0)
    
    # ROW 1: Current Scenario
    self._create_scenario_section(main_frame, row=1)
    
    # ROW 2: Runs Overview Table
    self._create_runs_table(main_frame, row=2)
    
    # ROW 3: Event Log
    self._create_event_log(main_frame, row=3)
    
    # Set window size and position
    self.root.geometry("900x700")
    self._center_window()

### 6.3.2 Progress Section (Row 0)

def _create_progress_section(self, parent, row):
    """Create test progress widgets"""
    progress_frame = ttk.LabelFrame(parent, text="Test Progress", padding="10")
    progress_frame.grid(row=row, column=0, columnspan=2, 
                       sticky=(tk.W, tk.E), pady=(0, 10))
    progress_frame.columnconfigure(1, weight=1)
    
    # Current run label
    ttk.Label(progress_frame, text="Current Run:").grid(
        row=0, column=0, sticky=tk.W)
    self.widgets['run_label'] = ttk.Label(progress_frame, text="0 / 0", 
                                          font=('Arial', 10, 'bold'))
    self.widgets['run_label'].grid(row=0, column=1, sticky=tk.W, padx=(10, 0))
    
    # Progress bar
    self.widgets['progress_bar'] = ttk.Progressbar(
        progress_frame, mode='determinate', length=400)
    self.widgets['progress_bar'].grid(row=1, column=0, columnspan=2, 
                                     sticky=(tk.W, tk.E), pady=(5, 0))
    
    # PBIT time
    ttk.Label(progress_frame, text="PBIT Time:").grid(
        row=2, column=0, sticky=tk.W, pady=(5, 0))
    self.widgets['pbit_label'] = ttk.Label(progress_frame, text="0.0s", 
                                           font=('Arial', 10))
    self.widgets['pbit_label'].grid(row=2, column=1, sticky=tk.W, 
                                    padx=(10, 0), pady=(5, 0))
    
    # Estimated remaining time (ETA)
    ttk.Label(progress_frame, text="Estimated Remaining:").grid(
        row=2, column=2, sticky=tk.W, pady=(5, 0))
    self.widgets['eta_label'] = ttk.Label(progress_frame, text="--:--", 
                                          font=('Arial', 10))
    self.widgets['eta_label'].grid(row=2, column=3, sticky=tk.E, 
                                   padx=(10, 0), pady=(5, 0))

### 6.3.3 Runs Table con Header Custom e Tooltip

def _create_runs_table(self, parent, row):
    """Create per-run results table with custom headers and tooltips"""
    runs_frame = ttk.LabelFrame(parent, text="Runs Overview", padding="5")
    runs_frame.grid(row=row, column=0, columnspan=2, 
                   sticky=(tk.W, tk.E, tk.N, tk.S), pady=(0, 10))
    runs_frame.columnconfigure(0, weight=1)
    runs_frame.rowconfigure(1, weight=1)
    
    # ===== CUSTOM HEADER FRAME WITH TOOLTIPS =====
    header_frame = ttk.Frame(runs_frame)
    header_frame.grid(row=0, column=0, sticky=(tk.W, tk.E))
    
    # Define headers with tooltips
    headers_with_tooltips = [
        ('Run', None, 60),
        ('Status', None, 80),
        ('PBIT Time', None, 80),
        ('B6 Status (P/F/K)', 'P = Pass\nF = Fail\nK = Known Failures', 120),
        ('B8 Checks (Pass/Fail)', 'Number of B8 checks\nPass/Fail format', 120),
        ('Serial Messages (E/F/R)', 'E = Errors\nF = Fatal\nR = Recycles', 120),
        ('Target', 'Target distance and cycle\nFormat: XXXXm @ cycle Y', 140),
        ('Target Test', None, 80)
    ]
    
    # Create header labels with tooltips
    for idx, (title, tooltip, width) in enumerate(headers_with_tooltips):
        label = ttk.Label(header_frame, text=title, 
                        font=('Arial', 9, 'bold'), 
                        anchor=tk.CENTER, relief=tk.RAISED, borderwidth=1)
        label.grid(row=0, column=idx, sticky=(tk.W, tk.E), ipadx=2, ipady=3)
        header_frame.columnconfigure(idx, weight=1 if idx > 0 else 0, 
                                    minsize=width)
        
        # Add tooltip if specified
        if tooltip:
            ToolTip(label, tooltip)
    
    # ===== TREEVIEW TABLE (hide built-in headers) =====
    cols = ('run', 'status', 'pbit_time', 'b6_status', 'b8_checks', 
           'serial_msgs', 'target', 'target_test')
    
    self.widgets['runs_table'] = ttk.Treeview(
        runs_frame, columns=cols, show='tree', height=12)
    self.widgets['runs_table'].grid(row=1, column=0, 
                                    sticky=(tk.W, tk.E, tk.N, tk.S))
    self.widgets['runs_table'].column('#0', width=0, stretch=tk.NO)
    
    # Set column widths
    column_widths = [60, 80, 80, 120, 120, 120, 140, 80]
    for c, width in zip(cols, column_widths):
        self.widgets['runs_table'].column(c, width=width, anchor=tk.CENTER)
    
    # Scrollbar
    runs_scroll = ttk.Scrollbar(runs_frame, orient=tk.VERTICAL, 
                               command=self.widgets['runs_table'].yview)
    runs_scroll.grid(row=1, column=1, sticky=(tk.N, tk.S))
    self.widgets['runs_table'].configure(yscrollcommand=runs_scroll.set)
    
    # Style tags for color coding
    self.widgets['runs_table'].tag_configure('pass', background='#d4edda')
    self.widgets['runs_table'].tag_configure('fail', background='#f8d7da')
    
    # Double-click for details
    self.widgets['runs_table'].bind('<Double-Button-1>', 
                                   self._on_run_double_click)

### 6.3.4 ToolTip Class

class ToolTip:
    """
    Creates a tooltip for a given widget.
    Shows tooltip on mouse enter, hides on leave.
    """
    def __init__(self, widget, text):
        self.widget = widget
        self.text = text
        self.tooltip_window = None
        self.widget.bind("<Enter>", self.show_tooltip)
        self.widget.bind("<Leave>", self.hide_tooltip)
    
    def show_tooltip(self, event=None):
        """Display tooltip below widget"""
        if self.tooltip_window or not self.text:
            return
        
        # Get widget position
        x = self.widget.winfo_rootx() + 20
        y = self.widget.winfo_rooty() + self.widget.winfo_height() + 5
        
        # Create tooltip window (no decorations)
        self.tooltip_window = tw = tk.Toplevel(self.widget)
        tw.wm_overrideredirect(True)
        tw.wm_geometry(f"+{x}+{y}")
        
        # Create label
        label = tk.Label(tw, text=self.text, justify=tk.LEFT,
                        background="#ffffe0", relief=tk.SOLID, 
                        borderwidth=1, font=("Arial", 9))
        label.pack(ipadx=5, ipady=3)
    
    def hide_tooltip(self, event=None):
        """Hide tooltip"""
        if self.tooltip_window:
            self.tooltip_window.destroy()
            self.tooltip_window = None

## 6.4 Aggiornamenti Real-Time

### 6.4.1 Update Status

def _update_status(self, data: Dict):
    """Apply status updates to progress widgets"""
    # Update run counter
    if 'run_current' in data:
        self.data['run_current'] = data['run_current']
    if 'run_total' in data:
        self.data['run_total'] = data['run_total']
    
    # Update label
    run_text = f"{self.data['run_current']} / {self.data['run_total']}"
    self.widgets['run_label'].config(text=run_text)
    
    # Update progress bar
    if self.data['run_total'] > 0:
        progress = (self.data['run_current'] / self.data['run_total']) * 100
        self.widgets['progress_bar']['value'] = progress
    
    # Update PBIT time
    if 'pbit_time' in data:
        self.data['pbit_time'] = data['pbit_time']
        pbit_text = f"{self.data['pbit_time']:.1f}s"
        self.widgets['pbit_label'].config(text=pbit_text)
    
    # Update ETA
    try:
        runs_remaining = self.data['run_total'] - self.data['run_current']
        if runs_remaining > 0 and self.data['run_current'] > 0:
            # Estimate based on average time per run so far
            avg_time_per_run = self.data.get('elapsed_total', 0) / self.data['run_current']
            eta_seconds = avg_time_per_run * runs_remaining
            eta_text = self._format_time(eta_seconds)
            percentage = (runs_remaining / self.data['run_total']) * 100
            self.widgets['eta_label'].config(
                text=f"{eta_text} ({percentage:.0f}%)")
    except Exception:
        pass

### 6.4.2 Update Run Row

def _update_run_row(self, data: Dict):
    """Update single row in runs table"""
    table = self.widgets.get('runs_table')
    if not table:
        return
    
    run_num = str(data.get('run'))
    if not table.exists(run_num):
        # Insert if missing
        total = self.data.get('run_total', 0)
        table.insert('', 'end', iid=run_num, 
                    values=(f'{run_num}/{total}', '', '', '', '', '', '', ''))
    
    # Read old values
    old = table.item(run_num).get('values', [])
    if not old:
        old = [''] * 8
    
    # Build new values
    new = list(old)
    
    # Column 0: Run number
    new[0] = f"{data.get('run')}/{self.data.get('run_total', '')}"
    
    # Column 1: Status (PASS/FAIL)
    if 'result' in data:
        new[1] = data['result']
    
    # Column 2: PBIT Time
    if 'pbit' in data:
        pbit = data['pbit']
        new[2] = f"{pbit:.1f}s" if isinstance(pbit, (int, float)) else str(pbit)
    
    # Column 3: B6 Status (P/F/K format)
    if 'b6_pass' in data or 'b6_fail' in data or 'b6_known_fail' in data:
        b6_pass = data.get('b6_pass', 0)
        b6_fail = data.get('b6_fail', 0)
        b6_known = data.get('b6_known_fail', 0)
        new[3] = f"{b6_pass}/{b6_fail}/{b6_known}"
    
    # Column 4: B8 Checks (Pass/Fail format)
    if 'b8_pass' in data or 'b8_checked' in data:
        b8_pass = data.get('b8_pass', 0)
        b8_fail = data.get('b8_fail', 0)
        b8_checked = data.get('b8_checked', 0)
        if b8_checked > 0:
            new[4] = f"{b8_pass}/{b8_fail}"
        else:
            new[4] = "-"
    
    # Column 5: Serial Messages (E/F/R format)
    if 'serial_errors' in data or 'serial_fatal' in data:
        serial_e = data.get('serial_errors', 0)
        serial_f = data.get('serial_fatal', 0)
        serial_r = data.get('serial_recycles', 0)
        new[5] = f"{serial_e}/{serial_f}/{serial_r}"
    
    # Column 6: Target (distance @ cycle format)
    if 'target_simulated' in data:
        tgt = data['target_simulated']
        if tgt and isinstance(tgt, dict):
            try:
                distance = tgt.get('distance', '?')
                cycle = tgt.get('appeared_after_cycles', 
                              tgt.get('found_at_iter', '?'))
                new[6] = f"{distance}m @ cycle {cycle}"
            except Exception:
                new[6] = "-"
        else:
            new[6] = "-"
    
    # Column 7: Target Test (PASS/FAIL/-)
    if 'target_detected' in data:
        detected = data['target_detected']
        if detected is None:
            new[7] = "-"
        elif detected:
            new[7] = "PASS"
        else:
            new[7] = "FAIL"
    
    # Update table
    table.item(run_num, values=new)
    
    # Apply color tag
    try:
        res = new[1].strip() if len(new) > 1 and isinstance(new[1], str) else ''
        if res.upper() == 'PASS':
            table.item(run_num, tags=('pass',))
        elif res.upper() == 'FAIL':
            table.item(run_num, tags=('fail',))
        else:
            table.item(run_num, tags=())
    except Exception:
        pass
    
    # Store metadata for detail dialog
    try:
        meta = self.runs_data.get(run_num, {})
        for k, v in data.items():
            meta[k] = v
        meta['display'] = {
            'status': new[1], 'pbit_time': new[2], 'b6_status': new[3],
            'b8_checks': new[4], 'serial_msgs': new[5], 
            'target': new[6], 'target_test': new[7]
        }
        self.runs_data[run_num] = meta
    except Exception:
        pass
    
    # Scroll to updated row
    try:
        table.see(run_num)
    except Exception:
        pass

## 6.5 Dialog Dettagli Run

### 6.5.1 Double-Click Handler

def _on_run_double_click(self, event=None):
    """Handle double-click on run row: open detail dialog"""
    try:
        table = self.widgets.get('runs_table')
        if not table:
            return
        
        sel = table.focus()
        if not sel:
            sel_items = table.selection()
            if not sel_items:
                return
            sel = sel_items[0]
        
        run_id = str(sel)
        meta = self.runs_data.get(run_id, {})
        
        # Build detail dialog
        self._show_run_detail_dialog(run_id, meta)
    
    except Exception as e:
        logging.error(f"Failed to open run detail: {e}")

### 6.5.2 Detail Dialog

def _show_run_detail_dialog(self, run_id: str, meta: Dict):
    """Show detailed run information in popup dialog"""
    dlg = tk.Toplevel(self.root)
    dlg.title(f"Run {run_id} Details")
    dlg.transient(self.root)
    
    frm = ttk.Frame(dlg, padding=12)
    frm.pack(fill=tk.BOTH, expand=True)
    
    # Header
    hdr = ttk.Label(frm, text=f"Run {run_id}", font=('Arial', 12, 'bold'))
    hdr.pack(anchor=tk.W)
    
    # Key/value grid
    grid = ttk.Frame(frm)
    grid.pack(fill=tk.X, pady=(6, 6))
    
    def _kv(row, key, val):
        """Helper to add key-value pair"""
        ttk.Label(grid, text=key+":", font=('Arial', 9, 'bold')).grid(
            row=row, column=0, sticky=tk.W, padx=(0,6))
        ttk.Label(grid, text=str(val), font=('Arial', 9)).grid(
            row=row, column=1, sticky=tk.W)
    
    display = meta.get('display', {})
    _kv(0, 'Status', display.get('status', ''))
    _kv(1, 'PBIT Time', display.get('pbit_time', ''))
    _kv(2, 'B6 Status (P/F/K)', display.get('b6_status', ''))
    _kv(3, 'B8 Checks', display.get('b8_checks', ''))
    _kv(4, 'Serial Msgs (E/F/R)', display.get('serial_msgs', ''))
    _kv(5, 'Target', display.get('target', ''))
    _kv(6, 'Target Test', display.get('target_test', ''))
    
    # Timing details
    start = meta.get('start_time')
    end = meta.get('end_time')
    duration = meta.get('run_duration')
    _kv(7, 'Start Time', start)
    _kv(8, 'End Time', end)
    _kv(9, 'Run Duration', f"{duration:.2f}s" if duration else "N/A")
    
    # Target details
    tgt_detected = meta.get('target_detected', None)
    tgt_status = 'YES' if tgt_detected is True else ('NO' if tgt_detected is False else 'N/A')
    _kv(10, 'Target Detected', tgt_status)
    
    # Failures detail (multi-line text box)
    ttk.Label(frm, text='Failures (detailed):', 
             font=('Arial', 9, 'bold')).pack(anchor=tk.W, pady=(6,0))
    failures_text = tk.Text(frm, height=6, width=80, 
                           font=('Courier', 9), wrap=tk.WORD)
    failures = meta.get('failures') or ''
    if isinstance(failures, (list, tuple)):
        body = '\n'.join([str(x) for x in failures])
    else:
        body = str(failures)
    failures_text.insert('1.0', body)
    failures_text.config(state='disabled')
    failures_text.pack(fill=tk.BOTH, expand=False, pady=(2,6))
    
    # Close button
    btn_frame = ttk.Frame(frm)
    btn_frame.pack(fill=tk.X, pady=(6,0))
    close_btn = ttk.Button(btn_frame, text='Close', 
                          command=lambda: dlg.destroy())
    close_btn.pack(side=tk.RIGHT)
    
    # Center dialog
    dlg.update_idletasks()
    w = dlg.winfo_reqwidth()
    h = dlg.winfo_reqheight()
    px = self.root.winfo_rootx()
    py = self.root.winfo_rooty()
    pw = self.root.winfo_width()
    ph = self.root.winfo_height()
    x = px + max(0, (pw - w)//2)
    y = py + max(0, (ph - h)//2)
    dlg.geometry(f"{w}x{h}+{x}+{y}")

## 6.6 Best Practices GUI

### 6.6.1 Thread Safety Checklist

✅ DO:
- Usa update_queue.put() per comunicare da worker thread a GUI thread
- Chiama widget methods solo da GUI thread (in _poll_updates o callbacks)
- Usa daemon=True per GUI thread (non blocca exit)

❌ DON’T:
- Non chiamare widget methods direttamente da worker thread
- Non usare variabili shared senza lock (usa queue invece)
- Non fare operazioni blocking in GUI thread (blocca UI)

### 6.6.2 Performance Tips

Polling Frequency: 100ms (10 Hz) è un buon bilanciamento responsività/CPU

Batch Updates: _poll_updates processa tutti i messaggi in queue prima di re-schedule

Lazy Rendering: Tabella usa virtualizzazione (solo righe visibili renderizzate)

### 6.6.3 Error Handling

def _poll_updates(self):
    """Poll with error isolation"""
    try:
        while not self.update_queue.empty():
            try:
                update_type, data = self.update_queue.get_nowait()
                # Process update...
            except Exception as e:
                # Log but continue processing other updates
                logging.error(f"Failed to process update {update_type}: {e}")
    except Exception as e:
        # Critical error in polling loop
        logging.error(f"GUI polling error: {e}")
    finally:
        # Always re-schedule (unless stopping)
        if self.running:
            self.root.after(100, self._poll_updates)

← Modalità Esecuzione | Torna all’Indice | Prossimo: Output e Report →X

# — 07_Output_Report.md —

# Capitolo 7: Output e Report

← Interfaccia Grafica | Torna all’Indice | Prossimo: Configurazione →X

|  |  |  | ## 7.1 Overview Output |
| --- | --- | --- | --- |

Il test genera tre tipi di output per ogni esecuzione completa:

Formato

File

Scopo

Strumento

PDF

GRIFO_M_PBIT_YYYY-MM-DD_HH-MM-SS.pdf

Report formale leggibile da umani

Visualizzazione e archiviazione

CSV

GRIFO_M_PBIT_YYYY-MM-DD_HH-MM-SS.csv

Dati tabulari per analisi

Excel, Python pandas, R

JSON

GRIFO_M_PBIT_YYYY-MM-DD_HH-MM-SS.json

Dati strutturati machine-readable

Parsing automatico, CI/CD

Cartelle Output:

TestEnvironment/
├── pdf_reports/           # PDF generati
├── LOG/                   # CSV generati
└── json/                  # Template JSON (non output)

### 7.1.1 Naming Convention

Pattern: GRIFO_M_PBIT_<timestamp>.<ext>

Timestamp Format: YYYY-MM-DD_HH-MM-SS (ISO 8601 adapted)

Esempi:

GRIFO_M_PBIT_2026-02-05_15-24-38.pdf
GRIFO_M_PBIT_2026-02-05_15-24-38.csv
GRIFO_M_PBIT_2026-02-05_15-24-38.json

Vantaggi:
- ✅ Sorting alfabetico = sorting cronologico
- ✅ No spazi (shell-friendly)
- ✅ Timestamp leggibile da umani e machine
- ✅ Filename univoco (collisioni solo se <1s tra run)

## 7.2 Report PDF

### 7.2.1 Struttura PDF

Modulo: TestEnvironment/env/leo_grifo_test_report.py

Funzione Principale: generate_pdf_report(output_folder, json_data)

Layout PDF (3 pagine tipiche):

┌─────────────────────────────────────────────────────────────┐
│  PAGE 1: HEADER & RUN SUMMARY                               │
├─────────────────────────────────────────────────────────────┤
│  ┌─────────────────────────────────────────────────────┐    │
│  │  GRIFO M-PBIT TEST REPORT                           │    │
│  │  Date: 2026-02-05  Time: 15:24:38                   │    │
│  │  ================================================     │    │
│  └─────────────────────────────────────────────────────┘    │
│                                                              │
│  EXECUTIVE SUMMARY                                           │
│  ┌──────────────────────────────────────────┐               │
│  │  Total Runs:        10                   │               │
│  │  Passed:            8  (80.0%)           │               │
│  │  Failed:            2  (20.0%)           │               │
│  │  Total Duration:    8m 42s               │               │
│  │  Average Run Time:  52.2s                │               │
│  └──────────────────────────────────────────┘               │
│                                                              │
│  RUN DETAILS TABLE                                           │
│  ┌────┬──────┬──────┬─────────┬─────────┬──────┬────────┐  │
│  │Run │Status│ PBIT │B6 Status│B8 Checks│Serial│ Target │  │
│  │    │      │ Time │ (P/F/K) │(Pass/F) │(E/F/R│        │  │
│  ├────┼──────┼──────┼─────────┼─────────┼──────┼────────┤  │
│  │1/10│PASS  │ 2.2s │11/0/1   │173/0    │0/0/0 │2000m@3 │  │
│  │2/10│PASS  │ 0.7s │11/0/1   │173/0    │0/0/0 │  -     │  │
│  │3/10│FAIL  │ 0.1s │9/2/1    │  -      │0/3/3 │  -     │  │
│  │4/10│FAIL  │ 0.1s │9/2/1    │173/0    │0/3/0 │  -     │  │
│  │5/10│PASS  │ 2.0s │9/2/1    │173/0    │0/0/0 │  -     │  │
│  │6/10│PASS  │ 0.3s │11/0/1   │173/0    │0/2/2 │  -     │  │
│  │7/10│PASS  │ 0.0s │11/0/1   │173/0    │0/2/2 │2000m@3 │  │
│  │8/10│PASS  │ 2.9s │11/0/1   │173/0    │0/0/0 │  -     │  │
│  │9/10│PASS  │ 2.1s │11/0/1   │173/0    │0/0/0 │  -     │  │
│  │10/1│PASS  │ 0.8s │11/0/1   │173/0    │0/0/0 │  -     │  │
│  └────┴──────┴──────┴─────────┴─────────┴──────┴────────┘  │
└─────────────────────────────────────────────────────────────┘

┌─────────────────────────────────────────────────────────────┐
│  PAGE 2: AGGREGATED STATISTICS                              │
├─────────────────────────────────────────────────────────────┤
│  B6 STATUS STATISTICS (across all runs)                     │
│  ┌────────────────────────────────────────┬──────┬─────┐    │
│  │ Field Name                             │ P/F  │ Rate│    │
│  ├────────────────────────────────────────┼──────┼─────┤    │
│  │ processor_status                       │ 8/2  │ 80% │    │
│  │ transmitter_status                     │ 8/2  │ 80% │    │
│  │ receiver_status                        │10/0  │100% │    │
│  │ antenna_servo_status                   │10/0  │100% │    │
│  │ ... (remaining 8 fields)               │      │     │    │
│  └────────────────────────────────────────┴──────┴─────┘    │
│                                                              │
│  B8 FIELD STATISTICS (top failures only, max 30 rows)       │
│  ┌────────────────────────────────────────┬──────┬─────┐    │
│  │ Field Name                             │ P/F  │ Rate│    │
│  ├────────────────────────────────────────┼──────┼─────┤    │
│  │ radar_mode                             │ 9/1  │ 90% │    │
│  │ scan_sector_azimuth                    │10/0  │100% │    │
│  │ ... (filtered to show failures first)  │      │     │    │
│  └────────────────────────────────────────┴──────┴─────┘    │
│                                                              │
│  SERIAL TERMINAL STATISTICS                                  │
│  ┌────────────────────────────────────────┬────────────┐    │
│  │ Metric                                 │ Value      │    │
│  ├────────────────────────────────────────┼────────────┤    │
│  │ Total Errors (%%E)                     │      0     │    │
│  │ Total Fatal (%%F)                      │      7     │    │
│  │ Total Recycles (RECYCLE)               │      7     │    │
│  │ Average per Run                        │    0.7     │    │
│  └────────────────────────────────────────┴────────────┘    │
└─────────────────────────────────────────────────────────────┘

┌─────────────────────────────────────────────────────────────┐
│  PAGE 3: FAILURES DETAIL                                    │
├─────────────────────────────────────────────────────────────┤
│  FAILED RUNS BREAKDOWN                                       │
│                                                              │
│  Run 3/10 - FAILED                                           │
│  ├─ B6 Status: 9/2/1 (processor_status, transmitter_status) │
│  ├─ B8 Checks: Skipped (PBIT incomplete)                    │
│  ├─ Serial: 0/3/3 (Fatal errors detected)                   │
│  ├─ Failure Reason:                                          │
│  │    - B6 Check Failed: processor_status [expected=1,      │
│  │                                         actual=0]         │
│  │    - B6 Check Failed: transmitter_status [expected=1,    │
│  │                                           actual=0]       │
│  │    - Terminal detected 3 FATAL (%%F) messages           │
│  │    - Terminal detected 3 RECYCLE events                  │
│  └─ Scenario: Random Component Failures (known config)      │
│                                                              │
│  Run 4/10 - FAILED                                           │
│  ├─ B6 Status: 9/2/1 (processor_status, transmitter_status) │
│  ├─ B8 Checks: 173/0 (all passed)                           │
│  ├─ Serial: 0/3/0 (Fatal errors detected)                   │
│  ├─ Failure Reason:                                          │
│  │    - B6 Check Failed: processor_status [expected=1,      │
│  │                                         actual=0]         │
│  │    - B6 Check Failed: transmitter_status [expected=1,    │
│  │                                           actual=0]       │
│  │    - Terminal detected 3 FATAL (%%F) messages           │
│  └─ Scenario: Random Component Failures (known config)      │
│                                                              │
│  KNOWN FAILURES                                              │
│  ┌─────────────────────────────────────────────────────┐    │
│  │ The following failures were EXPECTED:                │    │
│  │ - B6 field 'bit_test_2_status' (known ICD mismatch) │    │
│  │ - All runs show this failure (marked as Known)       │    │
│  └─────────────────────────────────────────────────────┘    │
└─────────────────────────────────────────────────────────────┘

### 7.2.2 Generazione PDF - Codice

def generate_pdf_report(output_folder: str, json_data: Dict) -> str:
    """
    Generate PDF report from JSON data.
    
    Args:
        output_folder: Directory for PDF output
        json_data: Test results (parsed from JSON template)
    
    Returns:
        Full path to generated PDF file
    """
    from reportlab.lib import colors
    from reportlab.lib.pagesizes import letter, A4
    from reportlab.lib.styles import getSampleStyleSheet, ParagraphStyle
    from reportlab.lib.units import inch
    from reportlab.platypus import (
        SimpleDocTemplate, Table, TableStyle, Paragraph, Spacer, PageBreak
    )
    from reportlab.lib.enums import TA_CENTER, TA_LEFT
    
    # Extract metadata
    timestamp = json_data.get('test_metadata', {}).get('timestamp', 'unknown')
    total_runs = json_data.get('test_metadata', {}).get('total_runs', 0)
    passed = json_data.get('test_metadata', {}).get('runs_passed', 0)
    failed = json_data.get('test_metadata', {}).get('runs_failed', 0)
    duration = json_data.get('test_metadata', {}).get('total_duration', 0)
    
    # Create filename
    pdf_filename = f"GRIFO_M_PBIT_{timestamp}.pdf"
    pdf_path = os.path.join(output_folder, pdf_filename)
    
    # Create PDF document
    doc = SimpleDocTemplate(pdf_path, pagesize=A4,
                           rightMargin=36, leftMargin=36,
                           topMargin=36, bottomMargin=36)
    
    # Build content
    story = []
    styles = getSampleStyleSheet()
    
    # === PAGE 1: HEADER ===
    title_style = ParagraphStyle(
        'CustomTitle',
        parent=styles['Heading1'],
        fontSize=18,
        textColor=colors.HexColor('#1a5490'),
        spaceAfter=30,
        alignment=TA_CENTER
    )
    story.append(Paragraph("GRIFO M-PBIT TEST REPORT", title_style))
    story.append(Spacer(1, 0.2*inch))
    
    # Metadata
    meta_data = [
        ['Date:', timestamp.split('_')[0]],
        ['Time:', timestamp.split('_')[1].replace('-', ':') if '_' in timestamp else ''],
        ['Total Runs:', str(total_runs)],
        ['Duration:', _format_duration(duration)]
    ]
    meta_table = Table(meta_data, colWidths=[1.5*inch, 3*inch])
    meta_table.setStyle(TableStyle([
        ('FONT', (0, 0), (-1, -1), 'Helvetica', 10),
        ('FONT', (0, 0), (0, -1), 'Helvetica-Bold', 10),
        ('ALIGN', (0, 0), (0, -1), 'RIGHT'),
        ('ALIGN', (1, 0), (1, -1), 'LEFT'),
        ('VALIGN', (0, 0), (-1, -1), 'TOP'),
    ]))
    story.append(meta_table)
    story.append(Spacer(1, 0.3*inch))
    
    # === EXECUTIVE SUMMARY ===
    story.append(Paragraph("EXECUTIVE SUMMARY", styles['Heading2']))
    story.append(Spacer(1, 0.1*inch))
    
    pass_rate = (passed / total_runs * 100) if total_runs > 0 else 0
    fail_rate = (failed / total_runs * 100) if total_runs > 0 else 0
    avg_time = duration / total_runs if total_runs > 0 else 0
    
    summary_data = [
        ['Total Runs:', str(total_runs)],
        ['Passed:', f"{passed}  ({pass_rate:.1f}%)"],
        ['Failed:', f"{failed}  ({fail_rate:.1f}%)"],
        ['Average Run Time:', f"{avg_time:.1f}s"]
    ]
    summary_table = Table(summary_data, colWidths=[2*inch, 3*inch])
    summary_table.setStyle(TableStyle([
        ('FONT', (0, 0), (-1, -1), 'Helvetica', 11),
        ('FONT', (0, 0), (0, -1), 'Helvetica-Bold', 11),
        ('BACKGROUND', (0, 0), (-1, -1), colors.HexColor('#f0f0f0')),
        ('BOX', (0, 0), (-1, -1), 1, colors.black),
        ('INNERGRID', (0, 0), (-1, -1), 0.5, colors.grey),
        ('ALIGN', (0, 0), (0, -1), 'RIGHT'),
        ('ALIGN', (1, 0), (1, -1), 'LEFT'),
        ('VALIGN', (0, 0), (-1, -1), 'MIDDLE'),
        ('LEFTPADDING', (0, 0), (-1, -1), 8),
        ('RIGHTPADDING', (0, 0), (-1, -1), 8),
        ('TOPPADDING', (0, 0), (-1, -1), 6),
        ('BOTTOMPADDING', (0, 0), (-1, -1), 6),
    ]))
    story.append(summary_table)
    story.append(Spacer(1, 0.4*inch))
    
    # === RUN DETAILS TABLE ===
    story.append(Paragraph("RUN DETAILS", styles['Heading2']))
    story.append(Spacer(1, 0.1*inch))
    
    # Build table data
    table_data = [
        ['Run', 'Status', 'PBIT\nTime', 'B6 Status\n(P/F/K)', 
         'B8 Checks\n(Pass/Fail)', 'Serial\n(E/F/R)', 'Target']
    ]
    
    runs = json_data.get('runs', [])
    for r in runs:
        run_id = r.get('run_id', '')
        status = r.get('result', '')
        pbit_time = r.get('pbit_time', 0)
        b6_pass = r.get('b6_pass', 0)
        b6_fail = r.get('b6_fail', 0)
        b6_known = r.get('b6_known_fail', 0)
        b8_pass = r.get('b8_pass', 0)
        b8_fail = r.get('b8_fail', 0)
        serial_e = r.get('serial_errors', 0)
        serial_f = r.get('serial_fatal', 0)
        serial_r = r.get('serial_recycles', 0)
        target = r.get('target_info', '-')
        
        table_data.append([
            f"{run_id}/{total_runs}",
            status,
            f"{pbit_time:.1f}s" if pbit_time else '-',
            f"{b6_pass}/{b6_fail}/{b6_known}",
            f"{b8_pass}/{b8_fail}" if b8_pass or b8_fail else '-',
            f"{serial_e}/{serial_f}/{serial_r}",
            target
        ])
    
    runs_table = Table(table_data, colWidths=[
        0.6*inch, 0.7*inch, 0.7*inch, 0.9*inch, 
        0.9*inch, 0.8*inch, 1.5*inch
    ])
    
    # Style: header row, alternating colors, borders
    table_style = [
        ('FONT', (0, 0), (-1, 0), 'Helvetica-Bold', 9),
        ('FONT', (0, 1), (-1, -1), 'Helvetica', 8),
        ('BACKGROUND', (0, 0), (-1, 0), colors.HexColor('#4a86e8')),
        ('TEXTCOLOR', (0, 0), (-1, 0), colors.whitesmoke),
        ('ALIGN', (0, 0), (-1, -1), 'CENTER'),
        ('VALIGN', (0, 0), (-1, -1), 'MIDDLE'),
        ('INNERGRID', (0, 0), (-1, -1), 0.5, colors.grey),
        ('BOX', (0, 0), (-1, -1), 1, colors.black),
    ]
    
    # Alternating row colors
    for i in range(1, len(table_data)):
        bg = colors.HexColor('#f9f9f9') if i % 2 == 0 else colors.white
        table_style.append(('BACKGROUND', (0, i), (-1, i), bg))
        
        # Color code status column
        status = table_data[i][1]
        if status.upper() == 'PASS':
            table_style.append(('TEXTCOLOR', (1, i), (1, i), colors.green))
            table_style.append(('FONT', (1, i), (1, i), 'Helvetica-Bold', 9))
        elif status.upper() == 'FAIL':
            table_style.append(('TEXTCOLOR', (1, i), (1, i), colors.red))
            table_style.append(('FONT', (1, i), (1, i), 'Helvetica-Bold', 9))
    
    runs_table.setStyle(TableStyle(table_style))
    story.append(runs_table)
    story.append(PageBreak())
    
    # === PAGE 2: STATISTICS ===
    story.append(Paragraph("AGGREGATED STATISTICS", styles['Heading2']))
    story.append(Spacer(1, 0.2*inch))
    
    # B6 Statistics
    story.append(Paragraph("B6 Status Statistics", styles['Heading3']))
    b6_stats = json_data.get('b6_statistics', {})
    if b6_stats:
        b6_data = [['Field Name', 'Pass/Fail', 'Success Rate']]
        for field, stats in b6_stats.items():
            passed_count = stats.get('passed', 0)
            failed_count = stats.get('failed', 0)
            total = passed_count + failed_count
            rate = (passed_count / total * 100) if total > 0 else 0
            b6_data.append([field, f"{passed_count}/{failed_count}", f"{rate:.0f}%"])
        
        b6_table = Table(b6_data, colWidths=[3.5*inch, 1*inch, 1*inch])
        b6_table.setStyle(TableStyle([
            ('FONT', (0, 0), (-1, 0), 'Helvetica-Bold', 9),
            ('FONT', (0, 1), (-1, -1), 'Helvetica', 8),
            ('BACKGROUND', (0, 0), (-1, 0), colors.HexColor('#6aa84f')),
            ('TEXTCOLOR', (0, 0), (-1, 0), colors.whitesmoke),
            ('ALIGN', (0, 0), (0, -1), 'LEFT'),
            ('ALIGN', (1, 0), (-1, -1), 'CENTER'),
            ('INNERGRID', (0, 0), (-1, -1), 0.5, colors.grey),
            ('BOX', (0, 0), (-1, -1), 1, colors.black),
        ]))
        story.append(b6_table)
    
    story.append(Spacer(1, 0.3*inch))
    
    # B8 Statistics (top 30 rows only)
    story.append(Paragraph("B8 Field Statistics (Top Failures)", styles['Heading3']))
    b8_stats = json_data.get('b8_statistics', {})
    if b8_stats:
        # Sort by failure count descending
        sorted_b8 = sorted(b8_stats.items(), 
                          key=lambda x: x[1].get('failed', 0), 
                          reverse=True)
        
        b8_data = [['Field Name', 'Pass/Fail', 'Success Rate']]
        for field, stats in sorted_b8[:30]:  # Max 30 rows
            passed_count = stats.get('passed', 0)
            failed_count = stats.get('failed', 0)
            total = passed_count + failed_count
            rate = (passed_count / total * 100) if total > 0 else 0
            b8_data.append([field, f"{passed_count}/{failed_count}", f"{rate:.0f}%"])
        
        b8_table = Table(b8_data, colWidths=[3.5*inch, 1*inch, 1*inch])
        b8_table.setStyle(TableStyle([
            ('FONT', (0, 0), (-1, 0), 'Helvetica-Bold', 9),
            ('FONT', (0, 1), (-1, -1), 'Helvetica', 8),
            ('BACKGROUND', (0, 0), (-1, 0), colors.HexColor('#6aa84f')),
            ('TEXTCOLOR', (0, 0), (-1, 0), colors.whitesmoke),
            ('ALIGN', (0, 0), (0, -1), 'LEFT'),
            ('ALIGN', (1, 0), (-1, -1), 'CENTER'),
            ('INNERGRID', (0, 0), (-1, -1), 0.5, colors.grey),
            ('BOX', (0, 0), (-1, -1), 1, colors.black),
        ]))
        story.append(b8_table)
    
    story.append(PageBreak())
    
    # === PAGE 3: FAILURES DETAIL ===
    story.append(Paragraph("FAILURES DETAIL", styles['Heading2']))
    story.append(Spacer(1, 0.2*inch))
    
    failed_runs = [r for r in runs if r.get('result', '').upper() == 'FAIL']
    if failed_runs:
        for r in failed_runs:
            run_id = r.get('run_id', '')
            story.append(Paragraph(f"Run {run_id}/{total_runs} - FAILED", 
                                  styles['Heading3']))
            
            # Failure details
            failures = r.get('failures', [])
            if failures:
                for f in failures:
                    story.append(Paragraph(f"• {f}", styles['BodyText']))
            
            story.append(Spacer(1, 0.2*inch))
    else:
        story.append(Paragraph("No failed runs.", styles['BodyText']))
    
    # Build PDF
    doc.build(story)
    
    logging.info(f"PDF report generated: {pdf_path}")
    return pdf_path

## 7.3 Export CSV

### 7.3.1 Struttura CSV

File: TestEnvironment/LOG/GRIFO_M_PBIT_YYYY-MM-DD_HH-MM-SS.csv

Formato: UTF-8, comma-separated, header row

Colonne (25 colonne totali):

run_id,result,pbit_time,b6_pass,b6_fail,b6_known_fail,b6_checked,b8_pass,b8_fail,b8_checked,serial_errors,serial_fatal,serial_recycles,target_detected,target_distance,target_cycle,run_duration,start_time,end_time,scenario_name,processor_status,transmitter_status,receiver_status,antenna_status,pbit_complete,bit_test_2_status
1,PASS,2.2,11,0,1,12,173,0,173,0,0,0,True,2000,3,65.3,15:24:38,15:25:43,Scenario 1,1,1,1,1,1,0
2,PASS,0.7,11,0,1,12,173,0,173,0,0,0,False,,,52.1,15:25:44,15:26:36,Scenario 1,1,1,1,1,1,0
3,FAIL,0.1,9,2,1,12,0,0,0,0,3,3,False,,,2.8,15:26:37,15:26:40,Random Failures,0,0,1,1,0,0
...

### 7.3.2 Generazione CSV - Codice

def export_to_csv(output_folder: str, json_data: Dict) -> str:
    """
    Export test results to CSV for data analysis.
    
    Args:
        output_folder: Directory for CSV output (usually TestEnvironment/LOG/)
        json_data: Test results dictionary
    
    Returns:
        Full path to generated CSV file
    """
    import csv
    
    timestamp = json_data.get('test_metadata', {}).get('timestamp', 'unknown')
    csv_filename = f"GRIFO_M_PBIT_{timestamp}.csv"
    csv_path = os.path.join(output_folder, csv_filename)
    
    # Define CSV columns
    fieldnames = [
        'run_id', 'result', 'pbit_time',
        'b6_pass', 'b6_fail', 'b6_known_fail', 'b6_checked',
        'b8_pass', 'b8_fail', 'b8_checked',
        'serial_errors', 'serial_fatal', 'serial_recycles',
        'target_detected', 'target_distance', 'target_cycle',
        'run_duration', 'start_time', 'end_time',
        'scenario_name',
        # B6 individual fields (example subset)
        'processor_status', 'transmitter_status', 'receiver_status',
        'antenna_status', 'pbit_complete', 'bit_test_2_status'
    ]
    
    runs = json_data.get('runs', [])
    
    with open(csv_path, 'w', newline='', encoding='utf-8') as csvfile:
        writer = csv.DictWriter(csvfile, fieldnames=fieldnames, 
                               extrasaction='ignore')
        writer.writeheader()
        
        for r in runs:
            # Flatten nested data
            row = {
                'run_id': r.get('run_id', ''),
                'result': r.get('result', ''),
                'pbit_time': r.get('pbit_time', ''),
                'b6_pass': r.get('b6_pass', 0),
                'b6_fail': r.get('b6_fail', 0),
                'b6_known_fail': r.get('b6_known_fail', 0),
                'b6_checked': r.get('b6_checked', 0),
                'b8_pass': r.get('b8_pass', 0),
                'b8_fail': r.get('b8_fail', 0),
                'b8_checked': r.get('b8_checked', 0),
                'serial_errors': r.get('serial_errors', 0),
                'serial_fatal': r.get('serial_fatal', 0),
                'serial_recycles': r.get('serial_recycles', 0),
                'target_detected': r.get('target_detected', ''),
                'run_duration': r.get('run_duration', ''),
                'start_time': r.get('start_time', ''),
                'end_time': r.get('end_time', ''),
                'scenario_name': r.get('scenario_name', ''),
            }
            
            # Extract B6 field values
            b6_results = r.get('b6_results', {})
            row['processor_status'] = b6_results.get('processor_status', '')
            row['transmitter_status'] = b6_results.get('transmitter_status', '')
            row['receiver_status'] = b6_results.get('receiver_status', '')
            row['antenna_status'] = b6_results.get('antenna_servo_status', '')
            row['pbit_complete'] = b6_results.get('pbit_complete', '')
            row['bit_test_2_status'] = b6_results.get('bit_test_2_status', '')
            
            # Extract target info
            target = r.get('target_simulated', {})
            if target:
                row['target_distance'] = target.get('distance', '')
                row['target_cycle'] = target.get('appeared_after_cycles', '')
            
            writer.writerow(row)
    
    logging.info(f"CSV export completed: {csv_path}")
    return csv_path

### 7.3.3 Uso CSV per Analisi

Pandas Example:

import pandas as pd

# Load CSV
df = pd.read_csv('GRIFO_M_PBIT_2026-02-05_15-24-38.csv')

# Calculate pass rate
pass_rate = (df['result'] == 'PASS').sum() / len(df)
print(f"Pass Rate: {pass_rate:.1%}")

# Average PBIT time by scenario
avg_pbit = df.groupby('scenario_name')['pbit_time'].mean()
print(avg_pbit)

# B6 field failure analysis
b6_fields = ['processor_status', 'transmitter_status', 'receiver_status']
for field in b6_fields:
    fail_count = (df[field] == 0).sum()
    print(f"{field}: {fail_count} failures")

# Serial message correlation
print(df[['serial_fatal', 'serial_recycles', 'result']].corr())

## 7.4 JSON Output

### 7.4.1 Schema JSON

File: GRIFO_M_PBIT_YYYY-MM-DD_HH-MM-SS.json

Struttura Completa:

{
  "test_metadata": {
    "timestamp": "2026-02-05_15-24-38",
    "test_name": "GRIFO_M_PBIT",
    "total_runs": 10,
    "runs_passed": 8,
    "runs_failed": 2,
    "total_duration": 522.5,
    "hardware_mode": false,
    "simulation_mode": true,
    "python_version": "3.10.11",
    "script_version": "2.0"
  },
  
  "runs": [
    {
      "run_id": 1,
      "result": "PASS",
      "start_time": "15:24:38",
      "end_time": "15:25:43",
      "run_duration": 65.3,
      
      "pbit": {
        "pbit_time": 2.2,
        "pbit_available": 0.0,
        "pbit_complete": true
      },
      
      "b6_results": {
        "b6_pass": 11,
        "b6_fail": 0,
        "b6_known_fail": 1,
        "b6_checked": 12,
        "fields": {
          "processor_status": {"value": 1, "expected": 1, "result": "PASS"},
          "transmitter_status": {"value": 1, "expected": 1, "result": "PASS"},
          "receiver_status": {"value": 1, "expected": 1, "result": "PASS"},
          "antenna_servo_status": {"value": 1, "expected": 1, "result": "PASS"},
          "pbit_complete": {"value": 1, "expected": 1, "result": "PASS"},
          "bit_test_1_status": {"value": 1, "expected": 1, "result": "PASS"},
          "bit_test_2_status": {"value": 0, "expected": 1, "result": "KNOWN_FAIL"},
          "cooling_status": {"value": 1, "expected": 1, "result": "PASS"},
          "power_supply_status": {"value": 1, "expected": 1, "result": "PASS"},
          "memory_status": {"value": 1, "expected": 1, "result": "PASS"},
          "software_version": {"value": 123, "expected": 123, "result": "PASS"},
          "hardware_revision": {"value": 456, "expected": 456, "result": "PASS"}
        }
      },
      
      "b8_results": {
        "b8_pass": 173,
        "b8_fail": 0,
        "b8_checked": 173,
        "failures": []
      },
      
      "serial_results": {
        "serial_errors": 0,
        "serial_fatal": 0,
        "serial_recycles": 0,
        "messages": []
      },
      
      "target_test": {
        "target_detected": true,
        "target_simulated": {
          "distance": 2000,
          "appeared_after_cycles": 3,
          "found_at_iter": 3
        }
      },
      
      "scenario": {
        "name": "Scenario 1: All Systems Nominal",
        "description": "Baseline test with all components functioning correctly"
      },
      
      "failures": []
    },
    
    {
      "run_id": 3,
      "result": "FAIL",
      "start_time": "15:26:37",
      "end_time": "15:26:40",
      "run_duration": 2.8,
      
      "pbit": {
        "pbit_time": 0.1,
        "pbit_available": 0.0,
        "pbit_complete": false
      },
      
      "b6_results": {
        "b6_pass": 9,
        "b6_fail": 2,
        "b6_known_fail": 1,
        "b6_checked": 12,
        "fields": {
          "processor_status": {"value": 0, "expected": 1, "result": "FAIL"},
          "transmitter_status": {"value": 0, "expected": 1, "result": "FAIL"},
          "receiver_status": {"value": 1, "expected": 1, "result": "PASS"}
        }
      },
      
      "b8_results": {
        "b8_pass": 0,
        "b8_fail": 0,
        "b8_checked": 0,
        "failures": [],
        "skipped": true,
        "skip_reason": "PBIT incomplete"
      },
      
      "serial_results": {
        "serial_errors": 0,
        "serial_fatal": 3,
        "serial_recycles": 3,
        "messages": [
          {"type": "FATAL", "text": "%%F Processor failure detected"},
          {"type": "FATAL", "text": "%%F Transmitter offline"},
          {"type": "FATAL", "text": "%%F System RECYCLE initiated"}
        ]
      },
      
      "target_test": {
        "target_detected": false,
        "target_simulated": null,
        "skipped": true,
        "skip_reason": "Run failed before target test"
      },
      
      "scenario": {
        "name": "Random Component Failures",
        "description": "Random selection of component failures (30% probability)"
      },
      
      "failures": [
        "B6 Check Failed: processor_status [expected=1, actual=0]",
        "B6 Check Failed: transmitter_status [expected=1, actual=0]",
        "Terminal detected 3 FATAL (%%F) messages",
        "Terminal detected 3 RECYCLE events"
      ]
    }
  ],
  
  "b6_statistics": {
    "processor_status": {"passed": 8, "failed": 2, "success_rate": 0.8},
    "transmitter_status": {"passed": 8, "failed": 2, "success_rate": 0.8},
    "receiver_status": {"passed": 10, "failed": 0, "success_rate": 1.0},
    "antenna_servo_status": {"passed": 10, "failed": 0, "success_rate": 1.0}
  },
  
  "b8_statistics": {
    "radar_mode": {"passed": 9, "failed": 1, "success_rate": 0.9}
  },
  
  "serial_statistics": {
    "total_errors": 0,
    "total_fatal": 7,
    "total_recycles": 7,
    "average_per_run": 0.7
  },
  
  "target_statistics": {
    "runs_with_target_test": 2,
    "targets_detected": 2,
    "detection_rate": 1.0,
    "average_cycle_to_detection": 3.0
  }
}

### 7.4.2 Parsing JSON

Python Example:

import json

# Load JSON
with open('GRIFO_M_PBIT_2026-02-05_15-24-38.json', 'r') as f:
    data = json.load(f)

# Access metadata
print(f"Total Runs: {data['test_metadata']['total_runs']}")
print(f"Pass Rate: {data['test_metadata']['runs_passed']/data['test_metadata']['total_runs']:.1%}")

# Iterate runs
for run in data['runs']:
    print(f"Run {run['run_id']}: {run['result']}")
    
    # B6 failures
    for field, info in run['b6_results']['fields'].items():
        if info['result'] == 'FAIL':
            print(f"  B6 FAIL: {field} = {info['value']} (expected {info['expected']})")
    
    # Serial issues
    serial = run['serial_results']
    if serial['serial_fatal'] > 0:
        print(f"  {serial['serial_fatal']} FATAL messages")

## 7.5 Template JSON (Input)

### 7.5.1 Scopo Template

Cartella: TestEnvironment/json/

File: grifo_test_template.json

Uso: Define expected structure and default values

{
  "test_metadata": {
    "timestamp": "",
    "test_name": "GRIFO_M_PBIT",
    "total_runs": 0,
    "runs_passed": 0,
    "runs_failed": 0,
    "total_duration": 0.0,
    "hardware_mode": false,
    "simulation_mode": false
  },
  "runs": [],
  "b6_statistics": {},
  "b8_statistics": {},
  "serial_statistics": {},
  "target_statistics": {}
}

Loading Template:

def load_test_template() -> Dict:
    """Load JSON template and initialize with defaults"""
    template_path = os.path.join(
        os.path.dirname(__file__),
        '../json/grifo_test_template.json'
    )
    
    with open(template_path, 'r') as f:
        template = json.load(f)
    
    # Initialize timestamp
    template['test_metadata']['timestamp'] = datetime.now().strftime(
        '%Y-%m-%d_%H-%M-%S'
    )
    
    return template

## 7.6 Best Practices Output

### 7.6.1 Checklist Generazione Report

✅ Always:
- Genera tutti e 3 i formati (PDF, CSV, JSON)
- Usa timestamp identico per tutti i file
- Salva in cartelle corrette (pdf_reports/, LOG/)
- Log success/failure con path completo

✅ Error Handling:

try:
    pdf_path = generate_pdf_report(output_folder, json_data)
    csv_path = export_to_csv(log_folder, json_data)
    json_path = save_json_output(output_folder, json_data)
    
    print(f"\n✅ Reports generated:")
    print(f"   PDF:  {pdf_path}")
    print(f"   CSV:  {csv_path}")
    print(f"   JSON: {json_path}")

except Exception as e:
    logging.error(f"Failed to generate reports: {e}")
    # Continue execution (reports are not critical for test validity)

### 7.6.2 Validazione Output

Check PDF:

import os

def validate_pdf_output(pdf_path: str) -> bool:
    """Verify PDF was created and is not empty"""
    if not os.path.exists(pdf_path):
        logging.error(f"PDF not found: {pdf_path}")
        return False
    
    size = os.path.getsize(pdf_path)
    if size < 1024:  # Less than 1KB
        logging.warning(f"PDF suspiciously small: {size} bytes")
        return False
    
    logging.info(f"PDF validated: {size} bytes")
    return True

Check CSV Row Count:

def validate_csv_output(csv_path: str, expected_runs: int) -> bool:
    """Verify CSV has expected number of rows"""
    import csv
    
    with open(csv_path, 'r') as f:
        reader = csv.reader(f)
        rows = sum(1 for _ in reader) - 1  # Exclude header
    
    if rows != expected_runs:
        logging.warning(f"CSV row count mismatch: {rows} != {expected_runs}")
        return False
    
    logging.info(f"CSV validated: {rows} runs")
    return True

← Interfaccia Grafica | Torna all’Indice | Prossimo: Configurazione →X

# — 08_Configurazione_Personalizzazione.md —

# Capitolo 8: Configurazione e Personalizzazione

← Output e Report | Torna all’Indice | Prossimo: Integrazione Hardware →X

|  |  | ## 8.1 Overview Configurazione |
| --- | --- | --- |

Il sistema è progettato per essere altamente configurabile senza modificare il codice core. La configurazione avviene tramite:

Meccanismo

File/Variabile

Scopo

CLI Arguments

--target, --simulate, --runs

Modalità esecuzione e parametri base

SCENARIOS Dict

GRIFO_M_PBIT.py (riga ~130-250)

Definizione scenari test e failure injection

KNOWN_FAILURES List

GRIFO_M_PBIT.py (riga ~115-125)

Failure attesi da ignorare

Constants

Top of modules

Timeout, retry, threshold

Mock Config

GRIFO_M_PBIT_mock.py

Comportamento simulazione

## 8.2 Parametri CLI

### 8.2.1 Sintassi Completa

python GRIFO_M_PBIT.py [--target | --simulate] [--runs N] [--gui] [--no-gui] [--fast-mode]

|  |  |  | ### 8.2.2 Parametri Disponibili |
| --- | --- | --- | --- |

Parametro

Tipo

Default

Descrizione

--target

Flag

False

Esegue su hardware reale (MIL-STD-1553)

--simulate

Flag

False

Esegue in simulation mode (mock objects)

--runs

Integer

10

Numero di cicli test da eseguire

--gui

Flag

True

Mostra GUI real-time

--no-gui

Flag

False

Disabilita GUI (headless)

--fast-mode

Flag

False

Salta verification delays (simulation only)

Nota: --target e --simulate sono mutually exclusive. Se nessuno dei due è specificato, default = --simulate.

### 8.2.3 Esempi Uso CLI

# Simulation standard (10 runs, GUI enabled)
python GRIFO_M_PBIT.py --simulate

# Simulation veloce (skip delays)
python GRIFO_M_PBIT.py --simulate --fast-mode

# Simulation 50 runs senza GUI (per CI/CD)
python GRIFO_M_PBIT.py --simulate --runs 50 --no-gui

# Hardware test (3 runs, GUI enabled)
python GRIFO_M_PBIT.py --target --runs 3

# Hardware test senza GUI (automation)
python GRIFO_M_PBIT.py --target --runs 1 --no-gui

### 8.2.4 Parsing Argomenti

def parse_arguments():
    """Parse CLI arguments for test configuration"""
    import argparse
    
    parser = argparse.ArgumentParser(
        description='GRIFO M-PBIT Automated Test',
        formatter_class=argparse.RawDescriptionHelpFormatter,
        epilog="""
Examples:
  python GRIFO_M_PBIT.py --simulate                # Simulation mode
  python GRIFO_M_PBIT.py --target --runs 5         # Hardware 5 runs
  python GRIFO_M_PBIT.py --simulate --no-gui       # Headless simulation
        """
    )
    
    # Execution mode (mutually exclusive)
    mode_group = parser.add_mutually_exclusive_group()
    mode_group.add_argument('--target', action='store_true',
                           help='Run on hardware (MIL-STD-1553)')
    mode_group.add_argument('--simulate', action='store_true',
                           help='Run in simulation mode (default)')
    
    # Test parameters
    parser.add_argument('--runs', type=int, default=10,
                       help='Number of test cycles (default: 10)')
    
    # GUI control
    gui_group = parser.add_mutually_exclusive_group()
    gui_group.add_argument('--gui', action='store_true', default=True,
                          help='Enable GUI (default)')
    gui_group.add_argument('--no-gui', action='store_true',
                          help='Disable GUI (headless mode)')
    
    # Performance
    parser.add_argument('--fast-mode', action='store_true',
                       help='Skip verification delays (simulation only)')
    
    args = parser.parse_args()
    
    # Set defaults
    if not args.target and not args.simulate:
        args.simulate = True  # Default to simulation
    
    if args.no_gui:
        args.gui = False
    
    return args

## 8.3 Sistema Scenari

### 8.3.1 Concetto Scenari

Gli scenari definiscono configurazioni di failure injection per testare comportamenti diversi:

Scenario 1: Baseline (tutto nominale)

Scenario 2-5: Failure specifici (processor, transmitter, antenna, ecc.)

Scenario 6: Random failures (30% probability)

Rotazione: Test cicla attraverso scenari in ordine sequenziale:

Run 1  → Scenario 1
Run 2  → Scenario 1
Run 3  → Scenario 2
Run 4  → Scenario 3
Run 5  → Scenario 4
Run 6  → Scenario 5
Run 7  → Scenario 6
Run 8  → Scenario 1  (ricomincia)
...

### 8.3.2 Definizione Scenari (SCENARIOS Dict)

# File: GRIFO_M_PBIT.py (~line 130)

SCENARIOS = {
    1: {
        'name': 'Scenario 1: All Systems Nominal',
        'description': 'Baseline test with all components functioning correctly',
        'b6_overrides': {},  # No failures injected
        'target_config': {
            'enabled': True,
            'distance': 2000,  # meters
            'appear_after_cycles': 3
        }
    },
    
    2: {
        'name': 'Scenario 2: Processor Failure',
        'description': 'Simulate processor subsystem failure',
        'b6_overrides': {
            'processor_status': 0  # Force fail
        },
        'target_config': {
            'enabled': False  # No target test if critical failure
        }
    },
    
    3: {
        'name': 'Scenario 3: Transmitter Failure',
        'description': 'Simulate transmitter offline',
        'b6_overrides': {
            'transmitter_status': 0
        },
        'target_config': {
            'enabled': False
        }
    },
    
    4: {
        'name': 'Scenario 4: Antenna Servo Failure',
        'description': 'Simulate antenna positioning failure',
        'b6_overrides': {
            'antenna_servo_status': 0
        },
        'target_config': {
            'enabled': True,  # Antenna failure doesn't prevent target test
            'distance': 1500,
            'appear_after_cycles': 5
        }
    },
    
    5: {
        'name': 'Scenario 5: Multiple Component Failures',
        'description': 'Combine processor + transmitter failures',
        'b6_overrides': {
            'processor_status': 0,
            'transmitter_status': 0,
            'cooling_status': 0
        },
        'target_config': {
            'enabled': False
        }
    },
    
    6: {
        'name': 'Scenario 6: Random Component Failures',
        'description': 'Random selection of failures (30% probability each)',
        'b6_overrides': 'random',  # Special keyword
        'random_failure_probability': 0.30,
        'target_config': {
            'enabled': True,
            'distance': 'random',  # 1000-3000m
            'appear_after_cycles': 'random'  # 2-8 cycles
        }
    }
}

### 8.3.3 Applicazione Scenario

def apply_scenario(run_number: int) -> Dict:
    """
    Select and apply scenario based on run number.
    
    Args:
        run_number: Current test run (1-based)
    
    Returns:
        Scenario configuration dictionary
    """
    import random
    
    # Rotate through scenarios
    num_scenarios = len(SCENARIOS)
    scenario_id = ((run_number - 1) % num_scenarios) + 1
    
    scenario = SCENARIOS[scenario_id].copy()
    
    # Handle random scenario
    if scenario.get('b6_overrides') == 'random':
        b6_fields = [
            'processor_status', 'transmitter_status', 'receiver_status',
            'antenna_servo_status', 'cooling_status', 'power_supply_status'
        ]
        
        overrides = {}
        prob = scenario.get('random_failure_probability', 0.3)
        
        for field in b6_fields:
            if random.random() < prob:
                overrides[field] = 0  # Inject failure
        
        scenario['b6_overrides'] = overrides
    
    # Handle random target config
    target_cfg = scenario.get('target_config', {})
    if target_cfg.get('distance') == 'random':
        target_cfg['distance'] = random.randint(1000, 3000)
    if target_cfg.get('appear_after_cycles') == 'random':
        target_cfg['appear_after_cycles'] = random.randint(2, 8)
    
    logging.info(f"Run {run_number}: {scenario['name']}")
    if scenario['b6_overrides']:
        logging.info(f"  B6 overrides: {scenario['b6_overrides']}")
    
    return scenario


def configure_mock_with_scenario(scenario: Dict):
    """
    Apply scenario configuration to mock objects.
    Only works in simulation mode.
    """
    if not hasattr(theGrifo1553, 'set_b6_overrides'):
        logging.warning("Not in simulation mode, scenario config ignored")
        return
    
    # Apply B6 overrides to mock
    b6_overrides = scenario.get('b6_overrides', {})
    theGrifo1553.set_b6_overrides(b6_overrides)
    
    # Apply target configuration
    target_cfg = scenario.get('target_config', {})
    if hasattr(theGrifo1553, 'configure_target'):
        theGrifo1553.configure_target(target_cfg)
    
    logging.debug(f"Mock configured with scenario: {scenario['name']}")

## 8.4 Known Failures List

### 8.4.1 Concetto Known Failures

Alcuni failure sono attesi e accettabili (es. ICD mismatch, known bugs). Il sistema li traccia separatamente e non causano FAIL del run.

Esempio: Campo bit_test_2_status sempre fallisce per bug noto nel firmware → aggiungi a KNOWN_FAILURES.

### 8.4.2 Definizione Known Failures

# File: GRIFO_M_PBIT.py (~line 115)

KNOWN_FAILURES = [
    'bit_test_2_status',  # Known ICD mismatch: always returns 0
    # Add more fields here as needed
]

### 8.4.3 Check con Known Failures

def check_b6_field(field_name: str, expected: int, actual: int) -> str:
    """
    Check B6 field with known failures handling.
    
    Returns:
        'PASS', 'FAIL', or 'KNOWN_FAIL'
    """
    if actual == expected:
        return 'PASS'
    
    # Check if in known failures list
    if field_name in KNOWN_FAILURES:
        logging.warning(f"B6 {field_name}: Known failure (expected={expected}, actual={actual})")
        return 'KNOWN_FAIL'
    
    # Unexpected failure
    logging.error(f"B6 {field_name}: FAIL (expected={expected}, actual={actual})")
    return 'FAIL'


def evaluate_run_result(b6_pass: int, b6_fail: int, b6_known: int,
                       b8_pass: int, b8_fail: int,
                       serial_fatal: int) -> str:
    """
    Determine final run result.
    Known failures do NOT cause FAIL.
    
    Returns:
        'PASS' or 'FAIL'
    """
    # Serial fatal errors always fail
    if serial_fatal > 0:
        return 'FAIL'
    
    # B6 unexpected failures
    if b6_fail > 0:
        return 'FAIL'
    
    # B8 failures
    if b8_fail > 0:
        return 'FAIL'
    
    # Known failures are OK
    return 'PASS'

### 8.4.4 Reporting Known Failures

Nel report PDF/CSV, i known failures sono tracciati separatamente:

PDF:

Run 1/10 - PASS
├─ B6 Status: 11/0/1 (P/F/K)
│  ├─ 11 PASS
│  ├─ 0 FAIL
│  └─ 1 KNOWN_FAIL: bit_test_2_status

CSV:

run_id,result,b6_pass,b6_fail,b6_known_fail
1,PASS,11,0,1

## 8.5 Costanti e Timeout

### 8.5.1 Costanti Principali

# File: GRIFO_M_PBIT.py (top of file)

# Test configuration
DEFAULT_NUM_RUNS = 10
MAX_RUNS = 100  # Safety limit

# Timeouts (seconds)
PBIT_TIMEOUT = 120.0  # Max time to wait for PBIT completion
B6_AVAILABLE_TIMEOUT = 10.0  # Max time for BIT data availability
B8_AVAILABLE_TIMEOUT = 10.0
TARGET_TEST_TIMEOUT = 300.0  # 5 minutes for target detection

# Power control
POWER_ON_DELAY = 1.0  # Delay after power ON
POWER_OFF_DELAY = 0.5  # Delay after power OFF

# Serial terminal
SERIAL_BUFFER_SIZE = 4096
SERIAL_TIMEOUT = 0.1  # Read timeout

# Target generation
TARGET_DEFAULT_DISTANCE = 2000  # meters
TARGET_MAX_CYCLES = 20  # Max iterations before timeout
TARGET_CYCLE_DURATION = 6.0  # seconds per radar scan cycle

# Verification delays (can be skipped with --fast-mode)
TELLBACK_VERIFICATION_DELAY = 0.5
STANDBY_VERIFICATION_DELAY = 0.5

### 8.5.2 Timeout Configuration Best Practices

Hardware Mode (real hardware, slow):

if args.target:
    PBIT_TIMEOUT = 120.0  # Hardware PBIT can take up to 2 minutes
    TARGET_TEST_TIMEOUT = 600.0  # Real radar needs more time

Simulation Mode (fast, deterministic):

if args.simulate:
    PBIT_TIMEOUT = 10.0  # Mock PBIT completes in ~1s
    TARGET_TEST_TIMEOUT = 30.0  # Mock target appears quickly

Fast Mode (skip delays):

if args.fast_mode:
    TELLBACK_VERIFICATION_DELAY = 0.0
    STANDBY_VERIFICATION_DELAY = 0.0
    POWER_ON_DELAY = 0.1

## 8.6 Configurazione Mock (Simulation)

### 8.6.1 Mock Configuration Constants

# File: GRIFO_M_PBIT_mock.py (top section)

# PBIT timing
MOCK_PBIT_DURATION_MIN = 0.5  # seconds
MOCK_PBIT_DURATION_MAX = 3.0
MOCK_PBIT_AVAILABLE_DELAY = 0.0  # BIT data immediately available

# B6 default values (all nominal)
MOCK_B6_DEFAULTS = {
    'processor_status': 1,
    'transmitter_status': 1,
    'receiver_status': 1,
    'antenna_servo_status': 1,
    'pbit_complete': 1,
    'bit_test_1_status': 1,
    'bit_test_2_status': 0,  # Known failure (always 0)
    'cooling_status': 1,
    'power_supply_status': 1,
    'memory_status': 1,
    'software_version': 123,
    'hardware_revision': 456
}

# B8 configuration
MOCK_B8_TOTAL_FIELDS = 185
MOCK_B8_FAILURE_PROBABILITY = 0.0  # 0% = all pass (can override per scenario)

# Serial terminal messages
MOCK_SERIAL_ERROR_PROBABILITY = 0.05  # 5% chance of %%E per message
MOCK_SERIAL_FATAL_PROBABILITY = 0.01  # 1% chance of %%F
MOCK_SERIAL_MESSAGE_RATE = 2  # Messages per second

# Target simulation
MOCK_TARGET_ALWAYS_DETECT = True  # If False, use probability
MOCK_TARGET_DETECTION_PROBABILITY = 0.9  # 90% success rate
MOCK_TARGET_MIN_CYCLES = 2
MOCK_TARGET_MAX_CYCLES = 8

### 8.6.2 Runtime Configuration (durante test)

class MockGrifo1553Interface:
    """Mock 1553 interface with runtime configuration"""
    
    def __init__(self):
        self.b6_values = MOCK_B6_DEFAULTS.copy()
        self.b6_overrides = {}
        self.target_config = {
            'enabled': True,
            'distance': 2000,
            'appear_after_cycles': 3
        }
    
    def set_b6_overrides(self, overrides: Dict[str, int]):
        """
        Override B6 field values (for scenario injection).
        
        Args:
            overrides: Dict of {field_name: value}
        """
        self.b6_overrides = overrides.copy()
        logging.debug(f"Mock B6 overrides set: {overrides}")
    
    def configure_target(self, config: Dict):
        """
        Configure target simulation behavior.
        
        Args:
            config: Dict with keys 'enabled', 'distance', 'appear_after_cycles'
        """
        self.target_config.update(config)
        logging.debug(f"Mock target config: {self.target_config}")
    
    def get(self, address: str) -> int:
        """Get B6 field value (with overrides applied)"""
        field_name = self._address_to_field_name(address)
        
        # Apply override if present
        if field_name in self.b6_overrides:
            return self.b6_overrides[field_name]
        
        # Return default
        return self.b6_values.get(field_name, 0)

## 8.7 Personalizzazione Avanzata

### 8.7.1 Aggiungere Nuovo Scenario

Step 1: Aggiungi a SCENARIOS dict:

SCENARIOS = {
    # ... scenari esistenti ...
    
    7: {
        'name': 'Scenario 7: Partial Receiver Failure',
        'description': 'Receiver degraded but operational',
        'b6_overrides': {
            'receiver_status': 0  # Force receiver fail
        },
        'b8_overrides': {
            # Specific B8 fields affected by receiver failure
            'receiver_sensitivity': 50,  # Degraded (expected 100)
            'signal_quality': 2  # Poor (expected 5)
        },
        'target_config': {
            'enabled': True,
            'distance': 1000,  # Shorter distance due to degraded receiver
            'appear_after_cycles': 10  # Takes longer to detect
        }
    }
}

Step 2: Se serve, estendi mock per supportare b8_overrides:

class MockGrifo1553Interface:
    def __init__(self):
        # ... existing ...
        self.b8_overrides = {}
    
    def set_b8_overrides(self, overrides: Dict[str, int]):
        """Override specific B8 field values"""
        self.b8_overrides = overrides.copy()

Step 3: Applica scenario nel loop principale:

scenario = apply_scenario(current_run)
configure_mock_with_scenario(scenario)

# Now test will use scenario configuration

### 8.7.2 Aggiungere Known Failure

Step 1: Identifica campo che fallisce in modo accettabile

# Esempio: nuovo campo B6 "secondary_power_status" ha bug noto

Step 2: Aggiungi a KNOWN_FAILURES:

KNOWN_FAILURES = [
    'bit_test_2_status',
    'secondary_power_status',  # New: always returns incorrect value
]

Step 3: Verifica report (dovrebbe mostrare “K” per Known):

B6 Status: 11/0/2 (P/F/K)
            ↑     ↑
            Pass  Known Failures

### 8.7.3 Modificare Timeout per Hardware Lento

Se hardware impiega >2 minuti per PBIT:

# File: GRIFO_M_PBIT.py

# OLD
PBIT_TIMEOUT = 120.0

# NEW
PBIT_TIMEOUT = 300.0  # 5 minutes for slow hardware

Oppure, rendilo configurabile via CLI:

parser.add_argument('--pbit-timeout', type=float, default=120.0,
                   help='PBIT completion timeout (seconds)')

args = parse_arguments()
PBIT_TIMEOUT = args.pbit_timeout

### 8.7.4 Custom Target Distances

Per testare detection a diverse range:

# Modifica SCENARIOS target_config
SCENARIOS[1]['target_config']['distance'] = 500   # Close range
SCENARIOS[2]['target_config']['distance'] = 3000  # Long range
SCENARIOS[3]['target_config']['distance'] = 1500  # Medium range

Oppure, aggiungi lista di distance da testare:

TARGET_TEST_DISTANCES = [500, 1000, 1500, 2000, 2500, 3000]

def get_target_distance_for_run(run_number: int) -> int:
    """Rotate through test distances"""
    idx = (run_number - 1) % len(TARGET_TEST_DISTANCES)
    return TARGET_TEST_DISTANCES[idx]

## 8.8 File di Configurazione Esterni (Opzionale)

### 8.8.1 YAML Configuration (Future Enhancement)

Per configurazione più flessibile, considera file YAML:

# grifo_test_config.yaml

test:
  default_runs: 10
  enable_gui: true
  fast_mode: false

timeouts:
  pbit: 120.0
  target: 300.0
  b6_available: 10.0

scenarios:
  - id: 1
    name: "All Systems Nominal"
    description: "Baseline test"
    b6_overrides: {}
    target:
      enabled: true
      distance: 2000
      cycles: 3
  
  - id: 2
    name: "Processor Failure"
    description: "Processor offline"
    b6_overrides:
      processor_status: 0
    target:
      enabled: false

known_failures:
  - bit_test_2_status
  - secondary_power_status

Loading YAML:

import yaml

def load_config(config_file: str = 'grifo_test_config.yaml') -> Dict:
    """Load test configuration from YAML file"""
    with open(config_file, 'r') as f:
        config = yaml.safe_load(f)
    return config

# Usage
config = load_config()
DEFAULT_NUM_RUNS = config['test']['default_runs']
PBIT_TIMEOUT = config['timeouts']['pbit']
SCENARIOS = {s['id']: s for s in config['scenarios']}
KNOWN_FAILURES = config['known_failures']

### 8.8.2 Environment Variables

Per deployment automation:

import os

# Override defaults from environment
DEFAULT_NUM_RUNS = int(os.getenv('GRIFO_TEST_RUNS', '10'))
PBIT_TIMEOUT = float(os.getenv('GRIFO_PBIT_TIMEOUT', '120.0'))
ENABLE_GUI = os.getenv('GRIFO_ENABLE_GUI', 'true').lower() == 'true'

Usage:

# Windows
set GRIFO_TEST_RUNS=50
set GRIFO_ENABLE_GUI=false
python GRIFO_M_PBIT.py --simulate

# Linux/Mac
export GRIFO_TEST_RUNS=50
export GRIFO_ENABLE_GUI=false
python GRIFO_M_PBIT.py --simulate

## 8.9 Checklist Personalizzazione

✅ Prima di modificare:
- [ ] Backup del file originale
- [ ] Identifica sezione esatta da modificare
- [ ] Verifica impatto su altri componenti

✅ Modifiche Comuni (safe):
- [ ] Aggiungi scenario a SCENARIOS dict
- [ ] Aggiungi campo a KNOWN_FAILURES list
- [ ] Modifica timeout constants
- [ ] Cambia DEFAULT_NUM_RUNS

✅ Modifiche Avanzate (test required):
- [ ] Nuovi parametri CLI
- [ ] Custom B8 check logic
- [ ] Nuovi campi B6/B8 (richiede ICD update)
- [ ] Modifiche a mock behavior

✅ Dopo modifiche:
- [ ] Test in simulation mode
- [ ] Verifica report generati correttamente
- [ ] Test con diversi scenari
- [ ] Documenta modifiche

← Output e Report | Torna all’Indice | Prossimo: Integrazione Hardware →X

# — 09_Integrazione_Hardware.md —

# Capitolo 9: Integrazione Hardware

← Configurazione | Torna all’Indice | Prossimo: Best Practices →X

|  |  |  | ## 9.1 Overview Integrazione Hardware |
| --- | --- | --- | --- |

Il test environment interagisce con tre tipi di hardware per controllo e monitoraggio GRIFO:

Hardware

Protocollo

Modulo

Scopo

1553 Bus

MIL-STD-1553

leo_grifo_1553.py

Command & data acquisition

Serial Terminal

RS-232 (UART)

leo_grifo_terminal.py

Status messages & diagnostics

Power Control

GPIO/Relay

leo_grifo_io_box.py

Power ON/OFF cycling

Architettura Hardware:

┌─────────────────────────────────────────────────────────────┐
│                   Test PC (Windows)                          │
│  ┌────────────────────────────────────────────────────┐     │
│  │  GRIFO_M_PBIT.py (Test Script)                    │     │
│  └───┬────────────────────┬────────────────────┬──────┘     │
│      │                    │                    │            │
│      │                    │                    │            │
│   ┌──▼──────────┐   ┌────▼─────────┐   ┌─────▼───────┐    │
│   │ 1553 Driver │   │Serial Driver │   │ IO Box API  │    │
│   │(SWIG/C++)  │   │ (pyserial)   │   │ (Ethernet)  │    │
│   └──┬──────────┘   └────┬─────────┘   └─────┬───────┘    │
└──────┼─────────────────┬─┼────────────────────┼────────────┘
       │                 │ │                    │
       │ MIL-STD-1553    │ │ RS-232             │ Ethernet
       │ (Bus interface) │ │ (Serial cable)     │ (TCP/IP)
       │                 │ │                    │
┌──────▼─────────────────┼─▼────────────────────▼────────────┐
│              GRIFO Radar System                             │
│  ┌─────────────────────────────────────────────────────┐   │
│  │  1553 RT (Remote Terminal)                          │   │
│  │  - Address: 0x1A                                    │   │
│  │  - Subaddresses: B6 (status), B8 (telemetry)       │   │
│  └─────────────────────────────────────────────────────┘   │
│                                                             │
│  ┌─────────────────────────────────────────────────────┐   │
│  │  Serial Debug Port                                  │   │
│  │  - Baud: 115200 / Parity: None / Stop: 1           │   │
│  │  - Protocol: ASCII messages (%%E, %%F, RECYCLE)    │   │
│  └─────────────────────────────────────────────────────┘   │
│                                                             │
│  ┌─────────────────────────────────────────────────────┐   │
│  │  Power Input                                        │   │
│  │  - Controlled by external relay (BrainBox)         │   │
│  └─────────────────────────────────────────────────────┘   │
└─────────────────────────────────────────────────────────────┘

┌─────────────────────────────────────────────────────────────┐
│              BrainBox (IO Controller)                        │
│  - Ethernet interface (192.168.1.100)                       │
│  - Relay outputs for power control                          │
│  - Digital I/O for status indicators                        │
└─────────────────────────────────────────────────────────────┘

## 9.2 MIL-STD-1553 Bus Interface

### 9.2.1 Cos’è MIL-STD-1553?

MIL-STD-1553 è un bus di comunicazione dual-redundant usato in sistemi militari/aerospaziali per:
- Command/Response: Bus Controller (BC) invia comandi, Remote Terminal (RT) risponde
- Deterministic: Timing predicibile, priorità definite
- Robust: Dual bus (A/B) per ridondanza

Topologia:

Bus Controller (BC)          ← Test PC con 1553 card
           |
    ┌──────┴──────┐
    │             │
  Bus A         Bus B    ← Redundant buses
    │             │
    └──────┬──────┘
           |
    Remote Terminal (RT)         ← GRIFO radar (address 0x1A)

### 9.2.2 SWIG Bindings (C++ → Python)

L’interfaccia 1553 è implementata in C++ e wrappata per Python via SWIG (Simplified Wrapper and Interface Generator).

Files Chiave:

PlatformSimulator/bin/
├── interpreter.py           # SWIG-generated Python module
├── _interpreter.pyd         # C++ compiled DLL (Windows)
└── [source C++ files]       # Native implementation

Python Interface:

# File: PlatformSimulator/bin/interpreter.py (SWIG-generated)

class Bus1553Interface:
    """
    Native 1553 bus interface (SWIG wrapper).
    DO NOT modify - auto-generated.
    """
    
    def __init__(self, bus_config: str):
        """Initialize 1553 bus controller"""
        pass
    
    def write_command(self, rt_address: int, subaddress: int, 
                     data: List[int]) -> bool:
        """
        Send command to RT.
        
        Args:
            rt_address: RT address (0-31)
            subaddress: Subaddress (0-31)
            data: List of 16-bit words
        
        Returns:
            True if ACK received
        """
        pass
    
    def read_response(self, rt_address: int, subaddress: int, 
                     word_count: int) -> List[int]:
        """
        Read data from RT.
        
        Args:
            rt_address: RT address (0-31)
            subaddress: Subaddress (0-31)
            word_count: Number of 16-bit words to read
        
        Returns:
            List of received words
        """
        pass
    
    def check_status(self, rt_address: int) -> int:
        """
        Check RT status word.
        
        Returns:
            Status word (16-bit)
            Bit meanings per MIL-STD-1553:
              Bit 0: Terminal Flag
              Bit 1: Dynamic Bus Control
              Bit 2: Subsystem Flag
              Bit 3: Busy
              Bit 4: Service Request
              Bit 5-7: Reserved
              Bit 8: Broadcast Command Received
              Bit 9: Message Error
              Bit 10: Instrumentation
              Bit 11-15: Reserved
        """
        pass

### 9.2.3 GrifoInstrumentInterface Wrapper

File: TestEnvironment/env/leo_grifo_1553.py

Scopo: Abstraction layer che nasconde complessità 1553 e fornisce API high-level.

class GrifoInstrumentInterface(TestCommonInterface):
    """
    High-level wrapper for GRIFO 1553 interface.
    Implements check/get/set pattern.
    """
    
    def __init__(self, bus_config: str = "default"):
        """
        Initialize 1553 interface.
        
        Args:
            bus_config: Bus configuration string (hardware-specific)
        """
        from PlatformSimulator.bin import interpreter
        
        self.bus = interpreter.Bus1553Interface(bus_config)
        self.rt_address = 0x1A  # GRIFO RT address
        
        # Field definitions (address mapping)
        self.b6_fields = {
            'processor_status': 0x00,
            'transmitter_status': 0x01,
            'receiver_status': 0x02,
            'antenna_servo_status': 0x03,
            'pbit_complete': 0x04,
            'bit_test_1_status': 0x05,
            'bit_test_2_status': 0x06,
            'cooling_status': 0x07,
            'power_supply_status': 0x08,
            'memory_status': 0x09,
            'software_version': 0x0A,
            'hardware_revision': 0x0B
        }
        
        self.b8_subaddress = 0x08  # B8 telemetry subaddress
        self.b8_word_count = 185   # B8 has 185 fields
    
    def check(self, field_name: str, expected_value: int, 
             timeout: float = 5.0) -> bool:
        """
        Check if field matches expected value.
        
        Args:
            field_name: B6 field name
            expected_value: Expected value
            timeout: Max time to wait (seconds)
        
        Returns:
            True if match, False otherwise
        """
        start = time.time()
        
        while time.time() - start < timeout:
            actual = self.get(field_name)
            if actual == expected_value:
                return True
            time.sleep(0.1)
        
        return False
    
    def get(self, field_name: str) -> int:
        """
        Read B6 field value.
        
        Args:
            field_name: B6 field name (e.g., 'processor_status')
        
        Returns:
            Field value (integer)
        
        Raises:
            ValueError: If field_name unknown
            RuntimeError: If 1553 communication fails
        """
        if field_name not in self.b6_fields:
            raise ValueError(f"Unknown B6 field: {field_name}")
        
        subaddress = 0x06  # B6 subaddress
        word_offset = self.b6_fields[field_name]
        
        try:
            # Read entire B6 block (more efficient than per-field)
            data = self.bus.read_response(self.rt_address, subaddress, 12)
            
            # Extract specific field
            value = data[word_offset]
            return value
        
        except Exception as e:
            raise RuntimeError(f"1553 read failed for {field_name}: {e}")
    
    def set(self, field_name: str, value: int) -> bool:
        """
        Write command to GRIFO (limited fields writable).
        
        Args:
            field_name: Command name (e.g., 'SILENCE', 'STANDBY')
            value: Command parameter
        
        Returns:
            True if ACK received
        """
        command_map = {
            'SILENCE': (0x10, 0x01),      # (subaddr, command_code)
            'STANDBY': (0x10, 0x02),
            'REMOVE_STANDBY': (0x10, 0x03),
            'SCAN_SECTOR_AZ': (0x11, None),  # value = azimuth
            'SCAN_SECTOR_EL': (0x12, None),  # value = elevation
        }
        
        if field_name not in command_map:
            raise ValueError(f"Unknown command: {field_name}")
        
        subaddr, cmd_code = command_map[field_name]
        
        # Build command message
        if cmd_code is not None:
            data = [cmd_code]
        else:
            data = [value]
        
        try:
            ack = self.bus.write_command(self.rt_address, subaddr, data)
            return ack
        
        except Exception as e:
            logging.error(f"1553 write failed for {field_name}: {e}")
            return False
    
    def read_b8_telemetry(self) -> Dict[str, int]:
        """
        Read all B8 telemetry fields.
        
        Returns:
            Dict mapping field names to values
        """
        try:
            data = self.bus.read_response(self.rt_address, 
                                         self.b8_subaddress, 
                                         self.b8_word_count)
            
            # Map to field names (simplified - real implementation uses ICD)
            b8_data = {}
            for i, value in enumerate(data):
                b8_data[f'b8_field_{i:03d}'] = value
            
            return b8_data
        
        except Exception as e:
            raise RuntimeError(f"B8 read failed: {e}")

### 9.2.4 Troubleshooting 1553

Problema: RuntimeError: 1553 read failed

Possibili Cause:
1. Hardware non connesso: Verifica cavo 1553 collegato correttamente
2. RT non powered: GRIFO deve essere acceso
3. Wrong RT address: Verifica rt_address = 0x1A
4. Bus conflict: Solo un BC deve essere attivo sul bus
5. Driver issue: Reinstalla driver 1553 card

Debug:

# Test 1553 connection
def test_1553_connection():
    """Verify 1553 bus is operational"""
    try:
        from PlatformSimulator.bin import interpreter
        bus = interpreter.Bus1553Interface("default")
        
        # Try to read RT status
        status = bus.check_status(0x1A)
        print(f"RT Status: 0x{status:04X}")
        
        if status & 0x0008:  # Bit 3 = Busy
            print("RT is BUSY")
        if status & 0x0200:  # Bit 9 = Message Error
            print("RT reports MESSAGE ERROR")
        
        return True
    
    except Exception as e:
        print(f"1553 connection FAILED: {e}")
        return False

## 9.3 Serial Terminal Interface

### 9.3.1 Protocol Serial GRIFO

Configurazione:
- Baud Rate: 115200
- Data Bits: 8
- Parity: None
- Stop Bits: 1
- Flow Control: None (hardware/software disabled)

Formato Messaggi:

Standard message: [TIMESTAMP] MESSAGE_TEXT\r\n
Error message:    [TIMESTAMP] %%E ERROR_DESCRIPTION\r\n
Fatal message:    [TIMESTAMP] %%F FATAL_ERROR\r\n
Recycle event:    [TIMESTAMP] RECYCLE: REASON\r\n

Esempi:

[00:02:15.324] System initialization complete
[00:02:18.891] %%E Receiver calibration drift detected
[00:02:19.003] %%F Processor watchdog timeout
[00:02:19.105] RECYCLE: Automatic recovery initiated
[00:02:22.456] Power-on self-test started
[00:02:25.789] BIT completed successfully

### 9.3.2 Implementazione Serial Interface

File: TestEnvironment/env/leo_grifo_terminal.py

import serial
import threading
import queue
import re
import logging

class GrifoSerialTerminal(TestCommonInterface):
    """
    Serial terminal interface for GRIFO diagnostics.
    Monitors ASCII messages and tracks errors/recycles.
    """
    
    def __init__(self, port: str = 'COM3', baudrate: int = 115200):
        """
        Initialize serial connection.
        
        Args:
            port: Serial port (e.g., 'COM3', '/dev/ttyUSB0')
            baudrate: Baud rate (default 115200)
        """
        self.port = port
        self.baudrate = baudrate
        self.serial = None
        self.running = False
        self.thread = None
        
        # Message buffer (thread-safe)
        self.message_queue = queue.Queue()
        
        # Statistics
        self.stats = {
            'total_messages': 0,
            'error_count': 0,      # %%E
            'fatal_count': 0,      # %%F
            'recycle_count': 0,    # RECYCLE
            'last_error': None,
            'last_fatal': None,
            'last_recycle': None
        }
        
        # Message patterns
        self.pattern_error = re.compile(r'%%E\s+(.+)')
        self.pattern_fatal = re.compile(r'%%F\s+(.+)')
        self.pattern_recycle = re.compile(r'RECYCLE:\s*(.+)')
    
    def connect(self) -> bool:
        """
        Open serial connection and start monitoring thread.
        
        Returns:
            True if connected successfully
        """
        try:
            self.serial = serial.Serial(
                port=self.port,
                baudrate=self.baudrate,
                bytesize=serial.EIGHTBITS,
                parity=serial.PARITY_NONE,
                stopbits=serial.STOPBITS_ONE,
                timeout=0.1,
                xonxoff=False,
                rtscts=False,
                dsrdtr=False
            )
            
            # Clear buffers
            self.serial.reset_input_buffer()
            self.serial.reset_output_buffer()
            
            # Start monitoring thread
            self.running = True
            self.thread = threading.Thread(target=self._monitor_loop, 
                                          daemon=True)
            self.thread.start()
            
            logging.info(f"Serial connected: {self.port} @ {self.baudrate}")
            return True
        
        except serial.SerialException as e:
            logging.error(f"Serial connection failed: {e}")
            return False
    
    def disconnect(self):
        """Stop monitoring and close serial port"""
        self.running = False
        if self.thread:
            self.thread.join(timeout=2.0)
        if self.serial and self.serial.is_open:
            self.serial.close()
        logging.info("Serial disconnected")
    
    def _monitor_loop(self):
        """Background thread: read and parse messages"""
        buffer = ""
        
        while self.running:
            try:
                if self.serial.in_waiting > 0:
                    chunk = self.serial.read(self.serial.in_waiting)
                    buffer += chunk.decode('ascii', errors='ignore')
                    
                    # Process complete lines
                    while '\n' in buffer:
                        line, buffer = buffer.split('\n', 1)
                        line = line.strip()
                        
                        if line:
                            self._process_message(line)
                
                else:
                    time.sleep(0.01)  # 10ms
            
            except Exception as e:
                logging.error(f"Serial monitor error: {e}")
                time.sleep(0.1)
    
    def _process_message(self, message: str):
        """Parse and categorize message"""
        self.stats['total_messages'] += 1
        
        # Check for error patterns
        if '%%E' in message:
            match = self.pattern_error.search(message)
            error_text = match.group(1) if match else message
            self.stats['error_count'] += 1
            self.stats['last_error'] = error_text
            logging.warning(f"Serial ERROR: {error_text}")
        
        elif '%%F' in message:
            match = self.pattern_fatal.search(message)
            fatal_text = match.group(1) if match else message
            self.stats['fatal_count'] += 1
            self.stats['last_fatal'] = fatal_text
            logging.error(f"Serial FATAL: {fatal_text}")
        
        elif 'RECYCLE' in message:
            match = self.pattern_recycle.search(message)
            recycle_reason = match.group(1) if match else message
            self.stats['recycle_count'] += 1
            self.stats['last_recycle'] = recycle_reason
            logging.warning(f"Serial RECYCLE: {recycle_reason}")
        
        else:
            # Normal message
            logging.debug(f"Serial: {message}")
        
        # Store in queue for external access
        self.message_queue.put({
            'timestamp': time.time(),
            'message': message,
            'type': self._classify_message(message)
        })
    
    def _classify_message(self, message: str) -> str:
        """Determine message type"""
        if '%%E' in message:
            return 'ERROR'
        elif '%%F' in message:
            return 'FATAL'
        elif 'RECYCLE' in message:
            return 'RECYCLE'
        else:
            return 'INFO'
    
    def get_statistics(self) -> Dict:
        """
        Get current statistics.
        
        Returns:
            Dict with counts and last messages
        """
        return self.stats.copy()
    
    def reset_statistics(self):
        """Clear statistics counters"""
        self.stats['error_count'] = 0
        self.stats['fatal_count'] = 0
        self.stats['recycle_count'] = 0
        self.stats['last_error'] = None
        self.stats['last_fatal'] = None
        self.stats['last_recycle'] = None
    
    def check(self, condition: str, expected: Any, 
             timeout: float = 5.0) -> bool:
        """
        Check for specific message or condition.
        
        Args:
            condition: 'no_fatal', 'no_error', 'no_recycle', or message pattern
            expected: Expected value or None
            timeout: Not used (returns current state)
        
        Returns:
            True if condition met
        """
        if condition == 'no_fatal':
            return self.stats['fatal_count'] == 0
        elif condition == 'no_error':
            return self.stats['error_count'] == 0
        elif condition == 'no_recycle':
            return self.stats['recycle_count'] == 0
        else:
            # Check for specific message in recent buffer
            # (simplified implementation)
            return True

### 9.3.3 Uso Serial Terminal nel Test

# Initialize
theTerminal = GrifoSerialTerminal(port='COM3')
theTerminal.connect()

# Reset stats before run
theTerminal.reset_statistics()

# ... execute test ...

# Check for issues
stats = theTerminal.get_statistics()
if stats['fatal_count'] > 0:
    logging.error(f"Run FAILED: {stats['fatal_count']} FATAL messages")
    run_result = 'FAIL'

if stats['recycle_count'] > 0:
    logging.warning(f"{stats['recycle_count']} RECYCLE events detected")

# Cleanup
theTerminal.disconnect()

### 9.3.4 Troubleshooting Serial

Problema: Serial connection failed

Possibili Cause:
1. Wrong port: Verifica COM port in Device Manager (Windows)
2. Port in use: Chiudi altri programmi (PuTTY, TeraTerm)
3. Permission denied: (Linux) aggiungi user a dialout group
4. Hardware disconnected: Verifica cavo USB/RS-232

Debug:

import serial.tools.list_ports

def list_serial_ports():
    """List available serial ports"""
    ports = serial.tools.list_ports.comports()
    print("Available serial ports:")
    for port in ports:
        print(f"  {port.device}: {port.description}")
    return [p.device for p in ports]

# Usage
available = list_serial_ports()
if 'COM3' in available:
    print("COM3 is available")

## 9.4 Power Control (BrainBox)

### 9.4.1 BrainBox Interface

Hardware: BrainBox IO controller (Ethernet-based relay controller)

File: TestEnvironment/env/leo_grifo_io_box.py

import socket
import logging

class BrainBoxInterface:
    """
    Interface to BrainBox IO controller for power control.
    Controls relay outputs via Ethernet (TCP/IP).
    """
    
    def __init__(self, ip: str = '192.168.1.100', port: int = 5000):
        """
        Initialize BrainBox connection.
        
        Args:
            ip: BrainBox IP address
            port: TCP port (default 5000)
        """
        self.ip = ip
        self.port = port
        self.socket = None
        
        # Relay mapping
        self.relays = {
            'GRIFO_POWER': 1,      # Relay 1 = GRIFO main power
            'COOLING_FAN': 2,      # Relay 2 = Cooling system
            'INDICATOR_LED': 3     # Relay 3 = Status LED
        }
    
    def connect(self) -> bool:
        """Establish TCP connection to BrainBox"""
        try:
            self.socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
            self.socket.settimeout(5.0)
            self.socket.connect((self.ip, self.port))
            logging.info(f"BrainBox connected: {self.ip}:{self.port}")
            return True
        
        except socket.error as e:
            logging.error(f"BrainBox connection failed: {e}")
            return False
    
    def disconnect(self):
        """Close connection"""
        if self.socket:
            self.socket.close()
            self.socket = None
    
    def set_relay(self, relay_id: int, state: bool) -> bool:
        """
        Set relay state.
        
        Args:
            relay_id: Relay number (1-8)
            state: True=ON, False=OFF
        
        Returns:
            True if successful
        """
        try:
            # BrainBox protocol: "REL <id> <state>\r\n"
            cmd = f"REL {relay_id} {'ON' if state else 'OFF'}\r\n"
            self.socket.sendall(cmd.encode('ascii'))
            
            # Wait for ACK
            response = self.socket.recv(64).decode('ascii').strip()
            if response == 'OK':
                logging.debug(f"Relay {relay_id} set to {state}")
                return True
            else:
                logging.error(f"Relay command failed: {response}")
                return False
        
        except Exception as e:
            logging.error(f"Relay control error: {e}")
            return False
    
    def power_grifo_on(self) -> bool:
        """Turn GRIFO power ON"""
        relay = self.relays['GRIFO_POWER']
        success = self.set_relay(relay, True)
        if success:
            logging.info("GRIFO power ON")
            time.sleep(1.0)  # Power stabilization delay
        return success
    
    def power_grifo_off(self) -> bool:
        """Turn GRIFO power OFF"""
        relay = self.relays['GRIFO_POWER']
        success = self.set_relay(relay, False)
        if success:
            logging.info("GRIFO power OFF")
            time.sleep(0.5)
        return success


# Global singleton
theBrainBox = BrainBoxInterface()

### 9.4.2 Power Cycling Logic

def power_cycle_grifo():
    """
    Complete power cycle: OFF → wait → ON → wait for boot
    """
    logging.info("Starting power cycle...")
    
    # Turn OFF
    if not theBrainBox.power_grifo_off():
        raise RuntimeError("Failed to power OFF GRIFO")
    
    # Wait for capacitors to discharge
    time.sleep(2.0)
    
    # Turn ON
    if not theBrainBox.power_grifo_on():
        raise RuntimeError("Failed to power ON GRIFO")
    
    # Wait for boot (GRIFO takes ~5s to boot)
    logging.info("Waiting for GRIFO boot...")
    time.sleep(5.0)
    
    # Verify 1553 is responsive
    max_retries = 10
    for attempt in range(max_retries):
        try:
            status = theGrifo1553.bus.check_status(0x1A)
            if status != 0xFFFF:  # 0xFFFF = no response
                logging.info("GRIFO boot complete, 1553 responsive")
                return True
        except Exception:
            pass
        
        time.sleep(1.0)
    
    raise RuntimeError("GRIFO failed to boot after power cycle")

## 9.5 Hardware Setup Checklist

### 9.5.1 Physical Connections

✅ Before Test:
- [ ] 1553 Bus: Cable collegato a PC 1553 card e GRIFO RT
- [ ] Serial: USB/RS-232 adapter collegato a GRIFO debug port
- [ ] Power: BrainBox relay connesso a GRIFO power input
- [ ] Network: BrainBox su stessa subnet del test PC (192.168.1.x)
- [ ] Safety: Emergency stop button accessibile

### 9.5.2 Software Prerequisites

✅ Drivers:
- [ ] 1553 card driver installato (verifica in Device Manager)
- [ ] Serial driver (CH340, FTDI, o built-in)
- [ ] Python pyserial package: pip install pyserial

✅ Configuration:
- [ ] Verifica COM port in leo_grifo_terminal.py
- [ ] Verifica BrainBox IP in leo_grifo_io_box.py
- [ ] Verifica RT address (0x1A) in leo_grifo_1553.py

### 9.5.3 Pre-Test Verification

def verify_hardware_connectivity():
    """Run before starting test to ensure all hardware is ready"""
    
    checks_passed = 0
    checks_total = 3
    
    # Check 1: 1553 Bus
    print("1. Testing 1553 connection...")
    try:
        from TestEnvironment.env.leo_grifo_1553 import theGrifo1553
        status = theGrifo1553.bus.check_status(0x1A)
        print(f"   ✓ 1553 OK (RT status: 0x{status:04X})")
        checks_passed += 1
    except Exception as e:
        print(f"   ✗ 1553 FAILED: {e}")
    
    # Check 2: Serial Terminal
    print("2. Testing serial connection...")
    try:
        from TestEnvironment.env.leo_grifo_terminal import theTerminal
        if theTerminal.connect():
            print(f"   ✓ Serial OK ({theTerminal.port})")
            theTerminal.disconnect()
            checks_passed += 1
        else:
            print("   ✗ Serial FAILED")
    except Exception as e:
        print(f"   ✗ Serial FAILED: {e}")
    
    # Check 3: BrainBox
    print("3. Testing BrainBox connection...")
    try:
        from TestEnvironment.env.leo_grifo_io_box import theBrainBox
        if theBrainBox.connect():
            print(f"   ✓ BrainBox OK ({theBrainBox.ip})")
            theBrainBox.disconnect()
            checks_passed += 1
        else:
            print("   ✗ BrainBox FAILED")
    except Exception as e:
        print(f"   ✗ BrainBox FAILED: {e}")
    
    # Summary
    print(f"\nHardware Check: {checks_passed}/{checks_total} passed")
    
    if checks_passed == checks_total:
        print("✓ All hardware ready for test")
        return True
    else:
        print("✗ Hardware issues detected - resolve before testing")
        return False

## 9.6 Safety Considerations

### 9.6.1 Hardware Safety

⚠️ WARNINGS:
- High Voltage: GRIFO operates at potentially dangerous voltages
- Power Cycling: Allow 2+ seconds between OFF and ON
- Emergency Stop: Ensure accessible kill switch
- Cooling: Verify cooling fan operational before extended tests

### 9.6.2 Software Safety Limits

# File: GRIFO_M_PBIT.py

# Safety limits
MAX_CONSECUTIVE_FAILURES = 5  # Abort if 5 failures in a row
MAX_POWER_CYCLES_PER_HOUR = 20  # Prevent excessive cycling
MIN_TIME_BETWEEN_CYCLES = 10.0  # Seconds

def check_safety_limits(consecutive_failures: int,
                       power_cycles_last_hour: int) -> bool:
    """
    Verify test is within safety limits.
    
    Returns:
        True if safe to continue, False to abort
    """
    if consecutive_failures >= MAX_CONSECUTIVE_FAILURES:
        logging.error(f"ABORT: {consecutive_failures} consecutive failures")
        return False
    
    if power_cycles_last_hour >= MAX_POWER_CYCLES_PER_HOUR:
        logging.error(f"ABORT: Too many power cycles ({power_cycles_last_hour}/hour)")
        return False
    
    return True

← Configurazione | Torna all’Indice | Prossimo: Best Practices →X

# — 10_Best_Practices.md —

# Capitolo 10: Best Practices e Troubleshooting

← Integrazione Hardware | Torna all’IndiceX

## 10.1 Best Practices Generali

### 10.1.1 Workflow Raccomandato

Prima Esecuzione (New User):

1. Leggere questo documento (almeno Capitoli 1-5)
2. Verificare prerequisites (Python, packages)
3. Eseguire simulation mode (safe, no hardware)
   → python GRIFO_M_PBIT.py --simulate --runs 3
4. Analizzare report generati (PDF, CSV, JSON)
5. Se OK, provare hardware mode (se disponibile)
   → python GRIFO_M_PBIT.py --target --runs 1

Esecuzione Regolare:

1. Verificare hardware connections (se --target)
2. Eseguire test con numero runs appropriato
3. Monitorare GUI real-time per anomalie
4. Analizzare report post-test
5. Archiviare report per trend analysis

Sviluppo/Debug:

1. Usare --simulate per rapid iteration
2. Usare --fast-mode per skip delays
3. Usare --no-gui per headless (CI/CD)
4. Analizzare log file per detailed diagnostics
5. Test incrementali su hardware reale

### 10.1.2 Configurazione Ambiente

✅ Python Environment:

# Usa virtual environment (raccomandato)
python -m venv venv
venv\Scripts\activate  # Windows
source venv/bin/activate  # Linux/Mac

# Installa dependencies
pip install reportlab pyserial pillow

✅ File Organization:

TestEnvironment/
├── LOG/              # CSV files (git ignore)
├── pdf_reports/      # PDF files (git ignore)
├── json/             # Template (commit to git)
├── env/              # Source code (commit)
└── scripts/          # Test scripts (commit)

✅ Git Configuration (.gitignore):

# Test outputs (large files, regenerable)
TestEnvironment/LOG/*.csv
TestEnvironment/pdf_reports/*.pdf
*.json  # Except templates

# Python
__pycache__/
*.pyc
*.pyo
*.pyd
venv/

# IDE
.vscode/
.idea/
*.swp

# Logs
logs/
*.log

### 10.1.3 Logging Best Practices

Configure Logging (all’inizio script):

import logging
import sys
from datetime import datetime

def setup_logging(log_folder: str = 'logs', level: str = 'INFO'):
    """
    Configure logging to file and console.
    
    Args:
        log_folder: Directory for log files
        level: Logging level ('DEBUG', 'INFO', 'WARNING', 'ERROR')
    """
    # Create log folder
    os.makedirs(log_folder, exist_ok=True)
    
    # Log filename with timestamp
    timestamp = datetime.now().strftime('%Y-%m-%d_%H-%M-%S')
    log_file = os.path.join(log_folder, f'GRIFO_M_PBIT_{timestamp}.log')
    
    # Configure root logger
    logging.basicConfig(
        level=getattr(logging, level.upper()),
        format='[%(asctime)s] %(levelname)-8s %(name)s: %(message)s',
        datefmt='%Y-%m-%d %H:%M:%S',
        handlers=[
            logging.FileHandler(log_file, encoding='utf-8'),
            logging.StreamHandler(sys.stdout)
        ]
    )
    
    logging.info(f"Logging initialized: {log_file}")
    return log_file

# Usage
log_file = setup_logging(log_folder='TestEnvironment/LOG', level='INFO')

Logging Levels (quando usarli):

# DEBUG: Detailed diagnostic (verbose)
logging.debug(f"B6 field read: {field_name}={value}")

# INFO: Normal operation progress
logging.info(f"Starting run {run_id}/{total_runs}")

# WARNING: Unexpected but non-critical
logging.warning(f"B6 field {field_name}: Known failure (expected)")

# ERROR: Failure that affects test result
logging.error(f"B6 Check Failed: {field_name} [expected={exp}, actual={act}]")

# CRITICAL: Test cannot continue
logging.critical(f"Hardware connection lost - aborting test")

## 10.2 Troubleshooting Common Issues

### 10.2.1 Import Errors

Problema: ModuleNotFoundError: No module named 'interpreter'

Causa: Python non trova SWIG module in PlatformSimulator/bin/

Soluzione:

# Verifica sys.path
import sys
print(sys.path)

# Aggiungi manualmente se necessario
sys.path.insert(0, r'C:\src\GRIFO\PlatformSimulator\bin')

# Oppure usa path bootstrap (TestEnvironment/scripts/__init__.py)
# Questo è già implementato nel wrapper

Verifica:

try:
    from PlatformSimulator.bin import interpreter
    print("✓ Interpreter module loaded")
except ImportError as e:
    print(f"✗ Import failed: {e}")

Problema: ImportError: DLL load failed

Causa: _interpreter.pyd (C++ DLL) ha dipendenze mancanti

Soluzione:
1. Verifica Visual C++ Redistributable installato
2. Usa Dependency Walker per identificare missing DLLs
3. Copia DLLs mancanti in PlatformSimulator/bin/

### 10.2.2 Hardware Connection Issues

Problema: RuntimeError: 1553 read failed

Diagnosi:

# Test 1: Driver loaded?
try:
    from PlatformSimulator.bin import interpreter
    print("✓ Driver module loaded")
except:
    print("✗ Driver NOT loaded")

# Test 2: Hardware responsive?
try:
    bus = interpreter.Bus1553Interface("default")
    status = bus.check_status(0x1A)
    print(f"✓ RT responsive: 0x{status:04X}")
except:
    print("✗ RT NOT responsive")

Soluzioni:
1. Verify power: GRIFO acceso?
2. Check cables: 1553 bus connesso correttamente?
3. RT address: Verifica rt_address = 0x1A è corretto
4. Driver reset: Riavvia PC / reinstalla driver

Problema: Serial connection failed: [Errno 5] Access denied

Causa: COM port già in uso o permission denied

Soluzione Windows:

# Verifica porta in Device Manager
Get-PnpDevice | Where-Object {$_.Class -eq "Ports"}

# Chiudi programmi che usano porta
taskkill /F /IM putty.exe
taskkill /F /IM TeraTerm.exe

Soluzione Linux:

# Aggiungi user a dialout group
sudo usermod -a -G dialout $USER
# Logout/login required

# Verifica permission
ls -l /dev/ttyUSB0

Problema: BrainBox connection failed: [Errno 10061] Connection refused

Causa: BrainBox non raggiungibile o IP errato

Diagnosi:

import socket

def test_brainbox_connection(ip='192.168.1.100', port=5000):
    """Test TCP connection to BrainBox"""
    try:
        sock = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
        sock.settimeout(2.0)
        sock.connect((ip, port))
        sock.close()
        print(f"✓ BrainBox reachable at {ip}:{port}")
        return True
    except socket.timeout:
        print(f"✗ BrainBox timeout (check network cable)")
        return False
    except socket.error as e:
        print(f"✗ BrainBox connection failed: {e}")
        return False

Soluzioni:
1. Ping test: ping 192.168.1.100
2. Network config: Verifica PC e BrainBox su stessa subnet
3. Firewall: Disabilita temporaneamente per test
4. Power cycle: Riavvia BrainBox

### 10.2.3 Test Execution Issues

Problema: Test bloccato su “Waiting for PBIT completion…”

Causa: PBIT non completa entro timeout

Diagnosi:

# Check PBIT status manually
pbit_complete = theGrifo1553.get('pbit_complete')
print(f"PBIT Complete flag: {pbit_complete}")

# Check serial for errors
stats = theTerminal.get_statistics()
print(f"Serial fatal count: {stats['fatal_count']}")

Soluzioni:
1. Increase timeout: PBIT_TIMEOUT = 300.0 (5 min)
2. Check serial: Fatal errors prevent PBIT completion
3. Power cycle: Reset GRIFO hardware
4. Hardware issue: PBIT genuinely failing (check diagnostics)

Problema: Tutti i run falliscono con B6 failures

Causa: B6 fields hanno valori inattesi

Diagnosi:

# Read all B6 fields
b6_fields = [
    'processor_status', 'transmitter_status', 'receiver_status',
    'antenna_servo_status', 'pbit_complete', 'cooling_status'
]

print("B6 Field Values:")
for field in b6_fields:
    value = theGrifo1553.get(field)
    print(f"  {field}: {value}")

Soluzioni:
1. Known failures: Aggiungi a KNOWN_FAILURES list se atteso
2. ICD mismatch: Verifica expected values vs ICD documentation
3. Hardware fault: Genuine component failure (require maintenance)

Problema: GUI non si apre / freezes

Causa: Thread deadlock o Tkinter issue

Soluzione:

# Run without GUI
python GRIFO_M_PBIT.py --simulate --no-gui

# Check for exceptions in GUI thread
# Look for errors in log file mentioning "GUI" or "Tkinter"

Alternative:
- Update Tkinter: Reinstall Python con latest version
- Run headless: --no-gui flag per CI/CD
- Check display: (Linux) set DISPLAY environment variable

### 10.2.4 Report Generation Issues

Problema: ImportError: No module named 'reportlab'

Soluzione:

pip install reportlab pillow

Problema: PDF vuoto o corrotto

Diagnosi:

import os

pdf_path = 'TestEnvironment/pdf_reports/GRIFO_M_PBIT_2026-02-05_15-24-38.pdf'

# Check file exists
if not os.path.exists(pdf_path):
    print("✗ PDF file not found")
else:
    size = os.path.getsize(pdf_path)
    print(f"PDF size: {size} bytes")
    
    if size < 1024:
        print("✗ PDF suspiciously small (likely corrupt)")
    else:
        print("✓ PDF size looks OK")

Soluzioni:
1. Check JSON data: Verifica json_data dict non è vuoto
2. ReportLab version: Update pip install --upgrade reportlab
3. Write permissions: Verifica write access a pdf_reports/
4. Disk space: Verifica spazio disponibile su disco

## 10.3 Performance Optimization

### 10.3.1 Fast Mode (Simulation)

Usa --fast-mode per:
- Development rapid iteration
- CI/CD automated testing
- Smoke tests

Effect:

if args.fast_mode:
    TELLBACK_VERIFICATION_DELAY = 0.0
    STANDBY_VERIFICATION_DELAY = 0.0
    POWER_ON_DELAY = 0.1
    
    # Skip unnecessary waits
    # Test completes in ~50% less time

Benchmark:

Normal Mode:  10 runs in ~8.5 minutes (51s/run)
Fast Mode:    10 runs in ~4.2 minutes (25s/run)
Speedup:      ~2x faster

### 10.3.2 Parallel Execution (Advanced)

Per testing massivo, considera parallel runs:

import multiprocessing

def run_test_instance(instance_id: int, num_runs: int):
    """Run test instance (separate process)"""
    import sys
    sys.argv = ['GRIFO_M_PBIT.py', '--simulate', '--no-gui', 
                f'--runs={num_runs}']
    
    # Import and run
    from TestEnvironment.scripts import GRIFO_M_PBIT
    # ... execute test ...

if __name__ == '__main__':
    # Run 4 instances in parallel (4 cores)
    num_instances = 4
    runs_per_instance = 25
    
    with multiprocessing.Pool(num_instances) as pool:
        pool.starmap(run_test_instance, 
                    [(i, runs_per_instance) for i in range(num_instances)])
    
    # Total: 4 * 25 = 100 runs
    # Time: ~same as 25 runs (parallelized)

Note: Solo per simulation mode (hardware mode richiede serialization)

### 10.3.3 Batch Processing

Per long-duration testing:

# Windows batch file: run_overnight.bat
@echo off
for /L %%i in (1,1,10) do (
    echo Running batch %%i of 10...
    python GRIFO_M_PBIT.py --simulate --runs 50 --no-gui
    timeout /t 60 /nobreak
)
echo All batches complete.

# Linux shell script: run_overnight.sh
#!/bin/bash
for i in {1..10}; do
    echo "Running batch $i of 10..."
    python GRIFO_M_PBIT.py --simulate --runs 50 --no-gui
    sleep 60
done
echo "All batches complete."

## 10.4 Debugging Techniques

### 10.4.1 Enable Debug Logging

# Change logging level to DEBUG
setup_logging(level='DEBUG')

# OR via environment variable
import os
os.environ['GRIFO_LOG_LEVEL'] = 'DEBUG'

Output:

[2026-02-05 15:24:38] DEBUG    leo_grifo_1553: B6 field read: processor_status=1
[2026-02-05 15:24:38] DEBUG    leo_grifo_1553: B6 field read: transmitter_status=1
[2026-02-05 15:24:38] INFO     GRIFO_M_PBIT: B6 checks: 11 PASS, 0 FAIL, 1 KNOWN

### 10.4.2 Interactive Debugging

Python Debugger (pdb):

import pdb

# Set breakpoint
pdb.set_trace()

# Commands:
# n - next line
# s - step into
# c - continue
# p <var> - print variable
# l - list code
# q - quit

VS Code Debugging:

// .vscode/launch.json
{
    "version": "0.2.0",
    "configurations": [
        {
            "name": "GRIFO Simulate",
            "type": "python",
            "request": "launch",
            "program": "${workspaceFolder}/TestEnvironment/scripts/GRIFO_M_PBIT.py",
            "args": ["--simulate", "--runs", "3"],
            "console": "integratedTerminal",
            "cwd": "${workspaceFolder}"
        }
    ]
}

### 10.4.3 Mock Object Inspection

Durante simulation, ispeziona mock state:

# Access mock object
mock_1553 = theGrifo1553  # Already MockGrifo1553Interface

# Inspect overrides
print(f"B6 overrides: {mock_1553.b6_overrides}")
print(f"Target config: {mock_1553.target_config}")

# Manually set field for testing
mock_1553.b6_values['processor_status'] = 0  # Force failure

### 10.4.4 Post-Mortem Analysis

Analizza test falliti:

import json

# Load JSON output
with open('TestEnvironment/LOG/GRIFO_M_PBIT_2026-02-05_15-24-38.json') as f:
    data = json.load(f)

# Find failed runs
failed_runs = [r for r in data['runs'] if r['result'] == 'FAIL']

print(f"Failed runs: {len(failed_runs)}")
for r in failed_runs:
    print(f"\nRun {r['run_id']}:")
    for failure in r.get('failures', []):
        print(f"  - {failure}")

## 10.5 Manutenzione e Estensione

### 10.5.1 Aggiornare ICD (Interface Control Document)

Quando: Nuovo firmware GRIFO cambia B6/B8 fields

Step:
1. Update field definitions in leo_grifo_1553.py:

self.b6_fields = {
    'processor_status': 0x00,
    # ... existing fields ...
    'new_field_name': 0x0C,  # New field at offset 12
}

Update expected values in test script:

def check_b6_status():
    # ... existing checks ...
    
    # New field check
    result = check_field('new_field_name', expected=1)
    if not result:
        failures.append('new_field_name')

Update mock in GRIFO_M_PBIT_mock.py:

MOCK_B6_DEFAULTS = {
    # ... existing ...
    'new_field_name': 1  # Default value
}

Test: Run simulation to verify new field tracked correctly

### 10.5.2 Aggiungere Nuovi Test Phases

Scenario: Aggiungere test di calibrazione dopo PBIT

Implementation:

def test_calibration_phase():
    """New test phase: calibration verification"""
    logging.info("Starting calibration phase...")
    
    # 1. Set calibration mode
    theGrifo1553.set('CALIBRATION_MODE', 1)
    time.sleep(2.0)
    
    # 2. Check calibration status
    cal_status = theGrifo1553.get('calibration_status')
    if cal_status != 1:
        logging.error("Calibration failed")
        return False
    
    # 3. Read calibration data
    cal_data = theGrifo1553.read_calibration_data()
    
    logging.info("Calibration phase complete")
    return True

# Add to main test loop
def main_test():
    for run in range(1, num_runs+1):
        power_cycle_grifo()
        
        pbit_ok = test_pbit_phase()
        if not pbit_ok:
            continue
        
        b6_ok = test_b6_phase()
        b8_ok = test_b8_phase()
        
        # NEW: Calibration phase
        cal_ok = test_calibration_phase()
        
        serial_ok = test_serial_phase()
        target_ok = test_target_phase()
        
        # Evaluate result (include cal_ok)
        # ...

### 10.5.3 Custom Report Templates

Personalizzare PDF Layout:

# File: TestEnvironment/env/leo_grifo_test_report.py

def generate_pdf_report_custom(output_folder: str, json_data: Dict) -> str:
    """Custom PDF report with company branding"""
    
    # ... standard setup ...
    
    # Add company logo
    from reportlab.lib.utils import ImageReader
    logo = ImageReader('company_logo.png')
    story.append(Image(logo, width=2*inch, height=1*inch))
    
    # Add custom header
    custom_header = Paragraph(
        "GRIFO M-PBIT Test Report - Confidential",
        custom_style
    )
    story.append(custom_header)
    
    # ... rest of report ...

## 10.6 CI/CD Integration

### 10.6.1 GitHub Actions Example

# .github/workflows/grifo_test.yml
name: GRIFO Automated Test

on:
  schedule:
    - cron: '0 2 * * *'  # Daily at 2 AM
  workflow_dispatch:     # Manual trigger

jobs:
  test:
    runs-on: windows-latest
    
    steps:
      - name: Checkout code
        uses: actions/checkout@v3
      
      - name: Setup Python
        uses: actions/setup-python@v4
        with:
          python-version: '3.10'
      
      - name: Install dependencies
        run: |
          pip install reportlab pyserial pillow
      
      - name: Run GRIFO test (simulation)
        run: |
          python TestEnvironment/scripts/GRIFO_M_PBIT.py --simulate --runs 20 --no-gui
      
      - name: Upload reports
        uses: actions/upload-artifact@v3
        with:
          name: test-reports
          path: |
            TestEnvironment/pdf_reports/*.pdf
            TestEnvironment/LOG/*.csv
            TestEnvironment/LOG/*.json
      
      - name: Check test result
        run: |
          # Parse JSON to check pass rate
          python -c "
          import json, glob
          jsons = glob.glob('TestEnvironment/LOG/*.json')
          with open(jsons[-1]) as f:
              data = json.load(f)
          pass_rate = data['test_metadata']['runs_passed'] / data['test_metadata']['total_runs']
          print(f'Pass rate: {pass_rate:.1%}')
          if pass_rate < 0.8:
              exit(1)  # Fail if <80% pass rate
          "

### 10.6.2 Jenkins Pipeline Example

// Jenkinsfile
pipeline {
    agent { label 'windows' }
    
    parameters {
        choice(name: 'MODE', choices: ['simulate', 'target'], description: 'Test mode')
        string(name: 'NUM_RUNS', defaultValue: '10', description: 'Number of runs')
    }
    
    stages {
        stage('Setup') {
            steps {
                bat 'pip install -r requirements.txt'
            }
        }
        
        stage('Run Test') {
            steps {
                script {
                    def mode_flag = params.MODE == 'target' ? '--target' : '--simulate'
                    bat "python TestEnvironment/scripts/GRIFO_M_PBIT.py ${mode_flag} --runs ${params.NUM_RUNS} --no-gui"
                }
            }
        }
        
        stage('Archive Reports') {
            steps {
                archiveArtifacts artifacts: 'TestEnvironment/pdf_reports/*.pdf, TestEnvironment/LOG/*.csv', fingerprint: true
            }
        }
        
        stage('Analyze Results') {
            steps {
                script {
                    // Parse JSON and generate trend report
                    bat 'python scripts/analyze_test_trends.py'
                }
            }
        }
    }
    
    post {
        failure {
            emailext(
                subject: "GRIFO Test Failed - Build ${env.BUILD_NUMBER}",
                body: "Test execution failed. Check console output.",
                to: 'test-team@company.com'
            )
        }
    }
}

## 10.7 FAQ

### Q1: Posso eseguire test su più GRIFO contemporaneamente?

A: Sì, ma richiede modifiche:
1. Ogni GRIFO deve avere RT address unico (0x1A, 0x1B, ecc.)
2. Ogni istanza test deve usare porta seriale separata
3. BrainBox deve controllare relay separati per ciascun GRIFO
4. Usa process isolation (multiprocessing) per evitare conflitti

### Q2: Come integro con sistema di tracking bug?

A: Aggiungi logging structured:

def log_failure_to_jira(failure: Dict):
    """Create JIRA ticket for test failure"""
    import requests
    
    jira_url = "https://jira.company.com/rest/api/2/issue"
    auth = ("user", "token")
    
    issue = {
        "fields": {
            "project": {"key": "GRIFO"},
            "summary": f"Test Failure: Run {failure['run_id']}",
            "description": "\n".join(failure['failures']),
            "issuetype": {"name": "Bug"},
            "labels": ["automated-test", "grifo-m-pbit"]
        }
    }
    
    response = requests.post(jira_url, json=issue, auth=auth)
    if response.status_code == 201:
        logging.info(f"JIRA ticket created: {response.json()['key']}")

### Q3: Come archivio report per compliance?

A: Usa versioning e database:

import shutil
import sqlite3
from datetime import datetime

def archive_report(pdf_path: str, csv_path: str, json_path: str):
    """Archive reports to compliance database"""
    
    # Create archive folder with date
    archive_root = r'\\network\share\GRIFO_Reports'
    date_folder = datetime.now().strftime('%Y-%m')
    archive_path = os.path.join(archive_root, date_folder)
    os.makedirs(archive_path, exist_ok=True)
    
    # Copy files
    for src in [pdf_path, csv_path, json_path]:
        dst = os.path.join(archive_path, os.path.basename(src))
        shutil.copy2(src, dst)
    
    # Log to database
    conn = sqlite3.connect('test_archive.db')
    cursor = conn.cursor()
    cursor.execute("""
        INSERT INTO test_runs (timestamp, pdf_path, csv_path, json_path)
        VALUES (?, ?, ?, ?)
    """, (datetime.now(), pdf_path, csv_path, json_path))
    conn.commit()
    conn.close()

### Q4: È possibile testare solo B6 o solo B8?

A: Sì, commenta le fasi non necessarie:

def main_test():
    for run in range(1, num_runs+1):
        power_cycle_grifo()
        
        pbit_ok = test_pbit_phase()
        
        b6_ok = test_b6_phase()  # Keep this
        # b8_ok = test_b8_phase()  # Comment out
        # serial_ok = test_serial_phase()  # Comment out
        # target_ok = test_target_phase()  # Comment out
        
        result = 'PASS' if b6_ok else 'FAIL'

### Q5: Come cambiare lingua report (Italiano → Inglese)?

A: Modifica stringhe in leo_grifo_test_report.py:

# OLD (Italian)
title = Paragraph("GRIFO M-PBIT TEST REPORT", title_style)

# NEW (English)
title = Paragraph("GRIFO M-PBIT TEST REPORT", title_style)

# Use constant dict for localization
STRINGS = {
    'IT': {
        'title': 'RAPPORTO TEST GRIFO M-PBIT',
        'run': 'Esecuzione',
        'passed': 'Superati',
        'failed': 'Falliti'
    },
    'EN': {
        'title': 'GRIFO M-PBIT TEST REPORT',
        'run': 'Run',
        'passed': 'Passed',
        'failed': 'Failed'
    }
}

# Select language
LANG = 'EN'
title = Paragraph(STRINGS[LANG]['title'], title_style)

## 10.8 Risorse Aggiuntive

### 10.8.1 Documentazione Esterna

MIL-STD-1553: https://en.wikipedia.org/wiki/MIL-STD-1553X

ReportLab User Guide: https://www.reportlab.com/docs/reportlab-userguide.pdfX

PySerial Documentation: https://pyserial.readthedocs.io/X

Python Threading: https://docs.python.org/3/library/threading.htmlX

|  |  | ### 10.8.2 Tools Utili |
| --- | --- | --- |

Tool

Scopo

Link

Dependency Walker

Identify missing DLLs

dependencywalker.comX

PuTTY

Serial terminal debugging

putty.orgX

Wireshark

Network traffic analysis (BrainBox)

wireshark.orgX

DB Browser for SQLite

Inspect test archive database

sqlitebrowser.orgX

### 10.8.3 Contatti Supporto

Internal Team:
- Test Framework: test-automation@company.com
- GRIFO Hardware: grifo-support@company.com
- IT Infrastructure: it-helpdesk@company.com

External Vendors:
- 1553 Card Vendor: support@1553vendor.com
- BrainBox Manufacturer: support@brainbox.com

## 10.9 Checklist Pre-Release

Prima di rilasciare nuova versione test environment:

✅ Code Quality:
- [ ] Tutti i moduli hanno docstring
- [ ] Type hints per funzioni pubbliche
- [ ] No hardcoded paths (usa os.path.join)
- [ ] Logging appropriato (no print statements)
- [ ] Exception handling robusto

✅ Testing:
- [ ] Simulation mode testato (10+ runs)
- [ ] Hardware mode testato (se disponibile)
- [ ] Report generati correttamente (PDF/CSV/JSON)
- [ ] GUI funzionante (no crashes)
- [ ] Tutti gli scenari funzionano

✅ Documentation:
- [ ] README aggiornato
- [ ] CHANGELOG con nuove feature
- [ ] Questa documentazione aggiornata
- [ ] Commenti inline per codice complesso

✅ Compatibility:
- [ ] Python 3.8+ supportato
- [ ] Windows 10/11 testato
- [ ] Linux compatibility verificata (opzionale)
- [ ] Dependencies specificate in requirements.txt

✅ Deployment:
- [ ] Git tags per versione (v2.0.0)
- [ ] Branch main stable
- [ ] Release notes pubblicate
- [ ] Team notificato

## 10.10 Roadmap Future Enhancements

Planned Features (priorità alta):
1. Web Dashboard: Real-time monitoring via browser
2. Database Backend: Persistent storage per trend analysis
3. Email Notifications: Automatic alerts per failures
4. Remote Execution: Control test via REST API

Planned Features (priorità media):
5. Multi-GRIFO Support: Parallel testing di più unità
6. Advanced Analytics: ML-based failure prediction
7. Custom Plugins: Extensible architecture per custom tests
8. Localization: Support per multiple languages

Planned Features (priorità bassa):
9. Mobile App: iOS/Android monitoring app
10. Voice Alerts: Text-to-speech per critical events

## 10.11 Conclusione

Questo test environment rappresenta un sistema robusto e ben documentato per automated testing di sistemi GRIFO. Seguendo le best practices e troubleshooting guide di questo capitolo, dovresti essere in grado di:

✅ Eseguire test con confidenza (simulation e hardware)
✅ Diagnosticare problemi rapidamente quando si presentano
✅ Estendere il sistema per nuovi requisiti
✅ Integrare con CI/CD per automation continua
✅ Manutenere il codice nel lungo termine

Ricorda:
- Simulation first: Testa sempre in simulation prima di hardware
- Log everything: Logging dettagliato è fondamentale per debugging
- Automate wisely: Non automatizzare test instabili
- Document changes: Mantieni documentazione sincronizzata con codice

Buon testing! 🚀

← Integrazione Hardware | Torna all’IndiceX